3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200049

Online Meeting, 17 January – 25 January 2022

3GPP TSG RAN WG1 Meeting #107-e	 R1-2112858

E-meeting, November 11th – 19th, 2021

Title:	Reply LS on beam information of PUCCH SCell in PUCCH SCell activation procedure

Response to:	R1-2108704/R4-2115339

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	RAN WG1

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

#### Name:	Frank Long

E-mail Address:	frank.longyi {@ }huawei.com

Attachments:

1. Overall Description:

RAN1 thanks RAN4 for the LS on beam information of PUCCH SCell in PUCCH SCell activation procedure. RAN1 answers are as follows.

Q1: Whether UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group

Answer: There is no restriction in the current RAN1 specification that would not allow UE to report CSI of a SCell belonging to secondary/primary PUCCH group by PUSCH or PUCCH of active serving cells belonging to primary/secondary PUCCH group. But there is no RAN1 consensus on whether all UEs supporting NR-CA with dual PUCCH-groups for the BC support such CSI report in Rel-15 and Rel-16. Support of such CSI report is indicated in Rel-17 with a new UE capability. Potential CSI processing timeline relaxation for UEs reporting the new UE capability can be discussed.

Q2: Whether the above observation is correct, i.e. the identified four cases are not supported by the current RAN1 and RAN2 specification.

Answer: RAN1 is not able to answer the question on whether the identified four cases are supported or not by current RAN1 specification.

Q3: Whether the above identified cases can be supported by RAN1 and RAN2 spec updates within Rel-17 timeframe.

Answer: RAN1 is not able to answer the question. However, RAN1 expects that reporting CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group supports the identified four cases.

2. Actions:

To: RAN4

ACTION: 	RAN1 respectfully asks RAN4 to take the above answers into account in their future work.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#107-bis-e		17 - 25 January 2022	   				Online

3GPP RAN1#108-e		21 February - 3 March 2022	   			Online