3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200072

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PUCCH Scell activation delay requirements with multiple Scell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-1-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The requirements for PUCCH SCell activation/deactivation with multiple SCell should be introduced in RRM specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the requirements for PUCCH SCell activation/deactivation with multiple SCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for PUCCH SCell activation/deactivation with multiple SCell will be not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 8.3.14, new 8.3.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.3.14	PUCCH SCell activation delay Requirement with Multiple SCells

The requirements in this clause shall apply for the UE configured with multiple downlink SCell and when PUCCH is configured for the SCell being activated.

The PUCCH SCell activation delay requirement defined in clause 8.3.12 is applied for PUCCH SCell activation.

SCell Activation Delay Requirement defind in clasue 8.3.2 is applied for downlink SCell activation.

The interruption on the PCell specified in section 8.2.2.7 shall meet.

8.3.15	PUCCH SCell deactivation delay Requirement with Multiple SCells

The requirements in this section shall apply for the UE configured with multiple downlink SCell and when PUCCH is configured for the SCell being activated.

The UE shall deactivate a SCell configured with PUCCH and other downlink SCells and meet the SCell deactivation delay requirements specified in section 8.3.3.

The interruption on the PCell specified in section 8.2.2.7 shall meet.