3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200073

Electronic Meeting, January 17-25, 2022

Title: 	Further discussion on UE Tx RF requirements for NTN

Source: 	CATT

Agenda item:	6.13.4.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RF requirements for NTN UE were discussed and a way forward was approved [1]. This document will further discuss remaining NTN UE Tx RF requirements.

- Transmit modulation quality

- Spurious emission for UE co-existence

- Discussion

Transmit modulation quality

In WF [1], there are following proposals:

- Option 1: this requirement can be reused for NTN UE for QPSK and 16QAM.

  - 64QAM and 256QAM should be excluded.

- Option 2: this requirement can be reused for NTN UE for QPSK, 16QAM and 64QAM.

  - 256QAM should be excluded.

- Option 3: other, please specify.

WF: Option 2 as starting point and further check whether UL 64QAM needs to be removed later.

In last meeting, we present our concern for 64QAM that may not have sufficient SINR in NTN scenario. We would prefer not to support 64QAM for both UL and DL unless its feasibility is justified.

Proposal 1: it is proposed to consider QPSK and 16QAM in Rel-17.

Spurious emission for UE co-existence

It is proposed to further clarify the co-existence band in the next RAN4 meeting in WF [1]. The general spurious requirements can be reused for UE if there is no special regulation requirement for NTN UE.

For UE co-existence requirement, there could be similar scenario between NTN UEs and between NTN UE and TN UE. -50dBm/MHz requirement could be reused from 38.101. Since the UE will be globally used, how many co-existence bands should be well considered.

Proposal 2: it is proposed to reuse -50dBm/MHz co-existence requirement for NTN UE.

- Conclusion

This document discussed Tx RF requirements for NTN UE.

Proposal 1: it is proposed to consider QPSK and 16QAM in Rel-17.

Proposal 2: it is proposed to reuse -50dBm/MHz co-existence requirement for NTN UE.

- Reference

[1]	R4-2120676, WF on NTN UE RF requirements, Huawei