3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200074

Electronic Meeting, January 17-25, 2022

Title: 	Further discussion on UE Rx RF requirements for NTN

Source: 	CATT

Agenda item:	6.13.4.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the Rx requirements for NTN UE were discussed, and a way forward was approved [1]. This document will further discuss NTN UE Rx RF requirements.

- Discussion

1).  Reference sensitivity

In last meeting, RAN4 agreed the general formula can be agreed to derive the REFSENS for NTN UE:

REFSENS, Sensitivity = -174dBm (kT) + 10*log(RX BW) + NF + SNR + IM – diversity gain

Further discuss the NF, SNR, IM, diversity gain and REFSENS requirements in next meeting for each satellite band.

The NF, SNR, IM, diversity gain for TN UE in same frequency can be reused for NTN UE.

Based on the physical spec, there is no difference between TN and NTN, the FRC for TN UE REFSENS may be reused for NTN UE.

Proposal 1: The requirements of NF, SNR, IM, diversity gain for TN UE can be reused for NTN UE on same frequency.

Proposal 2: The FRC for TN UE REFSENS can be reused for NTN UE.

Proposal 3: REFSENS for n24 and n65 may be reused for NTN UE in n255 and n256 with SCS and bandwidth further discussion.

2).  Maximum input level

It is noted in WF [1] that further evaluation is needed considering minimum CL between satellite and UE. Companies are invited for input in the next meeting. In last meeting, RRM session has done initial dynamic system simulation. The change of NTN DL signal will be very small seen from the following figure [2]. The maximum input power level may do need to specify up to -25dBm.

Figure 4 Time trace of downlink SINR for different handover configurations.

Observation: The maximum input power level may do need to specify up to -25dBm.

- Conclusion

This document discussed Rx RF requirements for NTN UE. The following proposals are concluded.

Proposal 1: The requirements of NF, SNR, IM, diversity gain for TN UE can be reused for NTN UE on same frequency.

Proposal 2: The FRC for TN UE REFSENS can be reused for NTN UE.

Proposal 3: REFSENS for n24 and n65 may be reused for NTN UE in n255 and n256 with SCS and bandwidth further discussion.

Observation: The maximum input power level may do need to specify up to -25dBm.

- Reference

[1]	R4-2120676, WF on NTN UE RF requirements, Huawei

[2]	R4-2119125, Dynamic system level assumptions and results for NTN moving cells mobility, Nokia, Nokia Shanghai Bell