3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200103

Electronic Meeting, January 17-25, 2022

Agenda item:	6.9.4.6

Source: 	CATT

Title: 	Further discussion on measurement procedure requirements for FR2 HST

Document for:	Discussion

Introduction

There were extensive discussions for FR2 HST during the past RAN4 meetings and a WF on RRM requirements for NR FR2 HST was approved [1] in RAN4#101-e meeting.

In this contribution, we provide our consideration of several open issues for measurement procedure requirements for FR2 HST and give our proposals.

Discussion

In RAN4#101-e meeting, it is agreed that RAN4 to define the enhancement for inter-frequency measurement in IDLE mode for FR1 HST as below:

For PSS/SSS and intra-freq measurement

- Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST with the same SMTC periodicity bound of 40ms unless technical issues identified - FFS if a different scaling factor is needed for scenario-B with two-side RRH

Our view is the scaling factor 1.5 is not related to the RX beam sweeping. The same M2 can be used in Scenario-B with two-side RRH. For PSS/SSS and intra-frequency measurement, Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps can be 6 and 18 for Scenario A and Scenario B.

Proposal 1: The different scaling factor is not needed for Scenario-B with two-side RRH. For each scenario, define unified requirement. For PSS/SSS and intra-frequency measurement, Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps can be 6 (Scenario A) and 18 (Scenario B).

Conclusion

This document presented our consideration of several open issues for measurement procedure requirements for FR2 HST. The following proposals are concluded as below:

Proposal 1: The different scaling factor is not needed for Scenario-B with two-side RRH. For each scenario, define unified requirement. For PSS/SSS and intra-frequency measurement, Mpss/sss_sync_w/o_gaps  and Mmeas_period_w/o_gaps can be 6 (Scenario A) and 18 (Scenario B).

Reference

[1] R4-2120416, WF on FR2 HST RRM requirements (part 2), Samsung