3GPP TSG-RAN WG4 Meeting #101-bis-e                                                    R4-2200131

Electronic Meeting, January 17 - 25, 2022

Source: 	CATT

Title: 	Simulation results for PRACH demodulation requirements for FR2 HST

Agenda item:	6.9.5.3.1

Document for:	Approval

- Introduction

In RAN4#101-e, a way forward on FR2 HST demodulation was approved [1].

In this contribution, we provide our simulation results for PRACH demodulation following latest agreements.

- Simulation results

As per the WF [1], the simulation assumptions for PRACH requirement are shown in Table A-1.

The simulation results for PRACH requirement for FR2 HST are shown in Table 2-1.

Table 2-1, Simulation results for PRACH requirement for FR2 HST

| PRACH |  |  |  |  |  | SNR@ probability of missed detection 1% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Tx | Number of Rx | Format | Channel | Frequency offset | Timing offset range | Ideal | Impairment |
| 1 | 2 | C2 | AWGN | 19444Hz | 0:0.48:4.8 | -12.59 | -10.09 |

- Conclusion

This contribution provides our simulation results for PRACH demodulation requirements for FR2 HST for alignment.

- References

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, Samsung, RAN4#101-e

- Annex

Table A-1, Simulation assumption for PRACH requirement for FR2 HST

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Time error tolerance | 0.07us |
| Timing offset | 4.8us with TO range 0:0.48:4.8 |