3GPP TSG-RAN WG4 Meeting #101-bis-e                                                                                                 R4-2200137

Electronic Meeting, January 17 - 25, 2022

Source: 	CATT

Title: 	Discussion on the BS RX RF requirements for 52.6-71GHz

Agenda item:	6.16.4.2

Document for:	Approval

- Introduction

In RAN4#101-e, a WF [1] on BS RF RX requirements for 52.6 – 71 GHz was approved. The step size for spurious emissions, ICS need to be further discussed.

In this contribution, we provide our views on the remaining issues in the WF [1].

- Discussion

OTA reference sensitivity level

The EISREFSENS is calculated using the following formula.

EISREFENS=-174+10*log10(BW)+NF+IM+SNR-G                                                                                                      (1)

Since the definition of OTA reference sensitivity level is based on EISREFSENS_50M, So, the EISREFSENS can be expressed as,

EISREFENS= EISREFENS_50M +10*log10(BW/BW50M)+(SNR-SNR50M)                                                                           (2)

Where,

-	BW is the maximum transmission bandwidth.

-	SNR is the SNR value for which we reach 95% throughput. Each company provided simulation results, and average will be done for each BW. The SNR value for existing FR2-1 FRC is shown as Table 2-1. The SNR50M is SNR for G-FR2-A1-1, equal to -1.1dB.

Table 2-1: the SNR value for existing FR2-1 FRC

| G-FR2-A1-1 | -1.1 |
| --- | --- |
| G-FR2-A1-2 | -1.1 |
| G-FR2-A1-3 | -1.2 |
| G-FR1-A1-4 | -1.2 |
| G-FR1-A1-5 | -1 |

The SNR- SNR50M of equation (2) is denpendent on the link simulation results for FRC for OTA reference sensitivity, From Table 2-1, it is observed that the difference of SNR between different FRC and G-FR2-A1-1(50M) is about 0.1 dB. So SNR-SNR50M is close to 0.

When it comes to FR2-2, the SNR value for G-FR2-A1-6 and G-FR2-A1-7 shall be specified by link level simulation, the Simulation assumptions in R4-1801031[2] for FR2 FRC simulation can be as starting point for 52.6-71GHz. The MCS index 4 and target coding rate = 308/1024, MMSE receiver, DMRS pattern: 1+1 (2, 11), no data in DMRS symbol, PTRS configuration: KPTRS=2, LPTRS=1, No phase noise simulation, no data in DMRS REs can be used for 52.6-71GHz. If the same MCS and target coding rate for FR2-1 are reused for FR2-2, we think SNR-SNR50M is also very small.

The 10*log10(BW/BW50M) of equation (2) is dependent on the FRC for reference sensitivity requirement. If the same 95% SU for FR2-1 is reused for FR2-2, the BW and TBS for G-FR2-A1-6 and G-FR2-A1-7 is shown in Table 2-2. The 10*log10(BW/BW50M)  for G-FR2-A1-6 and G-FR2-A1-7 is also calculated in Table 2-2. And SNR-SNR50M for G-FR2-A1-6 and G-FR2-A1-7 is assumed as 0.

Table 2-2: BW and TBS for G-FR2-A1-6 and G-FR2-A1-7

| FRC | CBW(MHz) | SCS(kHz) | BW | TBS(bits) | 10*log10(BW/BW50M) | SNR- SNR50M |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR2-A1-6 | 400 | 480 | [66PRBs] | [5632] | 10*log10(66*480/(66*60))= 9.03 | [0] |
| G-FR2-A1-7 | 400 | 960 | [32PRBs] | [2792] | 10*log10(32*960/(66*60))=8.90 | [0] |

Based on above 10*log10(BW/BW50M)  and SNR-SNR50M for G-FR2-A1-6 and G-FR2-A1-7 in Table 2-2, the OTA reference sensitivity level for FR2-2 is calculated in Table 2-3.

Table 2-3: OTA reference sensitivity for FR2-2

| Frequency Range | BS channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- | --- |
| FR2-2 | 100,400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
|  | 400, 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + [9] + ΔFR2_REFSENS |
|  | 400, 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + [9] + ΔFR2_REFSENS |

Observation 1: FRC and SNR value for each new FRC is to be confirmed once spectral utilization is agreed.

Receiver spurious emission

As per the WF [1], the remaining issues concerning receiver spurious emission are shown as below:

Spurious emissions - Use FR2 approach with necessary adaptations on step size for spurious emissions, i.e., align emission levels with TX requirements - Use TRP as emissions metric - Step size Fstep,x needs further discussion.

For receiver spurious emissions, it is reasonable that the step frequencies (Fstep,X,x=1-6) for NR operating bands in 52.6-71GHz  is aligned with  our proposed step frequencies for transmitter spurious emissions

Proposal 1: To specify the following Fstep,X for receiver spurious emissions.

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

ICS

As per the WF [1], the agreements and remaining issues concerning ICS are shown as below:

In-Channel Selectivity (ICS) - Specify the BS ICS requirement as [10] dB for NR operation in 52.6 – 71 GHz range. - For in-channel selectivity scale wanted power and interfering power levels for 100 MHz and 400 MHz carrier bandwidths.

For EISREFSENS_50M,

EISREFSENS_50M= -174dBm/Hz+10*log10(BW50M)+NF+SNR50M+IM-G                                                                  (3)

For interfering signal,

EISREFSENS_interfer = -174dBm/Hz+10*log10(BWinterfer)+NF+ICS-G                                                                         (4)

For Wanted signal,

EISREFSENS_wanted = -174dBm/Hz+10*log10(BWwanted)+NF+SNRwanted+IM-G+3dB                                              (5)

Where,

-	BW50M  is maximum transmission bandwidth for 60kHz SCS, equal to 66*12*60*1000 Hz.

-	BWinterfer is allocated bandwidth for interfering signal.

-	BWwanted is allocated bandwidth for FRC for wanted signal.

-	3dB is reference sensitivity degradation which is reused from legacy E-UTRA requirement.

-	IM is the implementation margin, equal to 2dB.

-	SNRwanted is the SNR value for FRC which we reach 95% throughput.  The SNRwanted for each FR2-1 FRC is shown in the following table.

-	SNR50M is the SNR value for FRC for 50MHz CBW/60kHz SCS (G-FR2-A1-1)  which we reach 95% throughput, equal to -1.1dB.

Using (5)-(3), we get,

EISREFSENS_wanted = EISREFSENS_50M +10*log10(BWwanted/ BW50M)+( SNRwanted - SNR50M)+3dB                              (6)

Using(4)-(3), we get,

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ICS- SNR50M-IM

= EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ICS- (-1.1)-2

= EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ICS-0.9                                                              (7)

In our understanding, the ICS in equation(7) is achieved by IoT simulation instead of the difference between the wanted signal and the interfering signal.

For FR2-1, In TR 38.817-2, the ICS is agreed as 14dBc for all BS type based on IoT simulation..  So, (7) can be expressed as,

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+13.1                                                                     (8)

The equation (6) and (8) were used for calculating wanted signal mean power and interfering signal mean power respectively for OTA in-channel selectivity requirement for FR2-1.

Following the latest agreement, ICS =[10] dB, which means that the interfering signal power for FR2-2 relative FR2-1 is significantly degraded.  Using ICS=10dB, equation (7) can be expressed as,

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+9.1                                                                     (9)

The equation (6) and (9) were used for calculating wanted signal mean power and interfering signal mean power respectively for OTA in-channel selectivity requirement for FR2-2.

Proposal 2: To use the following equations to calculate wanted signal mean power and interfering signal mean power respectively for OTA in-channel selectivity requirement for FR2-2.

EISREFSENS_wanted = EISREFSENS_50M +10*log10(BWwanted/ BW50M)+( SNRwanted - SNR50M)+3

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ ICS-0.9

Observation 2: ICS in equation is achieved by IoT simulation instead of the difference between the wanted signal and the interfering signal.

The BWwanted is dependent on the FRC for OTA in-channel selectivity requirement for FR2-2, and SNRwanted is dependent on the link simulation results for FRC for OTA in-channel selectivity requirement for FR2-2.

We can define the G-FR2-A1-8 with [33PRBs] for 480kHz SCS for ICS, and its interfering signal is DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB, and G-FR2-A1-9 with [16PRBs] for 960kHz SCS, and its interfering signal is DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB. And SNRwanted-SNR50M for G-FR2-A1-8 and G-FR2-A1-8 is assumed as 0.

Table 2-4: new FRC for ICS for FR2-2

| FRC | BWwanted | SCS(kHz) | TBS(bits) | 10*log10(BWwanted/ BW50M) | ( SNRwanted - SNR50M) |
| --- | --- | --- | --- | --- | --- |
| G-FR2-A1-8 | [33PRBs] | 480 | [2856] | 10*log10(33*480/(66*60))=6.02 | [0] |
| G-FR1-A1-9 | [16PRBs] | 960 | [1416] | 10*log10(16*960/(66*60))=5.89 | [0] |

Table 2-5: interfering signal for ICS for FR2-2

| FRC | Type of interfering signal | BWinterfer | SCS(kHz) | 10*log10(BWinterfer / BW50M) |
| --- | --- | --- | --- | --- |
| G-FR2-A1-8 | DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB | [32PRBs] | 480 | 10*log10(32*480/(66*60))=5.89 |
| G-FR1-A1-9 | DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB | [16PRBs] | 960 | 10*log10(16*960/(66*60))=5.89 |

Based on above 10*log10(BWwanted/ BW50M)  and ( SNRwanted - SNR50M) in Table 2-4,  and 10*log10(BWinterfer / BW50M) in Table 2-5, the OTA  in-channel selectivity requirement for FR2-2  is calculated in Table 2-6 using equation(6) and equation(9).

Table 2-6: OTA in-channel selectivity requirement for FR2-2

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-2 | 100, 400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + [9] + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
|  | 400, 800, 1600 | 480 | [G-FR2-A1-8] | EISREFSENS_50M + [9]+ ΔFR2_REFSENS | EISREFSENS_50M + [15]+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, [32 RB] |
|  | 400, 800, 1600,2000 | 960 | [G-FR2-A1-9] | EISREFSENS_50M + [9]+ ΔFR2_REFSENS | EISREFSENS_50M + [15] + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, [16 RB] |

Based on above discussion, we propose,

Proposal 3: To define the G-FR2-A1-8 for 480kHz SCS, and G-FR2-A1-9 for 960kHz SCS for OTA in-channel selectivity requirement for FR2-2.

- Conclusion

This contribution provides analysis on BS RX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Observation 1: FRC and SNR value for each new FRC is to be confirmed once spectral utilization is agreed.

Proposal 1: To specify the following Fstep,X for receiver spurious emissions.

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

Proposal 2: To use the following equations to calculate wanted signal mean power and interfering signal mean power respectively for OTA in-channel selectivity requirement for FR2-2.

EISREFSENS_wanted = EISREFSENS_50M +10*log10(BWwanted/ BW50M)+( SNRwanted - SNR50M)+3

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ ICS-0.9

Observation 2: ICS in equation is achieved by IoT simulation instead of the difference between the wanted signal and the interfering signal.

Proposal 3: To define the G-FR2-A1-8 for 480kHz SCS, and G-FR2-A1-9 for 960kHz SCS for OTA in-channel selectivity requirement for FR2-2.

- References

- R4-2120678, WF on BS RF Rx requirements and CR work split, Ericsson, RAN4#101-e

- Draft ETSI EN 303 722 V0.0.11 (2021-06),Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 GHz to 71 GHz band; Harmonised Standard for access to radio spectrum, ETSI

- R4-2117389, Discussion on the BS TX RF requirements for 52.6-71GHz,CATT,RAN4#101-e

- TR 38.817-2.