3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2200162

Electronic Meeting, January 17th - 25th, 2022

Title: 	On remaining open issue for NTN system parameters

Source: 	CATT

Agenda item:	6.13.1.1

Document for:	Discussion

- Introduction

Way Forward on NTN_solutions_Part1 was agreed in [1] which captured the agreements for NTN system parameters. This paper will discuss the remaining open issue for NTN system parameters.

- Discussion

  - Operating bands and channel bandwidth

The NTN bands as shown in Table 2-1 was agreed and has been included in TR 38.863 0.1.0 in [2]. However the format of this table might causing confusion since we are using descend order for NTN band numbering which is different from the conventional numbering method. It is proposed to change it to table 2-2 and adding a note. The corresponding Table 7.2.2-1 in TR 38.863 for Channel bandwidth, SCS and spectral utilization also needs to be updated.

Table 2-1: NTN bands in FR1 (7.2.1-1 in TR 38.863)

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |

Table 2-2: NTN bands in FR1 (modified)

| NTN satellite band No. (NOTE) | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

Proposal 1: it is proposed to update Table 7.2.1-1 and 7.2.2-1 in TR 38.863 to reflect the agreed numbering scheme for NTN.

  - Spectrum utilization

According to the co-existence simulation results and the analysis in [3], the factors impacting the SU are not changed. We think it’s reasonable to reuse the spectrum utilization from Rel-15 NR.

Proposal 2: It is proposed to reuse the spectrum utilization from Rel-15 NR for NTN.

  - Channel raster

It has been agreed to reuse the NR-ARFCN parameters below 3000 MHz and 100 kHz channel raster for S/L band. The applicable NR-ARFCNs per operating band need to be determined.

Table 2.3-1 NR-ARFCNs per operating band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 39600 – <20> – 40200 | 43400 – <20> – 44000 |
| n255 | 100 | 32530 – <20> – 33210 | 30500 – <20> – 31180 |

Proposal 3: It is proposed to define the applicable NR-ARFCN for S/L band as in Table 2.3-1.

  - sync raster

The synchronization raster requirements below 3000 MHz specified in TS 38.101-1 shall be reused for MSS S-Band as follows. Table

Table 2.4-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note 1) | 3N + (M-3)/2 | 2 – 7498 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: FFS for frequency range larger than 3000 MHz |  |  |  |

Table 2.4-2 Applicable SS raster entries per operating band

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |

Proposal 4: It is proposed to define the applicable SS raster entries for S/L band as in Table 2.4-2.

- Conclusion

This paper discussed the remaining issue for NTN system parameters. The following proposals are concluded,

Proposal 1: it is proposed to update Table 7.2.1-1 and 7.2.2-1 in TR 38.863 to reflect the agreed numbering scheme for NTN.

Proposal 2: It is proposed to reuse the spectrum utilization from Rel-15 NR for NTN.

Proposal 3: It is proposed to define the applicable NR-ARFCN for S/L band as in Table 2.3-1.

Proposal 4: It is proposed to define the applicable SS raster entries for S/L band as in Table 2.4-2.

- Reference

[1]	RP-2120774, Way Forward on NTN_solutions_Part1, Thales

[2]	RP-2120776, TR 38.863 0.1.0