3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2200163

Electronic Meeting, January 17th - 25th, 2022

Title: 	TP to TR 38.863 on channel raster and sync raster

Source: 	CATT

Agenda item:	6.13.1.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on channel raster and sync raster for TR 38.863 [1].

- Reference

[1] TR 38.863, 0.1.0

[2] R4-2200162, on remaining open issue for NTN system parameters, CATT

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

7.2.3	Channel raster and sync raster

7.2.3.1	 Channel raster

In order to be compatible with current design of sync raster below 3 GHz, the ARFCN parameters below 3000 MHz will be reused and 100 kHz was chosen as the channel raster for NR NTN bands below 3 GHz.

The global frequency raster defines a set of RF reference frequencies FREF and is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 7.2.3.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 7.2.3.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| NOTE: FFS for frequencies larger than 300 MHz. |  |  |  |  |

For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The applicable ARFCNs per operating and for satellite access node are defined in table 7.2.3.1-2.

Table 7.2.3.1-2 ARFCNs per operating band satellite access node

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 39600 – <20> – 40200 | 43400 – <20> – 44000 |
| n255 | 100 | 32530 – <20> – 33210 | 30500 – <20> – 31180 |

7.2.3.2 Sync raster

The synchronization raster requirements below 3000 MHz specified in TS 38.101-1 shall be reused for MSS S-Band as cited in Table 7.2.3.2-1.

Table 7.2.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note 1) | 3N + (M-3)/2 | 2 – 7498 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: FFS for frequency range larger than 3000 MHz |  |  |  |

Applicable SS raster entries per operating band are defined in Table 7.2.3.1-2.

Table 7.2.3.1-2 Applicable SS raster entries per operating band

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |

---------------------------------------------------End of Text proposal---------------------------------------------------------