3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2200169

Electronic Meeting, January 17th - 25th, 2022

Title: 	Satellite access node RF requirements for type 1-O - Tx part

Source: 	CATT

Agenda item:	6.13.3.1

Document for:	Discussion

- Introduction

WF on Tx RF requirements for satellite access node was agreed in [1]. The requirements for satellite access node type 1-O remain FFS. This paper gives analysis on Rx RF requirements for satellite access node type 1-O.

- Discussion

| Tx requirement |  | Classification | Coverage range |  | Number of | comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 | conformance directions |  |
| Radiated transmit power |  | Directional | OTA peak directions set | OTA peak directions set | 5 | The requirement in 38.104 can be used as the starting point. |
| OTA BS output power |  | TRP | See annex I |  |  | The TRP value can be determined once the NTN class in [2] is determined. |
| OTA output power dynamics |  | Directional | OTA peak directions set | OTA peak directions set | 1 | This requirement can be reused from 38.104 |
| OTA transmitter OFF power |  | Co-location | See clause 4.12 | N/A | See clause 4.12 | This requirement is not needed. |
|  |  | Directional | N/A | OTA peak directions set (Note 2) | 1 | This requirement is not needed. |
| OTA transient period |  | Co-location | See clause 4.12 | N/A | See clause 4.12 | This requirement is not needed. |
|  |  | Directional | N/A | OTA peak directions set (Note 2) | 1 | This requirement is not needed. |
| OTA modulation quality |  | Directional | OTA coverage range | OTA coverage range | 5 | This requirement for QPSK and 16QAM can be reused from 38.104 |
| OTA frequency error |  | Directional | OTA coverage range | OTA coverage range | 1 | This requirement can be reused from 38.104 |
| OTA time alignment error |  | Directional | OTA coverage range | OTA coverage range | 1 | This requirement is not needed given CA and MIMO feature are not supported in Rel-17. |
| OTA occupied bandwidth |  | Directional | OTA coverage range | OTA coverage range | 1 | This requirement can be reused from 38.104 |
| OTA ACLR |  | TRP | N/A | N/A | See annex I | 38dB as discussed in [2] |
| OTA operating band unwanted emission |  | TRP | N/A | N/A | See annex I | FFS |
| OTA transmitter spurious emission | General requirement | TRP | N/A | N/A | See annex I | This requirement is needed. Compliance to ERC 74-01 Space stations’ limits, i.e. ITU Radio Regulations Annex 3. |
|  | Protection of the BS receiver of own or different BS | Co-location | See clause 4.12 | N/A | See clause 4.12 | This requirement can be reused from 38.104. |
|  | Additional spurious emissions | TRP | N/A | N/A | See annex I | This requirement needs to be studied as discussed for conducted part in [2] |
|  | Co-location with other base stations | Co-location | See clause 4.12 | N/A | See clause 4.12 | This requirement is not needed. |
| OTA transmitter intermodulation |  | Co-location | See clause 4.12 | N/A | See clause 4.12 | This requirement is not needed. |

- Conclusion

This paper further discussed Tx RF requirements for satellite access node type 1-O and gave the proposals as shown in Table 2.1-1.

- Reference

[1]	R4-2120774, WF on Tx RF requirements for satellite access node, CATT

[2]	R4-2200169, Discussion on remaining issue for NTN BS RF requirement - Tx part, CATT