3GPP TSG-RAN WG4 Meeting # 101-bis-e															 R4-2200184

Electronic Meeting, January 17-25, 2022

Agenda Item: 6.19.3.3

Source: MediaTek Inc.

Title: Discussion on general and RRM requirements impacts in R17 feMIMO

Document for: Discussion

- Introduction

According to the WF [1] in the last meeting, RAN4 agreed to define the requirement for TRP specific BFR. In this paper, our views on the TRP specific BFR and remaining issues, including QCL definition for UL TCI state and simultaneous reception channel/RS with different QCL type D, are provided.

- Discussion

2.1 TRP specific BFR

RAN4 has agreed to define the requirement for the TRP specific BFR and the details were FFS in the last meeting. To our understanding, two BFD-RSs/CBD-RSs from two TRPs may be transmitted on the same OFDM symbol. For FR1, UE can receive two BFD-RSs/CBD-RSs from two TRPs simultaneously and the extra effort is needed to evaluate two BFD-RSs/CBD-RSs at the same time in based band modules. For FR2, UE may not be able to receive two BFD-RSs/CBD-RSs from two TRPs simultaneously if UE does not support simultaneous reception with multiple panel. Thus, considering worst case, for both FR1 and FR2, to introduce a new scaling factor PTRP for TRP specific BFR is suggested.

Proposal 1: For TRP specific BFR, for both FR1 and FR2, to introduce a new scaling factor PTRP to extend the evaluation period of BFD and CBD for multiple TRPs.

2.2 QCL definition of UL

For the DL, the applicability of the DL QCL defined in clause 3.6.7 in TS 38.133 is used in some RRM requirements, e.g. CSI-RS based L1-RSRP reporting.

The content extracted from clause 9.5.4.2 in TS 38.133 is provided as below for reference.

9.5.4.2 CSI-RS based L1-RSRP Reporting … - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. …

However, in the last meeting, RAN4 were discussing whether to update the applicability of QCL for UL due to SRS resource. To our understanding, in current TS 38.133, no requirement is based on the QCL Type-D with SRS resource. Thus, we suggest RAN4 to further study the needs of the update QCL definition for UL TCI state first.

Proposal 2: RAN4 should further check the need of the QCL definition for UL TCI state.

2.3 Simultaneous reception channel/RS with different QCL type D

According to WF in RF session [2], to our understanding, the same discussion whether to introduce the new requirement for simultaneous reception with multiple panels in R17 is discussing parallelly now. To avoid the confliction between RF and RRM sessions, we suggest to wait for the RF session’s conclusion on this issue.

Observation 1: In RF session, simultaneous reception channel/RS with different QCL type-D in R17 is being discussed.

In addition, due to the limited time for R17 issue, to our opinion, it would be difficult to discuss and define the requirement after the decision on simultaneous reception made by RF session. Thus, we suggest to postpone the discussion for the simultaneous reception channel/RS with different QCL type-D to the future lease.

Proposal 3: In R17, no RRM requirements are specified for simultaneous reception channel/RS with different QCL type D.

- Summary

In this paper, the discussion of RRM impact in R17 feMIMO is provided. We have the following proposal:

Proposal 1: For TRP specific BFR, for both FR1 and FR2, to introduce a new scaling factor PTRP to extend the evaluation period of BFD and CBD for multiple TRPs.

Proposal 2: RAN4 should further check the need of the QCL definition for UL TCI state.

Observation 1: In RF session, simultaneous reception channel/RS with different QCL type-D in R17 is being discussed.

Proposal 3: In R17, no RRM requirements are specified for simultaneous reception channel/RS with different QCL type D.

- Reference

- R4-2120322, WF on other FeMIMO RRM requirements, Intel, RAN4 #101-e

- R4-2120006, WF on FeMIMO RF, Samsung, RAN4 #101-e