3GPP TSG-RAN WG4 Meeting #101bis-e	R4- 2200261

Electronic Meeting, Jan. 17 – Jan. 25, 2022

Agenda Item:	6.9.4.3

Source:	Apple

Title:	Discussion on the mobility requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 100-e.

In this paper, we discuss the mobility requirement for idle/inactive state and connected state separately.

# 2	Discussion

### 2.1	Connected state mobility

The following agreements and open issues are captured in [2].

Agreement: Baseline: DRX upper bound for enhanced RRM HST FR2 requirements is [80]ms. Way forward: FFS, whether different requirements shall be defined for requirement Set1 and Set 2.

M2 scaling factor for short DRX

Way forward from GtW: - Baseline: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1 - FFS if a different scaling factor is needed for scenario-B with two-side RRH

To enhance the connected state mobility for FR2 HST, it is natural to reuse the two sets of Rx beams based on network signaling, as the two sets of Rx beams are derived based on network deployment. It has been agreed that the set 2 requirements for scenario-B have covered the case with two-side RRH deployment, therefore there is no need to define different scaling factor for scenario-B with two side RRH.

Proposal 1: For connected state mobility, requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers.

Proposal 2: For connected state mobility, no need to further define different scaling factor for scenario-B with two-side RRH.

### 2.2	Connection re-establishment requirement

The following WF were captured in [2] on connection re-establishment requriement.

Way forward: Further study the requirement on the time to identify target cell for RRC connection re-establishment to NR intra-frequency cell in FR2 HST deployments: - Time to identify Known NR cell - Option 1: MAX (TBD, 5 x N1 x TSMTC), the value of N1 refers to agreed RX beam number - Option 1a: TBD is 400 ms - Option 1b: TBD is: 200 ms - Option 2: Do not introduce any enhancements - Time to identify unknown NR cell - Option 1: MAX (1000 ms, 10 x N1 x TSMTC)), the value of N1 refers to agreed RX beam number - Option 2: Do not introduce any enhancements

For RRC re-establishment procedure, the proposed enhancement for NR intra-cell identification is shown in Table I. N1 scaling factor can follow the set1 and set2 UE Rx beam based on network signaling.

For known NR cell, the requirement is proposed as MAX(400ms, 5 x N1 x TSMTC), where 400ms is the default value for FR2.

Table I: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX(400ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800 Note1 |
| < -8 | FR2 | N/A | 3520 Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Proposal 3: Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC).

Proposal 4: Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC).

### 2.3	Handover

Handover

For handover, the known cell for FR2 has been agreed in RAN4 100e meeting. It is straightforward to enhance scaling factor in the handover requirement based on the Rx beam discussion.

Proposal 5: Handover requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers.

### 2.4	Idle/inactive state mobility

In FR1 HST enhancement, reduced number of samples combined with additional parameter M2, M3 and M4 are introduced in DRX cycle < 320ms. When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Similar approach can be introduced in FR2 HST, as shown in the following table, where FR2 scaling factor N1 is introduced on top of FR1 HST table.

Table II: Tdetect,NR, TmeasureNR, and Tevaluate,NR when UE is configured with RRM enhancement for high speed

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR[s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | TBD | 2.56 x M2 x N1 (8 x M2 x N1) | 0.32 x M3 x N1 (1 x M3 x N1) | 0.96 x M4 x N1 (3 x M4 x N1) |
| 0.64 | TBD | 5.12 x N1 (8 x N1) | 0.64 x N1 (1 x N1) | 1.92 x N1 (3x N1) |
| 1.28 | TBD | 8.96 x N1 (7 x N1) | 1.28 x N1 (1 x N1) | 3.84 x N1 (3 x N1) |
| 2.56 | TBD | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: When SMTC <=40ms, M2=M3=M4=1; and when SMTC>40ms, M2=1.5, M3=M4=2 |  |  |  |  |

The value of scaling factor N1 should reuse the agreement on number of UE Rx beams in different deployment scenario, discussed in separate agenda item.

Proposal 6: For idle/inactive mode cell reselection enhancement, use 320ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the set 1 and set 2 Rx beam numbers.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1: For connected state mobility, requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers.

Proposal 2: For connected state mobility, no need to further define different scaling factor for scenario-B with two-side RRH.

Proposal 3: Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC).

Proposal 4: Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC).

Proposal 5: Handover requirement can be enhanced corresponding to the set 1 and set 2 UE Rx beam numbers.

Proposal 6: For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the set 1 and set 2 Rx beam numbers.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2120292, Way forward for FR2 HST RRM (part 1), Nokia, Nov 2021

- R4-2120416, Way forward for FR2 HST RRM part 2, Samsung, Nov 2021