3GPP TSG-RAN WG4 Meeting #101bis-e	R4- 2200264

Electronic Meeting, Jan. 17 – Jan. 25, 2022

Agenda Item:	6.9.4.6

Source:	Apple

Title:	Discussion on the measurement requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 101-e.

In this paper, we discuss the measurement requirement in HST FR2.

# 2	Discussion

In [3], the following agreements have been captured:

PSS/SSS detection & intra-frequency measurement

  - Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST with the same SMTC periodicity bound of 40ms unless technical issues identified

  - FFS if a different scaling factor is needed for scenario-B with two-side RRH

L1-RSRP measurement

  - Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms unless technical issues identified

  - Separate sets of requirements for deployment Scenarios A and B

Current intra-frequency PSS/SSS detection define Mpss/sss_sync_w/o_gaps is 24 in FR2 for UE power class 2,3,4. For measurement period requirement Mmeas_period_w/o_gaps is also 24 in FR2 for UE power class 2,3,4. Those values are significantly larger than FR1 case due to UE Rx beam sweeping. In current FR2, N=8, and 24 samples can be written as 3xN. Following number of Rx beam discussion, the intra-frequency PSS/SSS detection and measurement period can be enhancement for scenario A and scenario B separately.

Proposal 1:

Mpss/sss_sync_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2.

Mmeas_preriod_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2.

Proposal 2: No separate sets of scaling factor for scenario-B with two side RRH.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1:

Mpss/sss_sync_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2.

Mmeas_preriod_w/o_gaps = 3N, where N=2 for set 1 and N=6 for set 2.

Proposal 2: No separate sets of scaling factor for scenario-B with two side RRH.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2120292, Way forward for FR2 HST RRM (part 1), Nokia, Nov 2021

- R4-2120416, Way forward for FR2 HST RRM part 2, Samsung, Nov 2021