3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200265

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.6.5.2

Source:	Apple Inc.

Title:	Discussion on PDSCH demod requirements with 1KQAM

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#100-e PDSCH demod requirements with 1024QAM in NR DL were discussed and way forward [1] was agreed.

- Define new PDSCH requirements for 1024 QAM by using the existing new applicability rule. - Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the test parameters.

In this contribution we provide our views on the open issues for PDSCH demod requirements with 1024QAM.

# 2. Discussion

The agreed simulation assumptions for PDSCH demod requirements are captured in Table 1.

Table 1: Simulation parameters for PDSCH demod with 1024QAM

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 FFS to schedule PDSCH in the special slots |
| MCS (See appendix) | Option 1: MCS23 Option 2: MCS24 Option 3: MCS25 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 and 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLA30-10 Option 2: TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | Option 1: 2.0% Option 2: 2.5% Option 3: 2.8% Note: The assumed Tx EVM should be same or less than the BS Tx EVM requirements. |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

In RAN4#101-e the TX EVM requirements for 1024QAM was agreed and captured in [2] as below.

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| NOTE 1: This requirement is applicable for frequencies equal to or below 4.2 GHz. NOTE 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

The TX EVM assumption used for PDSCH demod simulation assumptions is lower than the TX EVM requirement. Hence, we propose to use TX EVM of 2% in the simulation assumptions for PDSCH demod requirements with 1024QAM.

Proposal #1: Use TX EVM as 2% in simulation assumptions for defining PDSCH demod requirements with 1024QAM.

For the simulation assumptions in Table 1 we provide simulation results for PDSCH demod.

TX EVM: 2%

MCS: 23, 24, 25

Channel Model: TDLA30-10, TDLD30-5

TDD

SCS: 30KHz

CBW: 40MHz

S slots: Enable PDSCH scheduling

Figure 1: Simulation results for TDD

FDD

SCS: 15KHz

CBW: 10MHz

Figure 2: Simulation results for FDD

The simulation results are summarized in the Table below.

Table 2: Results summary

| Duplex Mode | Antenna Config | MCS | Propagation Model | SNR @70% Max TP |
| --- | --- | --- | --- | --- |
| FDD | 2x2 ULA Low | 23 | TDLA30-10 | 27.2 |
|  |  |  | TDLD30-5 | 26.7 |
|  |  | 24 | TDLA30-10 | 29.4 |
|  |  |  | TDLD30-5 | 28.7 |
|  |  | 25 | TDLA30-10 | 31.2 |
|  |  |  | TDLD30-5 | 30.5 |
|  | 2x4 ULA Low | 23 | TDLA30-10 | 23.1 |
|  |  |  | TDLD30-5 | 22.8 |
|  |  | 24 | TDLA30-10 | 25.1 |
|  |  |  | TDLD30-5 | 24.7 |
|  |  | 25 | TDLA30-10 | 26.9 |
|  |  |  | TDLD30-5 | 26.4 |
| TDD | 2x2 ULA Low | 23 | TDLA30-10 | 27.9 |
|  |  |  | TDLD30-5 | 27.4 |
|  |  | 24 | TDLA30-10 | 29.5 |
|  |  |  | TDLD30-5 | 29.0 |
|  |  | 25 | TDLA30-10 | 32.3 |
|  |  |  | TDLD30-5 | 31.4 |
|  | 2x4 ULA Low | 23 | TDLA30-10 | 23.6 |
|  |  |  | TDLD30-5 | 23.1 |
|  |  | 24 | TDLA30-10 | 25.0 |
|  |  |  | TDLD30-5 | 24.5 |
|  |  | 25 | TDLA30-10 | 27.4 |
|  |  |  | TDLD30-5 | 26.7 |

Based on the results, we observe that with 2RX the operating SNR is high with MCS 24 and 25 and not feasible to define requirements

Observation #1: Operating SNR with MCS 24 and 25 with 2RX are high and not feasible to define requirements.

Comparing the performance of TDLA and TDLD propagation channel, TDLD is better than TDLA by around 0.5 dB for MCS 23. Also, TDLD is a LOS channel model not very suitable for FR1 conditions.

Observation #2: Performance with TDLD is better than TDLA by 0.5 dB for MCS 23.

Based on the performance results and observations, we propose to define PDSCH demod requirements with 1024QAM for MCS23 with TDLA channel model.

Proposal #2: Define PDSCH demod requirements for 1024QAM with MCS23 and TDLA30-10 channel model.

# 3. Conclusion

In this paper, we provide our views on the open issues for PDSCH demod requirements with 1024QAM. Our observations and proposals are captured below:

Proposal #1: Use TX EVM as 2% in simulation assumptions for defining PDSCH demod requirements with 1024QAM.

Observation #1: Operating SNR with MCS 24 and 25 with 2RX are high and not feasible to define requirements.

dB for MCS 23. Also, TDLD is a LOS channel model not very suitable for FR1 conditions.

Proposal #2: Define PDSCH demod requirements for 1024QAM with MCS23 and TDLA30-10 channel model.

# Reference

- R4-2120699, “WF on DL1024QAM demodulation and CQI reporting requirements”, Ericsson.

- R4-2120770, “CR for TS 38.104: 1024QAM”, Huawei, HiSilicon.