3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200269

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.12.2.1.1

Source:	Apple Inc.

Title:	Discussion on PDSCH requirements in intercell interference scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-e PDSCH demodulation requirements with MMSE-IRC for inter-cell interference was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues, simulation assumptions for requirements definition and simulation results.

# 2. Discussion

## Common Test Parameters

In RAN4#101-e the agreements for common parameters were:

- Network Type - Synchronized network is baseline assumption, interested companies are encouraged to bring results for async scenario under the baseline assumption of MMSE-IRC receiver. - Propagation condition - Use TDLC300-100 for homogeneous deployment assumptions and TDLA30-10 for heterogeneous deployment assumptions

Open issues on common test parameters are:

- Network Type

- SSB Configuration

Network Type

Synchronized network for FDD and TD is the baseline assumption for requirements in ICI with MMSE-IRC receiver. For ICI scenarios the interference and target cells might not be synchronized but considering synchronized network would have the worse impact on UE performance. Moreover, we don’t see any impact to MMSE-IRC receiver processing for asynchronized network. We don’t see the purpose of considering asynchronous network for ICI scenarios.

Proposal #1: Only consider synchronized network for ICI requirements.

SSB Configuration

The SSB configuration for interference cell was discussed with the following options in [1]:

- SSB Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

- SSB Option 2: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

In [1] the SSB configuration was discussed and following options for requirements definition were presented:

- Option 1: Use SSB Option 1 for all test

- Option 2: Use SSB Option 2 for all test

- Option 3: Use different assumptions for different deployment scenarios:

  - Option 3A: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

  - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions

We already agreed that TRS from serving and interfering cells are overlapping. We prefer to keep SSB non overlapping as the impact to UE demodulation is unknown. The tracking loops might be affected in the actual test with overlapping SSBs and depending on the operating SINR. Give than SSB configuration is very flexible and multiple SSB configurations are possible between cells, we don’t see the necessity to have overlapping SSB for target and interfering cells.

Proposal #2: Configure non-overlapping SSBs on target and interfering cells.

## Interference model

In RAN4#101-e the agreements for common parameters were:

- Deployment - Introducing test cases with different parameters for Homogenous scenario and HetNet scenario with minimized test cases: - One test case applied for Homogenous for each duplex mode and 2Rx/4Rx - One test case applied for HetNet for each duplex mode and 2Rx/4Rx - If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode - INR values for HetNet Deployment - Baseline option: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

Open issues related to interference model are:

- INR values for Homogeneous deployment

- Number of explicitly modeled interference cells

- Time and frequency offsets for synchronized network

INR Values for Homogeneous Deployment

The INR values for homogeneous network was discussed and following options were finalized:

- Further discuss the following options for PDSCH requirements definition for synchronous network

  - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

  - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

Number of explicitly modelled interference cells

Options discussed for number of interference cells:

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: 2 interference cell for homogeneous deployment assumptions and 1 interference for heterogeneous deployment assumptions

    - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions

Based on the simulation assumptions agreed, we present results for homogenous deployment scenario with 1 and 2 interference cells with different interference levels.

Channel Model: TDLC300-100

Scenario 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

Table 1: Simulation Results for HomNet Scenario 1

| Ant. Conf. | Num Interf Cells | SNR (dB) |  | Gain | SINR with MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
|  |  | MMSE | MMSE-IRC |  |  |
| 2x2 | 1 | 10.6 | 9.8 | 0.8 | 3.3 |
|  | 2 | 13.4 | 12.9 | 0.5 | 3.5 |
| 2x4 | 1 | 6.9 | 5.4 | 1.5 | -1.2 |
|  | 2 | 9.8 | 8.2 | 1.6 | -1.1 |

The operating SINR and gain with MMSE-IRC are very comparable between 1 and 2 interference cells.

Scenario 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

Table 2: Simulation Results for HomNet Scenario 2

| Ant. Conf. | Num Interf Cells | SNR (dB) |  | Gain | SINR with MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
|  |  | MMSE | MMSE-IRC |  |  |
| 2x2 | 1 | 9.0 | 8.6 | 0.3 | 3.8 |
|  | 2 | 11.4 | 11.0 | 0.5 | 3.8 |
| 2x4 | 1 | 5.5 | 4.4 | 1.1 | -0.4 |
|  | 2 | 7.7 | 6.6 | 1.1 | -0.5 |

For scenario 2 as well the operating SINR and gain with MMSE-IRC are very comparable between 1 and 2 interference cells.

Since the INR levels are higher in scenario 2 the gain with MMSE-IRC is more compared to scenario 1 as expected.

Observation #1: The operating SINR and gain with MMSE-IRC are very comparable between 1 and 2 interference cells in both scenarios.

Observation #2: The gain with MMSE-IRC is more with scenario 1 compared to scenario 2 since the INR levels are higher.

The number of interference cells doesn’t impact UE processing with MMSE-IRC and given the comparable gains and operating SINR, we prefer to simplify the test setup and only enable 1 interference cell. Given the above observation, we recommend defining requirements for homogenous deployment with 1 interference cell with INR of 5.49 dB.

Proposal #3: For homogeneous network define requirements with 1 interference cell with INR 5.49 dB.

Proposal #4: For HetNet deployment define requirements with 1 interference cell.

Time and Frequency offset for synchronized network

In the WF [1] from last meeting the following were agreed:

  - FDD 15 kHz

    - Time offset: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

    - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

  - TDD 30 kHz

    - Time offset:

      - Option 1: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

      - Option 2: The serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

      - Other options are not precluded

    - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

For synchronized network with TDD, the time offset of 3us in TDD with 30KHz SCS would no longer meet the synchronized network assumption and option 2 above would be more suitable. Hence, we propose to use time offset of interference cell as 1us.

Proposal #5: Define requirements with time offset of interference cell in TDD as 1us.

## Simulation Results

For the simulation assumptions agreed in [1] and also captured in the appendix, we present the simulation results for FDD and TDD for 1 and 2 interference cells.

HomoNet Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

HomoNet Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

HetNet: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

FDD

Time offset: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

Table 3: Simulation results with FDD

| Ant. Config. | # Interf Cells | Deployment | Prop. Channel | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE | MMSE-IRC | Gain |
| 2x2 | 1 | HomoNet Opt 1 | TDLC300-100 | 10.6 | 9.8 | 0.8 |
| 2x4 | 1 | HomoNet Opt 1 | TDLC300-100 | 6.9 | 5.4 | 1.5 |
| 2x2 | 1 | HomoNet Opt 2 | TDLC300-100 | 9.0 | 8.6 | 0.3 |
| 2x4 | 1 | HomoNet Opt 2 | TDLC300-100 | 5.5 | 4.4 | 1.1 |
| 2x2 | 1 | HetNet | TDLA30-10 | 9.1 | 8.2 | 0.9 |
| 2x4 | 1 | HetNet | TDLA30-10 | 5.4 | 3.2 | 2.2 |
| 2x2 | 2 | HomoNet Opt 1 | TDLC300-100 | 13.4 | 12.9 | 0.5 |
| 2x4 | 2 | HomoNet Opt 1 | TDLC300-100 | 9.8 | 8.2 | 1.6 |
| 2x2 | 2 | HomoNet Opt 2 | TDLC300-100 | 11.4 | 11.0 | 0.5 |
| 2x4 | 2 | HomoNet Opt 2 | TDLC300-100 | 7.7 | 6.6 | 1.1 |
| 2x2 | 2 | HetNet | TDLA30-10 | 15.6 | 14.3 | 1.2 |
| 2x4 | 2 | HetNet | TDLA30-10 | 11.8 | 8.0 | 3.7 |

TDD

Time offset: The serving cell is 1us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

Table 4: Simulation results with TDD

| Ant. Config. | # Interf Cells | Deployment | Prop. Channel | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE | MMSE-IRC | Gain |
| 2x2 | 1 | HomoNet Opt 1 | TDLC300-100 | 11.1 | 10.4 | 0.6 |
| 2x4 | 1 | HomoNet Opt 1 | TDLC300-100 | 7.4 | 6.1 | 1.3 |
| 2x2 | 1 | HomoNet Opt 2 | TDLC300-100 | 9.5 | 9.1 | 0.4 |
| 2x4 | 1 | HomoNet Opt 2 | TDLC300-100 | 5.8 | 5.3 | 0.5 |
| 2x2 | 1 | HetNet | TDLA30-10 | 9.4 | 8.6 | 0.8 |
| 2x4 | 1 | HetNet | TDLA30-10 | 6.0 | 3.9 | 2.1 |
| 2x2 | 2 | HomoNet Opt 1 | TDLC300-100 | 14.0 | 13.4 | 0.6 |
| 2x4 | 2 | HomoNet Opt 1 | TDLC300-100 | 10.1 | 9.0 | 1.1 |
| 2x2 | 2 | HomoNet Opt 2 | TDLC300-100 | 11.8 | 11.5 | 0.4 |
| 2x4 | 2 | HomoNet Opt 2 | TDLC300-100 | 8.2 | 7.1 | 1.1 |
| 2x2 | 2 | HetNet | TDLA30-10 | 15.8 | 14.6 | 1.2 |
| 2x4 | 2 | HetNet | TDLA30-10 | 12.2 | 8.6 | 3.6 |

# 3. Conclusion

In this paper, we provide our views on the open issues, simulation assumptions for requirements definition and simulation results. Our observations and proposals are captured below:

Proposal #1: Only consider synchronized network for ICI requirements.

Proposal #2: Configure non-overlapping SSBs on target and interfering cells.

Observation #1: The operating SINR and gain with MMSE-IRC are very comparable between 1 and 2 interference cells in both scenarios.

Observation #2: The gain with MMSE-IRC is more with scenario 1 compared to scenario 2 since the INR levels are higher.

Proposal #3: For homogeneous network define requirements with 1 interference cell with INR 5.49 dB.

Proposal #4: For HetNet deployment define requirements with 1 interference cell.

Proposal #5: Define requirements with time offset of interference cell in TDD as 1us.

# Reference

- R4-2120707, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel.

# Appendix

Table A-1: Simulation parameters for PDSCH requirements in ICI

| Parameter | Target Cell | Interference Cell(s) |
| --- | --- | --- |
| Duplex mode | FDD TDD |  |
| Active DL BWP index | 1 | 1 |
| CBW/SCS | FDD:10 MHz/ 15 KHz TDD: 40MHz/ 30KHz |  |
| RB allocation | Full bandwidth |  |
| MIMO layer | Rank 1 | Mixed rank: {70% Rank 1, 30% Rank 2} |
| Antenna Config and Correlation | 2x2, ULA Low 2x4, ULA Low |  |
| Channel | TDLA-30-10 TDLC-300-100 |  |
| SNR | Simulation parameter | Homogenous deployment: Opt1: 5.49/{7.77,2,29} Opt2: 3.1/{5.43, -1.5} Heterogeneous deployment: 4.84/{11.39, 5.45} |
| Frequency Offset | 0 | Cell 1: 300Hz Cell 2: -100 Hz |
| Time Offset | 0 | FDD: 3us, -1us TDD: 1us, -0.25us |
| PDSCH configuration |  |  |
| Mapping type | Type A | Type A |
| Starting symbol (S) | 2 | 2 |
| Length (L) | 12 | 12 |
| PDSCH DMRS configuration |  |  |
| DMRS Type | Type 1 | Type 1 |
| Number of additional DMRS | 1 | 1 |
| Maximum number of OFDM symbols for DL front loaded DMRS | 1 | 1 |
| DMRS Ports | 1 Layer:1000 | 1 Layer:1000 2 Layers 1000,1001 |
| Precoding Model |  |  |
| Granularity | 2 PRB |  |
| Type | Randomly generated per slot from SP Type I codebook |  |
| HARQ Parameters |  |  |
| Number of HARQ Processes | 4 | N/A |
| Maximum HARQ transmission | 4 | N/A |