3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200281

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.20.4

Source:	Apple Inc.

Title:	On impact to demod requirements for Reduced capability devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In this first meeting to discuss demod requirements for Reduced capability devices in NR we provide our views on the scope of demod and CSI requirements.

# 2. Discussion

For reduced capability devices in NR the following features have been introduced

- Reduced UE bandwidth

- Duplex operation

- Reduced number of Rx branches

Table 1: Red Cap UE features

| Feature | Details | Impact to demod requirements |
| --- | --- | --- |
| Reduced UE bandwidth | In FR1 maximum supported UE BW is 20MHz In FR2 maximum supported UE BW is 100MHz | In FR1 TDD requirements are defined mostly for 40MHz CBW |
| Duplex Operation | HD-FDD operation | Doesn’t impact demod requirements |
| Reduced RX | Max number of RX branches supported could be 1 or 2 | For FR1 and FR2 we don’t have 1 RX requirements. |

For reduced capability UEs we need to introduce requirements with 1RX with BW 10MHz for FDD and 20MHz for TDD. Since Red Cap UEs can also support 2 RX, the current TDD requirements might not be applicable to Red Cap UEs with 2RX since most are for 40MHz CBW.

Proposal #1: For reduced capability devices introduce requirements with 1RX for FDD 10MHx CBW and TDD 20MHz CBW.

Proposal #2: For reduced capability devices capable of 2 RX introduce requirement in TDD with 20MHz CBW.

For single carrier we have a number of requirements for PDSCH demod and CSI reporting. We tabulate the different categories of requirements and provide views if they should be introduced for Red Cap in the table below.

Table 2: Demodulation Requirements

| Requirement | FR applicability | Applicability to RedCap UE and Comments |
| --- | --- | --- |
| Minimum requirements for PDSCH Mapping Type A | Both FR | All mandatory features with and without capability signalling are applicable to RedCap UE. Define requirements |
| Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH | FR1 only | Applicable to Red Cap UE FFS for requirements |
| Minimum requirements for PDSCH Mapping Type B | Both FR | Applicable to Red Cap UE Don’t define requirements |
| Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence | FR1 only | Applicable to Red Cap UE FFS for requirements |
| Minimum requirements for PDSCH 0.001% BLER | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH repetitions over multiple slots | Both FR | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH Mapping Type B and UE processing capability 2 | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH pre-emption | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH HST-SFN | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for HST-DPS | FR1 only | Don’t define requirements |
| Minimum requirements for PDSCH Single-DCI based SDM scheme | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH Multi-DCI based transmission scheme | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH with single-DCI based FDM Scheme A | FR1 only | Optional feature Don’t define requirements |
| Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme | FR1 only | Optional feature Don’t define requirements |
| PDCCH minimum requirements with 1 Tx Antenna | Both FR | Applicable to Red Cap UE Define requirements |
| PDCCH minimum requirements with 2 Tx Antenna | Both FR | Applicable to Red Cap UE Define requirements |
| PDCCH Minimum requirements for power saving | Both FR | FFS for requirements |
| PBCH minimum requirements | Both FR | Applicable to Red Cap UE Define requirements |
| SDR requirements | Both FR | Existing requirements are applicable. |

For UE demodulation requirements further discuss on defining requirements for the following:

- Minimum requirements for PDSCH Mapping Type A

- Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

- Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence

- PDCCH minimum requirements with 1 Tx Antenna

- PDCCH minimum requirements with 2 Tx Antenna

- PDCCH	minimum requirements for power saving

- PBCH minimum requirements

Proposal #3: For UE demodulation requirements further discuss on defining the following requirements for Red Cap UE:

- Minimum requirements for PDSCH Mapping Type A

- Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

- Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence

- PDCCH minimum requirements with 1 Tx Antenna

- PDCCH minimum requirements with 2 Tx Antenna

- PDCCH Minimum requirements for power saving

- PBCH minimum requirements

Table 3: CSI Reporting Requirements

| Requirement | FR applicability | Applicability to RedCap UE and Comments |
| --- | --- | --- |
| CQI reporting requirements in AWGN with Table 2 | Both FR | Applicable to Red Cap UE Define requirements |
| CQI reporting requirements in AWGN with Table 3 | FR1 only | Optional for Red Cap UE Don’t define requirements |
| CQI reporting in fading channel for wideband CQI | Both FR | Applicable to Red Cap UE Define requirements |
| CQI reporting in fading channel for subband CQI | Both FR | Applicable to Red Cap UE Define requirements |
| Single PMI with 4TX TypeI-SinglePanel Codebook | FR1 only | Applicable to Red Cap UE Define requirements |
| Single PMI with 2TX TypeI-SinglePanel Codebook | FR2 only | Applicable to Red Cap UE Define requirements |
| Single PMI with 8TX TypeI-SinglePanel Codebook | FR1 only | Applicable to Red Cap UE Don’t requirements |
| Multiple PMI with 16TX TypeI-SinglePanel Codebook | FR1 only | Optional feature Don’t requirements |
| Single PMI with 32TX TypeI-SinglePanel Codebook | FR1 only | Optional feature Don’t define requirements |
| Multiple PMI with 16TX TypeII Codebook | FR1 only | Optional feature Don’t define requirements |
| Multiple PMI with 16TX Enhanced Type II Codebook | FR1 only | Optional feature Don’t define requirements |
| Rank Reporting requirements | Both FR | Not applicable to UE supporting 1 RX only Don’t define requirements |

For CSI reporting requirements further discuss and define requirements for the following:

- CQI reporting requirements in AWGN with Table 2

- CQI reporting in fading channel for wideband CQI

- CQI reporting in fading channel for subband CQI

- Single PMI with 2TX TypeI-SinglePanel Codebook

- Single PMI with 4TX TypeI-SinglePanel Codebook

Proposal #4: For CSI reporting requirements further discuss and define the following requirements for Red Cap UE:

- CQI reporting requirements in AWGN with Table 2

- CQI reporting in fading channel for wideband CQI

- CQI reporting in fading channel for subband CQI

- Single PMI with 2TX TypeI-SinglePanel Codebook

- Single PMI with 4TX TypeI-SinglePanel Codebook

# 3. Conclusion

In this paper, we provide our views on the scope of demod and CSI requirements for reduced capability devices in NR. Our proposals are captured below:

Proposal #1: For reduced capability devices introduce requirements with 1RX for FDD 10MHx CBW and TDD 20MHz CBW.

Proposal #2: For reduced capability devices capable of 2 RX introduce requirement in TDD with 20MHz CBW.

Proposal #3: For UE demodulation requirements further discuss on defining the following requirements for Red Cap UE:

- Minimum requirements for PDSCH Mapping Type A

- Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH

- Minimum requirements for PDSCH Mapping Type A and LTE-NR coexistence

- PDCCH minimum requirements with 1 Tx Antenna

- PDCCH minimum requirements with 2 Tx Antenna

- PDCCH Minimum requirements for power saving

- PBCH minimum requirements

Proposal #4: For CSI reporting requirements further discuss and define the following requirements for Red Cap UE:

- CQI reporting requirements in AWGN with Table 2

- CQI reporting in fading channel for wideband CQI

- CQI reporting in fading channel for subband CQI

- Single PMI with 2TX TypeI-SinglePanel Codebook

- Single PMI with 4TX TypeI-SinglePanel Codebook

# Reference