3GPP TSG-RAN WG4 Meeting # 101-bis-e					R4-2200286

Electronic Meeting, January 17-25, 2022

Agenda item:	4

Source: 	Apple

Title: 	Initial views on R17 feature list

Document for:	Discussion

- Introduction

In this document, we share our initial views on Rel-17 NR UE features.

- NR UE feature

  - Extending current NR operation to 71GHz

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X.Extending current NR operation to 71GHz | X-1 | UE support of max. CBW for supported SCS | Capability of supported max. CBW - 400MHz for 120kHz SCS - 1600MHz for 480kHz SCS - 2000MHz for 960kHz SCS Note: this capability may need to be split into three capabilities, i.e. one for each supported SCS |  | yes | no | UE cannot transmit or receive with the max. CBW | per Band | No | No |  |  | Optional with capability signalling |
|  | X-4 | UL support of 64QAM in the UL | Capability of support of 64QAM in the UL |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | X-5 | Active BWP switching delay | Reuse R15 capability bwp-SwitchingDelay There is another capability bwp-SwitchingMultiCCs-r16 in R16 which is likely to be reused as well, pending further discussion |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | X-6 | Initial transmit timing error | Depend on the outcome of ongoing discussion. There is a possibility of having UE capability on Te and the supported SSB and UL SCS combinations |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

- We note that proposals are under discussion to potentially introduce UE ON/ON transient time for 480/960 SCS. In our understanding, this may not be necessary, and further discussion about the potential gains from such a capability is still needed.

- UE beam direction-only switching time was discussed during the RAN4 #100 meeting, with a majority of companies expressing concerns with introducing a capability which exceeds the FR2-1 baseline.

  - NR and MR-DC Measurement Gap Enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X R17 NR and MR-DC Measurement Gap Enhancements | X-1 | pre-configured MG | - Support of NW-controlled pre-configured MG activation/deactivation - Support of UE autonomous pre-configured MG activation/deactivation | N/A | Yes | N/A | UE cannot support pre-configured MG. | Per UE | No | No | N/A | The capability is to indicate UE support NW-controlled pre-configured MG activation/deactivation and/or UE autonomous pre-configured MG activation/deactivation. | Optional with capability signalling |
|  | X-2 | multiple independent and concurrent gaps | - Support of multiple independent and concurrent gaps | N/A | Yes | N/A | UE cannot support multiple independent and concurrent gaps | Per UE | No | No | N/A | The capability is to indicate UE support multiple independent and concurrent gaps. | Optional with capability signalling |
|  | X-3 | multiple independent and concurrent gaps | - Indicate the max number of supported concurrent gap across all FRs | [X-2] | Yes | N/A | NW doesn’t know the max number of supported concurrent gap across all FRs | Per UE | No | No | N/A | The capability is to indicate max number of supported concurrent gap across all FRs. The candidate values are: ● {3, 4} It applies only to per-FR gap capable UE | Optional with capability signalling |
|  | X-4 | multiple independent and concurrent gaps | - Indicate the supported maximum MGL overhead | [X-2] | Yes | N/A | NW doesn’t know the maximum MGL overhead when configuring multiple independent and concurrent gaps | Per UE | No | No | N/A | The capability is to indicate the supported maximum MGL overhead. The candidate values are: ● {type1, type2} Type1 means the supported maximum MGL overhead is same as the max overhead that UE can support in Rel-15/16. Type 2 means the MGRP for each MG cannot be smaller than 40ms | Optional with capability signalling |
|  | X-5 | NCSG | - Support of NCSG | N/A | Yes | N/A | UE cannot support NCSG | Per UE | No | No | N/A | The capability is to indicate UE support NCSG. | Optional with capability signalling |
|  | X-6 | NCSG | - Indicate the supported NCSG patterns | [X-5] | Yes | N/A | UE can only support NCSG pattern #0 and #1 | Per UE | No | No | N/A | The capability is to indicate the supported NCSG patterns. It is a bit string (size 22). The leading / leftmost bit (bit 0) corresponds to the NCSG pattern 2, the next bit corresponds to the NCSG pattern 3, as specified in TS 38.133 and so on. | Optional with capability signalling |

  - NR RF requirement enhancements for frequency range 1 (FR1)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- In the context of requirements for intra-band UL CA + UL MIMO, differentiation of requirement applicability based on PA architecture may be needed. It is recommended to revisit this issue once the related technical discussion converges.

  - NR RF requirement enhancements for frequency range 2 (FR2)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL gap for Tx power management | 2-1 | Support of UL gap for Tx power management | Capability of performing BPS sensing for Tx power management |  | yes | no | UE does not support UL gap for Tx power management | FR2 only | No | No |  |  | Optional with capability signalling |
|  | 2-2 | Support of UL gap configurations | Capability of support specific UL gap configuration | 2-1 | yes | no |  | FR2 only | No | No |  |  | Optional with capability signalling |
| UL gap for coherent UL MIMO | 2-3 | Support of UL gap for coherent UL MIMO | Capability of performing coherent UL MIMO calibration in UL gap |  | yes | no | UE does not support UL gap for coherent UL MIMO calibration | FR2 only | No | No |  |  | Optional with capability signalling |

- A capability to allow the UE to signal a frequency separation class for inter-band CA combinations (FS_inter_CBM) based on the CBM assumption had been proposed in the past. Based on our understanding that CBM requirements will incorporate a dependency on the frequency separation between the inter-band CBM carriers, we do not consider such a capability necessary.

- According to our understanding, if we define CBM requirements based on the multi-chain assumption (and the associated relaxation values for EIS), then it should not be necessary to define new signaling for CBM with multi-chain.

It is recommended to revisit these issues once the related technical discussion converges.

  - Further RRM enhancement for NR and MR-DC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-17 Further RRM enhancement for NR and MR-DC | x-1 | SRS antenna port switching | 1) Support RRM requirements of SRS antenna port switching | [Rel-15 NR RAN1 UE feature list feature 2-55 (SRS Tx switch)] | Yes | N/A | Network cannot know the interruption time when SRS antenna port switching happens for this UE. Therefore, either network may not trigger SRS antenna port switch or there will be performance degradation | Per UE | No | FR1 | N/A | Functionality of SRS antenna port switching has already been supported since R15. RRM requirement is expected to be introduced in R17. Thus, R17 UE shall meet corresponding RRM requirement. The requirements apply when SRS antenna port switching is on NR carrier. | Optional with capability signalling |
|  | x-2 | Handover with PSCell | 1) Support of RRM requirements of handover with PSCell, including the following scenarios: - from NR SA to EN-DC - from EN-DC to EN-DC - from NE-DC to NE-DC - from NR-DC to NR-DC |  | Yes | N/A | Network cannot know the delay for handover with PSCell addition/change and corresponding interruption length for this UE. Therefore, either network may not trigger handover with PSCell addition/change or there will be performance degradation | Per UE | No | Yes | N/A | Functionality of handover with PSCell addition/change has already been supported since R15. RRM requirement is expected to be introduced in R17. Thus, R17 UE shall meet corresponding RRM requirement. | Optional with capability signalling |
|  | x-3 | PUCCH SCell activation | 1) Support of RRM requirement of PUCCH SCell activation |  | Yes | N/A | Network cannot know the PUCCH SCell activation delay and corresponding interruption length for this UE. There will be performance degradation when PUCCH SCell activation is commanded. | Per UE | No | No | N/A | Functionality of PUCCH SCell activation has already been supported since R15. RRM requirement is expected to be introduced in R17. Thus, R17 UE shall meet corresponding RRM requirement. | Optional with capability signalling |
|  | x-4 | CSI reporting cross PUCCH groups | 1) Support reporting CSI of a SCell belonging to secondary/primary PUCCH group by PUSCH or PUCCH of active serving cells belonging to primary/secondary PUCCH group |  | Yes | N/A | Network cannot know which serving cell could be used for CSI reporting of SCell belonging to secondary/primary PUCCH group. There will be performance degradation or activation failure when CSI reporting of being-activated SCell is configured during the PUCCH SCell activation. | Per UE | No | No | N/A | The capability is to indicate whether UE could support reporting CSI of a being activated SCell belonging to secondary/primary PUCCH group by PUSCH or PUCCH of active serving cells belonging to primary/secondary PUCCH group during the PUCCH SCell activation. | Optional with capability signalling |

  - Further enhancement on NR demodulation performance

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X NR CRS-IM | X-1 | Advanced receiver for CRS IM in NR | LLR weighing for CRS interference mitigation from neighbour cell CRS in NR UE | N/A | No | N/A | UE cannot perform CRS interference mitigation | Per FSPC (per CC per band per BC) | No | Applicable only to FR1 | N/A | Applicable only for 15KHz SCS | Optional without capability signalling |

  - Increasing UE power high limit for CA and DC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Because the work item on increasing UE power high limit for CA and DC has just started, we recommend returning to this topic of signaling related to this feature in the next meeting.

  - Tx Diversity

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TxD | X-1 | TxD support per band per band combination | Support of transmit diversity per band per band combination | TxD support per band | Yes | No | UE uses a single Tx | Per FSPC (per CC per band per BC) | No | FR1 only | N/A |  | Optional with capability signalling |

- New signaling was agreed to be introduced in Rel-17, and RAN2 has introduced a per-band optional capability with a restriction to FR1 only

- In the case of a UE supporting TxD in a particular band within a band combination, we should consider the scenario of a UE falling back to single Tx operation (i.e. in a single carrier configuration the UE supports TxD, but when used with inter-band CA the UE falls back to single Tx); we do have this signaling for EN-DC, but not for inter-band CA; thus, we need an additional per band per band combination capability that indicates that the UE supports TxD in band X applicable to all inter-band UL CA combinations

  - HPUE FDD

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HPUE FDD | X-1 | Hybrid duplex operation | Support of hybrid duplex operation | N/A | Yes | No | UE does not support hybrid duplex operation | Per Band | FDD only | FR1 only | N/A |  | Optional with capability signalling |

We propose to introduce a new UE capability of supporting hybrid duplex operation for PC2 FDD bands.  It can indicate to the need to configure UE to half-duplex mode or back to full-duplex mode as needed. The mode configuration can potentially be done semi-statically through RRC reconfiguration or dynamically through MAC-CE process.

  - Coverage enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage enhancement | X-1 | Maximum duration of UE transmission for joint channel estimation | UE supports the indicated maximum duration of transmission for joint channel estimation: [{2, 4, 8, 16, 32} slots] | N/A | Yes | No | UE does not support transmission for joint channel estimation | Per Band | No | No | N/A |  | Optional with capability signalling |

With the discussion related to UE capabilities for the Coverage Enhancement feature still ongoing, it is recommended to return to this topic in the next meeting.