3GPP TSG-RAN WG4 Meeting # 101-bis-e 	R4-2200313

# Electronic Meeting, 17th – 25th January 2022

Source:		Verizon, Samsung, Ericsson

Title:			TP for TR 37.827 for DC_2-13_n66-n77

Agenda item:		5.33.1

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.827 to introduce additional DC_2-13_n66-n77 configuration according to the request in [1] and based on the approved text proposals in [2].

- Reference

[1] RP-213467, Revised WID: Power Class 2 for EN-DC with x LTE band + y NR  TDD band

[2] TPs to 37.827

- R4-2108901, TP for TR 37.827 for DC_2-13_n66-n77,

# Text Proposal

<Start of Text Proposal>

## 5.9	DC_2-13_n66-n77

### 5.9.1	Transmitter Characteristics

#### 5.9.1.1	Maximum Output Power

Table 5.9.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.9.1.2		Configurations for EN-DC

Table 5.9.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A_n66A-n77A DC_2A-2A-13A_n66A-n77A DC_2A-13A_n66A-n77C | DC_2A_n77A DC_13A_n77A |

#### 5.9.1.3	Co-existence study

Co-existence study for DC_2-13_n66-n77 was covered by the studies of lower fallback modes.

### 5.9.2	Receiver Characteristics

#### 5.9.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

<End of Text Proposal>