3GPP TSG-RAN WG4 Meeting #101Bis-e	R4-2200344

Jan 2022

Title:	Maximum duration handling for JCE

Source:	Qualcomm Incorporated

Agenda item:	6.18.2.4

Release:	Rel-17

Work Item:	NR_cov_enh

Responsible WG:	RAN1

Document for:	Approval

# 1.   Introduction

Issue 1-5-1: What factors determine the maximum duration

- Option 1: The maximum time the UE not adjusting its frequency/time

- Option 2: Phase and power tolerance within the duration

Issue 1-5-3: Whether the maximum duration is dependent on the modulation order of transmission

- No.

- Note: It has been agreed to only focus on the modulation orders not higher than QPSK.

The length of maximum duration is:

- Option 1: Band specific

- Option 2: FR specific

Down select among the following options once we have the results of the simulations:

- Option 1: Subject to a single maximum duration value.

  - The value is defined for a given set of factors which are depends on the conclusion for the other issues under discussion.

- Option 2: Subject to multiple maximum duration value and UE could report the supported value(s)

WF [15] in RAN4#101-e concluded the following agreement for the OFF power requirement in the gap between slots in the same bundle:

In this paper we discuss.

# 2. 	Discussion

Maximum duration depends on when JCVE still bring benefits in the presence of an implementation with real world impairments or in other words deviations from ideal performance. In this sence the issue 1-5-1 in WF [15] is slightly strange since both option 1 and 2 phenomena impact. But the underlying analysis and criteria how to define maximum duration depend on requirements that need to be agreed. Our proposals has been to allow 1 dB loss of SNR in link simulation for the same throughput because UE non-idealities. We discuss this issue in companion paper [20] and the data in that paper indicates that for fixed value of phase and amplitude degradation, the SNR loss will increase with more slots. Once Ran4 has agreed the requirements for acceptable EVM or phase and amplitude variation, it should be up to the UE to declare its capability. In the Figure 1 we show the SNR degradation with different UE variation values.

Figure 1. Increase in required CINR with same phase discontinuity for longer bundles

This same data is shown in [20] and also we can look at the table 3 from the same source and conclude that the setting a fixed EVM requirement, maximum duration can be defined by UE capability and requirement for the UE.

Table 3. EVM with 2 dB amplitude variation for DFT-s, X = phase variation

| Bundle length (number of slots) | X=10 ⁰ |  | X=20 ⁰ |  |
| --- | --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 1 | Option 2 |
| 1 | -16.3 | -16.3 | -16.3 | -16.3 |
| 2 | -12.1 | -12.2 | -11.1 | -11.7 |
| 4 | -10.7 | -10.8 | -9.6 | -9.8 |
| 8 | -10.2 | -9.9 | -9.0 | -8.4 |
| 16 | -9.8 | -8.9 | -8.6 | -6.6 |

Observation: Setting one fixed performance criteria for DMRS bundling independent of number of bundled slots will define how many slots UE can support in one bundle

Proposal: Maximum duration for the UE is determined by UE meeting the requirements set for DMRS bundling

# Conclusion

We discussed maximum duration of the DMRS bundling and made one observation and one proposal

Observation: Setting one fixed performance criteria for DMRS bundling independent of number of bundled slots will define how many slots UE can support in one bundle

Proposal: Maximum duration for the UE is determined by UE meeting the requirements set for DMRS bundling

# References

[1]	R4-2100014, ”LS on PUCCH and PUSCH repetition”, RAN1, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[2]	R4-2103393, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[3]	R4-2103363, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[4]	R4-2105417, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#98-Bis-e

[5]	R4-2105418, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition” Huawei, RAN4#98-Bis-e

[6]	R4-2107611, “Reply LS on PUCCH and PUSCH repetition”, RAN1, RAN4#99e

[7]	R4-2107880, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#99e

[8]	R4-2107881, “WF on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, RAN4#99e

[9]	R4-2111194, “Reply LS to RAN1 latest question on phase discontinuity”, Ericsson, RAN4#99e

[10]	R4-2111706, “LS on joint channel estimation for PUSCH and PUCCH”, RAN1, RAN4#100-e, Online, Aug 2021

[11] 	R4-2114991, “Reply LS on PUCCH and PUSCH transmissions”, Qualcomm, RAN4#100-e, Online, Aug 2021

[12]	R4-2114992, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, HiSilicon, RAN4#100-e, Online, Aug 2021

[13]	R4-2117006, “Reply LS on PUCCH and PUSCH repetition”. RAN1, RAN4#101e, Nov 2021

[14]	R4-2120002, ”Reply LS on PUCCH and PUSCH transmissions”, RAN4 (Qualcomm Incorporated), 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[15]	R4-2120003, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[16]	R4-2119494, “UE requirement for phase continuity”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[17] R4-2200339, “draft CR for EVM based requirements”, Qualcomm, RAN4#101Bis-e, Jan, 2022

[18]	R4-2119495, “Simulation results and criterias for setting requriements for UE”, Qualcomm Incorporated, RAN4#101e, Nov 2021

[19]	R4-2114550, “Requirements for phase continuity for transmission repetitions”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #100-e, Aug 2021

[20]	R4-2200338, “Discussion on UE requirement for JCE	“, Qualcomm Incorporated, RAN4#101Bis-e, Jan 2022