3GPP TSG-WG RAN4 Meeting #101-bis-e	R4-2200349

# Online, 17 – 25 January, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 					TP to TR 36.717-02-01: CA_2-38

Agenda Item: 		8.1.2 [LTE_CA_R17_2BDL_1BUL]

Document for:		Approval

# Introduction

This TP introduces the 2 band E-UTRA CA configuration CA_2A-38A for 2DL/1UL.

TP to TR 36.717-02-01

## <Start of Changes>

## 5.x	CA_2-38

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-38A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.x.2 	Co-existence studies

Table 5.x.2-1 and 5.x.2-2 summarize frequency ranges for harmonics and/or harmonics mixing.

Table 5.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 |

Table 5.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 |

There is no harmonics or harmonics mixing relations for this band combination.

### 5.x.3	∆TIB and ∆RIB values

The relaxations are for the high-high band combination.

Table 5.x.3-1: IB,c

| CA_2-38 | 2 | 0.5 |
| --- | --- | --- |
|  | 38 | 0.5 |

Table 5.x.3-2: R IB,c

| CA_2-38 | 2 | 0 |
| --- | --- | --- |
|  | 38 | 0 |

### 5.x.4 	REFSENS

No REFSENS exception is needed for this band combination.

## <End of Changes>