3GPP TSG-RAN WG4 #101-bis-e	R4-2200376

Electronic Meeting, January 17-25, 2022

Agenda item:	6.6.5.2

Source:	MediaTek Inc.

Title:	Discussion on the PDSCH requirements for 1024QAM

Document for:	Discussion

- Introduction

In RAN4#101-e, a way forward on DL1024QAM was agreed [1]. RAN4 agreed to define PDSCH requirements for 1024QAM.

- Define new PDSCH requirements for 1024 QAM by using the existing new applicability rule. - Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the test parameters.

In this contribution, we present simulation results based on the agreed simulation assumptions in appendix A. Also, we provide our views for the parameters to define test cases.

- Discussion

We think TDL-D (with LOS channel) provides a suitable propagation condition for 1024QAM. Also, compare the simulation results between TDL-A and TDL-D, it can be shown that the difference of required SNR values to achieve 70% of maximum throughput between different EVM values is larger with TDL-A channel. Hence, we prefer to define test cases with TDL-D.

Proposal 1: Consider TDL-D to define PDSCH requirements for 1024QAM.

The agreements below were made on UE/BS RF core part in RAN4#101-e:

- 1024QAM BS Tx EVM is

  - 2.5%, applicable for frequencies equal to or below 4.2GHz

  - 2.8%, applicable for frequencies above 4.2GHz

We understand that the BS RF requirements are the minimum requirements for the worst case. In practical, the achievable Tx EVM could be better than the BS RF requirements. However, form the simulation results, the difference of required SNR values to achieve 70% of maximum throughput between EVM = 2.0% and EVM = 2.5%, EVM =2.0% and  EVM = 2.8% are not negligible for MCS25. Hence, we propose to consider EVM = 2.5% or 2.8 %.

Proposal 2: Consider PDSCH requirements for 1024QAM with EVM = 2.5% or 2.8%.

Table 1.	Summary for the required SNR to achieve 70% of the maximum throughput

|  |  | EVM 2% | EVM 2.5% | EVM 2.8% |
| --- | --- | --- | --- | --- |
| 2x2 TDL-A | MCS23 | 27 dB | 27.5 dB | 28 dB |
|  | MCS24 | 28.9 dB | 29.6 dB | 30.2 dB |
|  | MCS25 | 31.2 dB | 32.6 dB | 33.9 dB |
| 2x4 TDL-A | MCS23 | 23.1 dB | 23.6 dB | 23.9 dB |
|  | MCS24 | 24.6 dB | 25.1 dB | 25.6 dB |
|  | MCS25 | 26.8 dB | 27.5 dB | 28.1 dB |
| 2x2 TDL-D | MCS23 | 25.4 dB | 25.5 dB | 25.6 dB |
|  | MCS24 | 27 dB | 27.5 dB | 27.6 dB |
|  | MCS25 | 28.5 dB | 29.5 dB | 29.9 dB |
| 2x4 TDL-D | MCS23 | 22.5 dB | 22.7 dB | 23 dB |
|  | MCS24 | 23.9 dB | 24.3 dB | 24.6 dB |
|  | MCS25 | 25.6 dB | 26.2 dB | 26.8 dB |

| Figure 1. Simulation results |  |
| --- | --- |

- Conclusion

In this contribution, we provide simulation results for 1024QAM considering different MCS, EVM and propagation channel. The proposals are summarized as below:

Proposal 1: Consider TDL-D to define PDSCH requirements for 1024QAM.

Proposal 2: Consider PDSCH requirements for 1024QAM with EVM = 2.5% or 2.8%.

- Reference

- R4-2120699, “WF on DL1024QAM demodulation and CQI reporting requirements”, Ericsson

Appendix

Table 1. Simulation assumptions

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 FFS to schedule PDSCH in the special slots |
| MCS (See appendix) | Option 1: MCS23 Option 2: MCS24 Option 3: MCS25 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 and 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLA30-10 Option 2: TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | Option 1: 2.0% Option 2: 2.5% Option 3: 2.8% Note: The assumed Tx EVM should be same or less than the BS Tx EVM requirements. |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot, PRB bundling granularity (codebook configuration Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |