3GPP TSG-RAN WG4 #101-bis-e	R4-2200377

Electronic Meeting, January 17-25, 2022

Agenda item:	6.12.2.1.1

Source:	MediaTek Inc.

Title:	Discussion on the PDSCH requirements for scenarios with inter-cell inter-user interference

Document for:	Discussion

- Introduction

In RAN4#101-e, a way forward for PDSCH demodulation requirements for inter-cell interference MMSE-IRC was agreed [1]. In this contribution, we present simulation results based on the agreed simulation assumptions and provide our views for the outstanding issues.

- Discussion

After RAN4#101-e, there are some options for INR values for both homogeneous and heterogeneous networks.

INR values for Homogeneous deployment assumptions - Further discuss the following options for PDSCH requirements definition for synchronous network - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell INR values for HetNet deployment assumptions - Baseline option: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell. Baseline option can be updated in case technical issue will be observed

Also, there are options for the number of explicitly modeled interference cells.

Number of explicitly modeled interference cells - Option 1: 1 interference cell for all tests - Option 2: 2 interference cells for all tests - Option 3: Use different assumptions for different deployment scenarios: - Option 3A: 2 interference cell for homogeneous deployment assumptions and 1 interference for heterogeneous deployment assumptions - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions

Based on the simulation assumptions in Appendix B, we provide simulation results in Appendix A considering different INR values and time and frequency offset. Also, the summary for simulation results is shown in Table 1 below.

Table 1. Summary for the simulation results

| Duplex mode | Channel model | Number of UE Rx antenna | INR1 | INR2 | MCS | SNR for 70% of Max T-put |  | Gain | Corresponding SINR for SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE-MRC |  |  |
| FDD | TDLA 30-10 | 2 Rx | 11.39 | 5.45 | MCS13 | 13.8 | 17.1 | 3.3 | 1.2 |
|  |  |  | 4.84 | N/A | MCS 13 | 9.5 | 11.0 | 1.5 | 3.4 |
|  |  | 4 Rx | 11.39 | 5.45 | MCS 13 | 6.7 | 13.3 | 6.6 | -5.9 |
|  |  |  | 4.84 | N/A | MCS 13 | 5.0 | 7.6 | 2.6 | -1.1 |
|  | TDLC 300-100 | 2 Rx | 7.77 | 2.29 | MCS 13 | 13.4 | 15.2 | 1.8 | 4.0 |
|  |  |  | 5.49 | N/A | MCS 13 | 11.1 | 12.6 | 1.5 | 4.5 |
|  |  |  | 5.43 | -1.50 | MCS 13 | 11.8 | 13.4 | 1.6 | 4.6 |
|  |  |  | 3.1 | N/A | MCS 13 | 10.3 | 11.1 | 0.8 | 5.5 |
|  |  | 4 Rx | 7.77 | 2.29 | MCS 13 | 8.3 | 11.8 | 3.5 | -1.1 |
|  |  |  | 5.49 | N/A | MCS 13 | 6.5 | 9.1 | 2.6 | -0.1 |
|  |  |  | 5.43 | -1.50 | MCS 13 | 7.4 | 9.7 | 2.3 | 0.2 |
|  |  |  | 3.1 | N/A | MCS 13 | 6.1 | 7.6 | 1.5 | 1.3 |

From the simulation results, the gain of MMSE-IRC over MMSE-MRC are almost larger than 1 dB for all cases. Also, it can be shown that the SINR are larger than -6 dB for all cases. To minimize the test cases, we propose to introduce two test cases for FDD, one for 2Rx and one for 4Rx. Considering different deployment/propagation channel and number of modeled interference cells, we suggest using the following settings to define test cases for FDD.

- Test case for 2Rx: TDLA 30-10, INR1: 11.39 dB, INR2: 5.45 dB

- Test case for 4Rx: TDLC 300-100, INR1: 5.49 dB

Proposal 1: Consider INR value 5.49 dB for Homogeneous deployment assumptions.

Proposal 2: Consider INR values 11.39 dB and 5.45 dB for HetNet deployment assumptions.

Proposal 3: Consider different number of explicitly modeled interference cells for different deployment scenarios: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions.

Proposal 4: Consider two test cases for FDD with the following settings:

  - Test case for 2Rx: TDLA 30-10, INR1: 11.39 dB, INR2: 5.45 dB

  - Test case for 4Rx: TDLC 300-100, INR1: 5.49 dB

- Conclusion

In this contribution, we provide simulation results for PDSCH requirements under the scenario of inter-cell interference. Also, we provide our views on the remining issues. The proposals are summarized as below:

Proposal 1: Consider INR value 5.49 dB for Homogeneous deployment assumptions.

Proposal 2: Consider INR values 11.39 dB and 5.45 dB for HetNet deployment assumptions.

Proposal 3: Consider different number of explicitly modeled interference cells for different deployment scenarios: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions.

Proposal 4: Consider two test cases for FDD with the following settings:

  - Test case for 2Rx: TDLA 30-10, INR1: 11.39 dB, INR2: 5.45 dB

  - Test case for 4Rx: TDLC 300-100, INR1: 5.49 dB

- Reference

- R4-2120707, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel

Appendix A

| Figure 1. Simulation results for INRs 11.39 dB and 5.45 dB |  |
| --- | --- |

| Figure 2. Simulation results for INR 4.84 dB |  |
| --- | --- |

| Figure 3. Simulation results for INRs 7.77 dB and 2.29 dB |  |
| --- | --- |

| Figure 4. Simulation results for INR 5.49 dB |  |
| --- | --- |

| Figure 5. Simulation results for INRs 5.43 dB and -1.50 dB |  |
| --- | --- |

| Figure 6. Simulation results for INR 3.1 dB |  |
| --- | --- |

Appendix B

Table 1. Simulation assumptions

| Parameters | Serving cell FDD | Interferer cell(s) |
| --- | --- | --- |
| INR values | - Homogeneous deployment - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell - HetNet deployment - Baseline option: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell. |  |
| PDSCH allocation in frequency domain | Full PRB |  |
| Physical cell ID | 0 | 1 or 2 (depending on the number of interferer cells) |
| Channel bandwidth | 10MHz | 10MHz |
| SCS | 15kHz | 15kHz |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH |
| SSB configuration | Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources |  |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port FDD: Periodicity 20 slots; Offset 10 for CSI-RS resource 1 and 2; 11 for CSI-RS resource 3 and 4 | Same as Serving cell |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​ | Type A, Start symbol 2, Duration 12​ |
| PRB bundling size for target PDSCH | 2 | 2 |
| PDSCH DMRS configuration | Type 1, 1+1 | Type 1, 1+1 |
| MCS | MCS 13 (16QAM, CR=0.5) | 16QAM randomly modulated symbols |
| Rank | 1 | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| PDSCH precoder for both serving cell and interferer cells | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2 |  |
| Antenna configuration | 2T2R, 2T4R |  |
| Antenna Correlation | ULA Low |  |
| Propagation condition | TDLA30-10 for HetNet deployment TDLC300-100 for Homogeneous deployment |  |
| Number of HARQ Processes | 4 | N/A |
| Maximum HARQ transmission | 4 | N/A |
| Time and frequency offsets | - Time offset: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled) - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled) |  |