3GPP TSG-RAN WG4 #101-bis-e	R4-2200382

Electronic Meeting, January 17-25, 2022

Agenda item:	6.20.4

Source:	MediaTek Inc.

Title:	Views on UE demodulation requirements for RedCap

Document for:	Discussion

- Introduction

In RAN#92-e meeting, a WI for reduced capability NR devices was agreed [1]. In this paper, we provide our views on UE demodulation requirements for RedCap.

- Discussion

The reduced capability NR devices are aimed for the following uses cases: industrial wireless sensor network, video surveillance and wearable devices. The consideration of use case requirements drives the choices of key physical-layer parameters for RedCap, which impacts the complexity and cost of the device. The key factors to reduce UE complexity are summarized in Table 1, which includes bandwidth reduction, reducing the maximum number of MIMO layers, and the relaxation of the maximum downlink modulation order.

Table 1. Capabilities for RedCap devices

|  | FR1 | FR2 |
| --- | --- | --- |
| Maximum device bandwidth | 20MHz | 100MHz |
| Rx branch | 1 or 2 | 1 or 2 |
| Maximum number of downlink MIMO layers | 1 for a RedCap UE with 1 Rx branch 2 for a RedCap UE with 2 Rx branch | 1 for a RedCap UE with 1 Rx branch 2 for a RedCap UE with 2 Rx branch |
| Maximum downlink modulation order | 64QAM (256QAM optional) | 64QAM |
| Duplex operation | HD-FDD, FD-FDD and TDD | TDD |

Considering the capabilities for the reduced capability devices, we think it is necessary to define demodulation requirements for RedCap. Currently, in TS38.101-4, there are no test cases considering 1 Rx branch for PDCCH, PDSCH and CSI requirements. Also, for FR1, the conventional BW used for TDD is 40MHz, which is larger than the maximum bandwidth 20MHz. Hence, we propose to introduce new test cases for RedCap.

Observation 1: In TS38.101-4, there are no test cases considering 1 Rx branch for PDCCH, PDSCH and CSI requirements.

Observation 2: For FR1, the conventional BW used for TDD is 40MHz, which is larger than the maximum bandwidth 20MHz.

Proposal 1:  Define PDSCH, PDCCH and CSI requirements for RedCap.

- Conclusion

In this contribution, we provide our views on UE demodulation requirements for RedCap. The proposals are summarized as below:

Observation 1: In TS38.101-4, there are no test cases considering 1 Rx branch for PDCCH, PDSCH and CSI requirements.

Observation 2: For FR1, the conventional BW used for TDD is 40MHz, which is larger than the maximum bandwidth 20MHz.

Proposal 1:  Define PDSCH, PDCCH and CSI requirements for RedCap.

- Reference

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson