3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2200400

Electronic Meeting, January 17 – January 25, 2022

Source:	vivo

Title:	Simulation results for PDCCH RLM and BFD

Agenda item:		6.20.3.1.4

Document for:	Information

- Introduction

In this contribution, we provide PDCCH simulation results for RLM and BFD based on simulation assumptions in [1].

- Simlulation results

Table 1-2 show the simulation results of in-sync evaluation in the differet CCEs and channel models for FR1 and FR2 respectively.

Table 1：SNR for 2% BLER for FR1 （dB）

|  | CCE | AWGN |  | TDL-A 30 |  | TDL-B 100 |  | TDL-C 300 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1Rx | 2Rx | 1Rx | 2Rx | 1Rx | 2Rx | 1Rx | 2Rx |
| SCS=15KHz | 4 | -0.80 | -3.31 | 6.01 | -0.29 | 5.10 | -0.46 | 4.57 | -0.84 |
|  | 8 | -4.02 | -6.37 | 2.57 | -3.02 | 1.12 | -3.60 | 0.53 | -4.07 |
|  | 16 | -5.58 | -8.19 | -0.74 | -5.42 | -2.35 | -6.27 | -3.08 | -6.74 |
| SCS=30KHz | 4 | -0.83 | -3.36 | 5.76 | -0.35 | 4.76 | -0.82 | 4.17 | -0.94 |
|  | 8 | -4.00 | -6.40 | 2.20 | -3.16 | 0.40 | -4.15 | -0.09 | -4.28 |
|  | 16 | -5.56 | -8.20 | -1.07 | -5.50 | -3.06 | -6.69 | -3.13 | -6.77 |

Table 2：SNR for 2% BLER for FR2 (dB)

|  | CCE | AWGN |  | TDL-A 30 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1Rx | 2Rx | 1Rx | 2Rx |
| SCS=15KHz | 4 | -0.84 | -3.36 | 5.24 | -0.63 |
|  | 8 | -4.01 | -6.42 | 1.00 | -3.67 |
|  | 16 | -5.58 | -8.21 | -2.05 | -6.14 |
| SCS=30KHz | 4 | -0.84 | -3.37 | 4.90 | -0.56 |
|  | 8 | -4.01 | -6.44 | 0.79 | -3.82 |
|  | 16 | -5.56 | -8.24 | -2.28 | -6.15 |

Observation 1: The required SNR at 2% BLER is about -4dB without considering power boosting for CCE=4 for AWGN channel when 2Rx is used.

Observation 2: The required SNR at 2% BLER is about -4dB without considering power boosting for CCE=8 for AWGN channel when 1Rx is used.

Table 3-4 show the simulation results of out-of-sync evaluation in the differet CCEs and channel models for FR1 and FR2 respectively.

Table 3：SNR for 10% BLER for FR1 (dB)

|  | CCE | AWGN |  | TDL-A 30 |  | TDL-B 100 |  | TDL-C 300 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1Rx | 2Rx | 1Rx | 2Rx | 1Rx | 2Rx | 1Rx | 2Rx |
| SCS=15KHz | 4 | -1.56 | -4.24 | 2.31 | -2.49 | 1.95 | -2.64 | 1.79 | -2.61 |
|  | 8 | -4.62 | -7.24 | -0.59 | -5.10 | -1.35 | -5.38 | -1.69 | -5.52 |
|  | 16 | -6.56 | -9.19 | -3.25 | -6.52 | -4.26 | -7.78 | -4.70 | -8.05 |
| SCS=30KHz | 4 | -1.59 | -4.26 | 2.29 | -2.54 | 1.86 | -2.74 | 1.75 | -2.57 |
|  | 8 | -4.61 | -7.24 | -0.72 | -5.15 | -1.79 | -5.66 | -2.03 | -5.70 |
|  | 16 | -6.54 | -9.20 | -3.34 | -7.32 | -4.61 | -8.04 | -4.69 | -8.05 |

Table 4：SNR for 10% BLER for FR2 (dB)

|  | CCE | AWGN |  | TDL-A 30 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1Rx | 2Rx | 1Rx | 2Rx |
| SCS=15KHz | 4 | -1.58 | -4.27 | 2.05 | -2.64 |
|  | 8 | -4.62 | -7.25 | -1.31 | -5.39 |
|  | 16 | -6.56 | -9.22 | -4.02 | -7.63 |
| SCS=30KHz | 4 | -1.59 | -4.27 | 2.00 | -2.55 |
|  | 8 | -4.61 | -7.26 | -1.44 | -5.38 |
|  | 16 | -6.57 | -9.23 | -4.07 | -7.21 |

Observation 3: The required SNR at 10% BLER is about -7dB without considering power boosting for CCE=8 for AWGN channel when 2Rx is used.

Observation 4: The required SNR at 10% BLER is about -6dB without considering power boosting for CCE=16 for AWGN channel when 1Rx is used.

- Summary

Observation 1: The required SNR at 2% BLER is about -4dB without considering power boosting for CCE=4 for AWGN channel when 2Rx is used.

Observation 2: The required SNR at 2% BLER is about -4dB without considering power boosting for CCE=8 for AWGN channel when 1Rx is used.

Observation 3: The required SNR at 10% BLER is about -7dB without considering power boosting for CCE=8 for AWGN channel when 2Rx is used.

Observation 4: The required SNR at 10% BLER is about -6dB without considering power boosting for CCE=16 for AWGN channel when 1Rx is used.

References

[1] R4-2120386, “Simulation assumption on hypothetical PDCCH performance for RLM and BFD for RedCap UE”, Ericsson