3GPP RAN WG4 Meeting #101bis		R4-2200429

Electronic meeting, 17 – 26th January 2022

Agenda item:	5.1.1

Source:	Apple

Title:	Overview of the Region 1 countries implementing lower 6GHz unlicensed band

WI/SI:	NR_6GHz_unlic_EU

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Following the publication of the 6GHz regulatory decision for the EU/CEPT countries is November 2020, 3GPP RAN#90 meeting decided to resume the corresponding WI [2] that aims at adding 3GPP support for the 6GHz band in that regulatory region. As per 3GPP RAN plenary guidance made during the RAN#93 meeting, RAN WG4 was tasked to define a new band for the unlicensed operation in the lower 6GHz band. In addition to that, RAN#94 meeting has also clarified that a dedicated lower 6GHz unlicensed band should be defined as 5925-6425MHz.

Even though the corresponding WI for the lower 6GHz unlicensed band was triggered by the EU/CEPT regulations, there are other Region 1 countries that recently allowed license-exempt operation in that range. Thus, in this discussion paper we provide further updates on several Region 1 countries and their regulatory parameters.

# 2	Further updates to the lower unlicensed 6GHz band

In this section we provide a brief overview of the 6GHz regulatory decisions made in Morocco and UAE:

-	Morocco. On May 2021, the Moroccan National Telecommunications Regulatory Agency (ANRT) published a new decision on Short Range Devices (SRD) in 5925-6425MHz frequency range with the following technical parameters:

-	23dBm EIRP for indoor usage only, not allowed inside vehicles;

-	14dBm EIRP for indoor and outdoor usage for the portable devices, not allowed for unmanned flying devices (i.e. drones).

-	UAE. On December 2020, the telecommunications Regulatory Authority (TRA) of UAE has designated an additional 500 MHz radio spectrum, specifically 5925-6425MHz, for indoor use at an EIRP of 250 mW under class authorization.

As can be seen from the summary, Morocco defines the LPI operation with the same maximum EIRP of 23dBm as EU/CEPT. However, unlike the EU/CEPT region, there are no additional PSD restrictions. Thus, we can consider NS_01 for the case when the lower 6GHz band is used in Morocco. Quite similarly, UAE defines the LPI operation with the maximum EIRP of 24dBm and no PSD restrictions.

Proposal 1a:	Re-use 3GPP band n102 for Morocco and UAE.

Proposal 1b:	Existing NS_01 can be re-used for LPI in Morocco and UAE.

## 2.2	Summary of the required NS values

Table 2.2-1 below presents an updated summary of NS values for several Region 1 countries that have already decided to open the lower 6GHz band for the unlicensed operation.

Table 2.2-1: Summary of NS values required for NR-U PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | [NS_xx] (new) | TBD |
| Morocco | N/A | NS_01 | TBD |
| UAE | N/A | NS_01 | N/A |

Proposal 2:	Include the summary of the required NS values into TR 38.849.

# 3	Conclusions

In this contribution we have presented summary of several Region 1 countries that have opened the lower 6GHz band for the unlicensed operation.

Proposal 1a:	Re-use 3GPP band n102 for Morocco and UAE.

Proposal 1b:	Existing NS_01 can be re-used for LPI in Morocco and UAE.

Proposal 2:	Include the summary of the required NS values into TR 38.849.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP‐202592, "WID on introduction of lower 6GHz NR unlicensed operation for Europe", Nokia, Nokia Shanghai Bell

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.6a) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.6a) |  | 14 dBm |  |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |