3GPP RAN WG4 Meeting #101bis-e		R4-2200439

Electronic meeting, Jan. 17 – Jan. 25, 2021

Agenda item:	6.4.2.1.2

Source:	Apple Inc.

Title:	Views on FR2 inter-band DL CA CBM for different band groups

WI/SI:	NR_RF_FR2_req_enh2- Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During the RAN4#101e meeting it was discussed the definition of the ΔRIB,P,n (dB) and ΔRIB,S,n (dB) for inter-band DL CA CBM with different band groups considering multi-chain architecture.

In this contribution, we share our simulation results for the estimation of the SNR difference between the multi-chain architecture with CBM and multi-chain architecture with IBM. These results share a preliminary view on the additional delta relaxation required for the inter-band DL CA CBM for different band groups compared to inter-band DL CA IBM.

# 2	Discussion

## 2.1	Single-chain and multi-chain inter-band DL CA CBM

RAN4 agreed to define CBM requirements in such a manner that both single chain and multi-chain architectures are possible, these are summarized in [1]. The following Table distinguishes between three UE architectures; the figures in the first row represent the possible UE architectures and the figures in the second row show the beam reference signal allocation provided from the network configuration.

- Single Chain architecture with CBM

The UE architecture with single chain is supported with a common phase shifter and LO.

For the CBM, the network configures the RS on the PCell only, and the SCell beam performance is degraded due to the frequency separation. The beam squint effect contributes to the performance degradation.

- Multi-chain architecture with CBM

The architecture with multi-chain is supported with independent phase shifters and LOs. One of the main differences between multi-chain architecture with IBM and the multi-chain architecture with CBM comes from the network perspective. The network configuration for CBM provides the RS on the PCell only, and the SCell beam performance is degraded, since the beam of CC2 will choose its beam based on the measurement performed in CC1.

- Multi-chain architecture with IBM

The architecture with multi-chain is supported with independent phase shifters and LOs. The UE hardware capability with independent phase shifter settings supports independent beams with different AoA.

For IBM the network configures the RS on both PCell and SCell. Since in IBM the network provides RS on both CCs, the beam selection is more accurate compared to the CBM case.

RAN4 has agreed on the support of CBM for single-chain and multi-chain architecture. For the single chain architecture case, beam squint contributes to the performance degradation. For the 28 GHz frequency group we have shown in a previous contribution that an impact on EIS spherical coverage of 1.8 dB, whereas for the 39 GHz band group the impact is 2.6 dB [2]. The UE performance loss becomes more severe with increasing carrier frequency separation, the performance degradation as discussed in [3] can be more than 10 dB for two band groups (28 GHz + 39 GHz). Due to the large performance degradation, CBM between different band groups is not feasible with single-chain architecture and therefore, we propose that the requirement definition for inter-band DL CA between band group should only be based on multi-chain architecture.

Proposal 1:	For CBM between different band groups is not feasible with single-chain architecture. The requirement definition for inter-band DL CA between different band groups should only be based on multi-chain architecture.

## 2.2	EIS relaxation for different band groups

In the RAN4 previous meeting some companies have proposed to adopt the same relaxation factor for REFSENS and EIS spherical coverage between CBM and IBM UEs for different frequency group.  There are important factors that need to be considered, when comparing multi-chain CBM to multi-chain IBM.

One important key point is that the multi-chain architecture with CBM has independent phase shifters, such that the calculated coefficients for the beamforming weight from one of the phase shifters will have to apply to the other phase shifter. This will cause a calibration error when considering the independent chains with the CBM configuration that will contribute to the beamforming degradation. Similar issue does not occur for the multi-chain IBM where the beamforming weight can be estimated independently, and where the refence signal is schedule for both CCs. For the scenario with inter-band CA with CBM cross-array calibration of beams is essential.

Simulation conditions:

- The assumption of co-location of both CCs in the different band groups is needed from the network perspective for the feasibility of the scenario with inter-band CA CBM for different band groups.

- The channel model is static (gNB is in front of the UE, no rotation applied) and assumes CDL-D with LOS propagation condition.

- The simulation compares both inter-band CA with different band groups (28 GHz + 39 GHz) with IBM and CBM. For CBM the reference signal is in 28 GHz band only.

- Cross-array calibration of beam: Same number of beams for 28 GHz band and 39 GHz band. Since the reference signal is in 28 GHz band only, the simulation has selected the beam for 28 GHz band and mapped the same index beam for 39 GHz band.

- MCS 26 (64 QAM) with 120 kHz SCS at 100 MHz.

Main difference between IBM and CBM scenarios is the beam mapping accuracy. CBM needs more relaxation due to the “blind” beam selection. The channel model for this simulation is static with LOS between UE and gNB, in case of a scenario with additional rotation, it would make more difficult the correct beam selection and consequently, the SNR delta between IBM and CBM curves would be larger.

Figure 1: Relative Throughput versus SNR

Figure 1 provides results for the relative throughput versus SNR considering an off-beam peak and it compares the inter-band CA IBM to inter-band CA CBM case with different band groups (28 GHz + 39 GHz). The SNR delta between IBM and CBM for the relative throughput at 95% is around 5 dB.

This shows a preliminary result under the simulation conditions listed above. Further analysis is required for QPSK modulation at beam peak direction to evaluate, if the delta between IBM and CBM remains at 5 dB for the ΔRIB,P,n (dB) for REFSENS relaxation. In addition, further analysis is required for the derivation of the spherical coverage relaxation ΔRIB,S,n (dB).

Furthermore, the simulation results show the impact of the cross-array calibration of the beam for different band groups with only BMRS in one of the bands compared to cross-array calibration with BMRS in both bands. It has been shown that the misalignment between beam results in a performance degradation, which needs to be considered for the derivation of ΔRIB,P,n (dB) and ΔRIB,S,n (dB).

Observation 1:	The simulation results show the impact of the cross-array calibration of the beam for different band groups with only BMRS in one of the bands compared to cross-array calibration with BMRS in both bands.

Observation 2:	Preliminary simulation results show that the delta between inter-band DL CA CBM and inter-band DL IBM is 5 dB.

Proposal 2:	Additional relaxation ΔRIB,P,n (dB) and ΔRIB,S,n (dB) for inter-band DL CA CBM is required compared to inter-band DL CA IBM:

- ΔRIB,P,n (dB) for multi-chain CBM with different band groups =  ΔRIB,P,n_IBM (dB) + X1

- ΔRIB,S,n (dB) for multi-chain CBM with different band groups =  ΔRIB,S,n_IBM (dB) + X2

# 3	Conclusions

This contribution provides our preliminary view on the additional delta relaxation required for the inter-band DL CA CBM for different band groups compared to inter-band DL CA IBM. In summary, we have made the following proposals and observations:

Proposal 1:	For CBM between different band groups is not feasible with single-chain architecture. The requirement definition for inter-band DL CA between different band groups should only be based on multi-chain architecture.

Observation 1:	The simulation results show the impact of the cross-array calibration of the beam for different band groups with only BMRS in one of the bands compared to cross-array calibration with BMRS in both bands.

Observation 2:	Preliminary simulation results show that the delta between inter-band DL CA CBM and inter-band DL IBM is 5 dB.

Proposal 2:	Additional relaxation ΔRIB,P,n (dB) and ΔRIB,S,n (dB) for inter-band DL CA CBM is required compared to inter-band DL CA IBM:

- ΔRIB,P,n (dB) for multi-chain CBM with different band groups =  ΔRIB,P,n_IBM (dB) + X1

- ΔRIB,S,n (dB) for multi-chain CBM with different band groups =  ΔRIB,S,n_IBM (dB) + X2

# 4	References

R4-2114960, “Inter-band DL CA based on CBM for FR2”, Apple 3GPP RAN4 #100e, Aug 2021

R4-2006842, “View on radiative degradation mechanisms for larger frequency separation”, Apple, 3GPP RAN4 #95, June 2020

R4-2014912, “More on FR2 inter-band DL CA”, Apple, 3GPP RAN4 #97-e, November 2020