3GPP RAN WG4 Meeting #101bis-e		R4-2200442

Online, January 17th – 25th, 2022

Agenda item:	6.20.2.1.2

Source:	Apple

Title:	RedCap UE HD-FDD REFSENS requirements

WI/SI:	NR_redcap-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The RedCap UE REFSENS requirements have been discussed in last two RAN4 meetings [1-2]. For HD-FDD versus FD-FDD REFSENS requirement, although a 0.8dB tightening was agreed as baseline in general, whether additional tightening or relaxation is needed would still be checked band by band, as was concluded in the approved WF [2]. In this contribution, we share our view on why the 0.8dB tightening for HD-FDD versus FD-FDD REFSENS cannot be justified for all FDD bands and propose additional relaxation for bands with REFSENS close to that of TDD bands.

# Discussion

For the potential REFSENS tightening between HD-FDD and FD-FDD, there are two aspects to be considered, one is the insertion loss difference between the duplexer and single-band filter with T/R switch, the other is the Tx self-interference noise contribution. To derive the HD-FDD REFSENS requirements, we only need to evaluate the insertion loss difference between HD-FDD and FD-FDD front-end filter architectures and then apply the difference to 5MHz channel bandwidth REFSENS as the HD-FDD baseline REFSENS since Tx self-interference would not have impact on the 5MHz channel bandwidth as already analyzed in [3].

For the insertion loss difference between HD-FDD and FD-FDD front-end filter architectures, there had been studies during the development of LTE category 0 UE for MTC applications. In [4], the observation was that the insertion loss difference between SAW duplex filters and SAW RF filters for bands 3 and 8 is in the order of 0.5 dB.

Observation 1: The insertion loss difference between SAW duplex filters and SAW RF filters for bands 3 and 8 is in the order of 0.5 dB [4].

Though the REFSENS tightening for LTE category 0 UE under HD-FDD was eventually specified as 0.8 dB for most of the FDD bands [5], in our view, the 0.8dB number cannot be justified for all FDD bands, especially for those bands with REFSENS close to that of TDD bands (which are not subject to higher duplexer insertion loss), such as Band 1 and Band 18 whose 5MHz REFSENS is the same as most TDD bands at -100 dBm which has already been the tightest REFSENS specified among all FR1 bands. To avoid UE production yield impact from REFSENS tightening for those bands with REFSENS close to TDD bands, we propose the tightening as summarized in the following table.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Table 2-1 Proposed HD-FDD 5MHz REFSENS tightening for different NR bands

Proposal 1: The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in Table 2-1.

Since under half-duplex operation, the REFSENS is not affected by UL transmission, therefore, the REFSENS for channel BW wider than 5 MHz can simply be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS. On the other hand, for half-duplex operation, there is no need to impose UL RB number restriction. Therefore, full allocation is proposed as the UL configuration for HD-FDD REFSENS requirements.

Proposal 2: HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS.

Proposal 3: UL configuration for HD-FDD REFSENS requirements is specified with full allocation.

# 3	Conclusion

In this contribution, we share our view on why the 0.8dB tightening for HD-FDD versus FD-FDD REFSENS cannot be justified for all FDD bands and propose additional relaxation for bands with REFSENS close to that of TDD bands.

Observation 1: The insertion loss difference between SAW duplex filters and SAW RF filters for bands 3 and 8 is in the order of 0.5 dB [5].

Proposal 1: The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Proposal 2: HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS.

Proposal 3: UL configuration for HD-FDD REFSENS requirements is specified with full allocation.

Based on the proposals 1 – 3 above, the RedCap UE REFSENS requirements for HD-FDD with 2Rx can be concluded as shown in Table 3-1 and Table 3-2.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.2 | -95.2 |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n85 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |

Table 3-1 Proposed RedCap UE REFSENS requirements for HD-FDD with 2Rx

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n70 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n85 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |

Table 3-2 Proposed RedCap UE REFSENS UL configurations for HD-FDD

# 4	References

R4-2114995 “WF on RedCap REFSENS in FR1”, Sony, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021

R4-2120067 “WF on the RedCap RF”, Ericsson, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2112385 “RedCap UE REFSENS requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021

R4-146904 “RX filter insertion loss comparison between HD and FD”, Sony Mobile Communications Japan, Inc., 3GPP TSG-RAN WG4 Meeting #73, San Francisco, USA, November 17th – 21st, 2014

3GPP TS 36.101 V17.4.0 (2021-12)