3GPP RAN WG4 Meeting #101bis-e		R4-2200447

Online, January 17th – 25th, 2022

Agenda item:	5.36.2.2

Source:	Apple Inc.

Title:	HPUE A-MPR Proposal for NS_100

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During last RAN4 meeting, RAN4#101-e, a new way forward on Tx requirements was agreed [1]. The way forward asks interested companies to provide PC2 A-MPR proposals for certain network signalling. This contribution provides proposals for NS_100.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

###### 2.2 Simulation results

Simulations have been conducted for all channel bandwidth from 5MHz to 20MHz. A PC2 PA model for band n1 was used. Selected results can be found in section 3. DFT-s-OFDM and CP-OFDM for QPSK are shown. According to the simulations we propose to reuse the same A-MPR as for PC3.

Table 1: A-MPR for NS_100 (UTRA protection) (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2.0 |
|  | QPSK | ≤ 2.0 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4.0 |
|  | 16 QAM | ≤ 4.0 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

Proposal: For NS_100 re-use PC3 A-MPR values for PC2.

# 3  Simulations

| DFT-s-OFDM | CP-OFDM |
| --- | --- |

# 4	Conclusions

In this contribution we presented simulations results for NS_100 using band n1 PC2 amplifier model. Based on the results we propose the following:

Proposal: For NS_100 re-use PC3 A-MPR values for PC2.

# 4	References

- R4-2119946, “WF on NR FDD PC2 HPUE“, China Unicom, 3GPP TSG-RAN WG4 Meeting #101-e