3GPP RAN WG4 Meeting #101-bis-e		R4-2200448

Electronic Meeting, January 17 - 26, 2022

Agenda item:	6.2.3.1

Source:	Apple

Title:	Further views on manufacturing tolerances

Work Item:	NR_FR1_TRP_TRS-Core

Document for:	Discussion

# 1	Introduction

During the RAN4 #101 meeting, the following agreements related to manufacturing tolerances were captured [2]:

Issue 1-3-1: How to treat Manufacturing Tolerances Proposal 1: The following framework to take manufacturing tolerances into account for OTA requirements is proposed: 1) It is assumed that nominal UEs are used to collect radiated performance data in the performance phase of the work; 2) A candidate value X to achieve a passing rate of Y% is derived from the radiated performance data; 3) An offset Z is defined to relax X, such that the resulting OTA requirement reflects a passing rate of Y% in the full population of devices with 95% confidence Proposal 2: select a variety of phones during the measurement campaign to indirectly account for manufacturing tolerances. Agreements - RAN4 to further discuss the following proposed options: - Option 1: framework to take manufacturing tolerances into account for OTA requirements, such that: 1) It is assumed that nominal UEs are used to collect radiated performance data in the performance phase of the work; 2) A candidate value X to achieve a passing rate of Y% is derived from the radiated performance data; 3) An offset Z is defined to relax X, such that the resulting OTA requirement reflects a passing rate of Y% in the full population of devices with 95% confidence - Option 2: select a variety of phones during the measurement campaign to indirectly account for manufacturing tolerances - Option 3: apply Option 1 under some agreed UE selection criteria, then process the measured data according to Option 2 - Other options are not precluded

This contribution provides our further views on how to treat manufacturing tolerances.

# 2	Discussion

As we outlined in [3] and [4], the impact of manufacturing tolerances needs to be carefully considered during the TRP/TRS work in order to ensure that the OTA requirements defined by 3GPP are applicable to regulatory assignment of pass/fail limits to this requirement with the expectation that any device available in the market shall comply.

To further illustrate this concept, a number of random variables were generated to represent the average TRP and TRP over a population of devices and then compared against a pass/fail limit.  Table 1 below summarizes the relevant parameters.

Table 1: Parameters used in the manufacturing tolerance illustration

| Parameter | Value | Notes |
| --- | --- | --- |
| Number of UE types | 21 | Represents an example of 21 different UE models |
| Average TRP | ~uniform(11, 19) dBm | An example of the spread of different average TRP (nominal design target) across the 21 different UE models |
| TRP standard deviation | ~uniform(0.5, 2) dB | Represents the variability of actual TRP over the population of devices due to manufacturing tolerances |
| Example pass/fail limit | 13 dBm |  |
| Example manufacturing tolerance | 1 dB |  |

The anlaysis proceeds according to the following steps:

1.	UE types are determined to be "passing" or "failing" based on whether their average TRP exceeds the example pass/fail limit.  The TRP distributions of "failing" UE types are plotted in grey, and those of "passing" in color, in Figure 1 below.

2.	A passing rate is calculated from the average TRP data of the "passing" UE types; this is labeled "r(avg TRP)."

3.	The TRP values of all UEs in the population and within the group of "passing" UE types are also compared to the same exmaple pass/fail limit to calculate the passing rate of the "passing" population, and is labeled "r(pop TRP)."

4.	An example tolerance for manufacturing variations of 1 dB is used to relax the population pass/fail limit, and a passing rate of the "passing" population is calculated; this is labeled "r(pop TRP with tolerance)."

Figure 1: Illustration of the impact of manufacturing tolerance on OTA limits

Observation 1:	The passing rate computed from the means of each UE type's TRP distribution overestimates the passing rate computed from the actual population.

Observation 2:	By defining a manufacturing tolerance, which is used to relax the population pass/fail limit, a similar passing rate as expected from the average TRP statistics can be obtained.

To relate this simulation exercise to the WF from the last meeting, we observe that it is not possible for RAN4 to directly sample the entire population of all available devices in order to define OTA pass/fail limits.  Instead, RAN4 relies on collecting TRP data over a limited number of devices under test in the performance device pool.  Devices in this pool are typically chosen based on their nominal performance characteristics, and are likely to represent the mean of the distribution of TRP values over all devices of that particular type.  Thus, a pass/fail limit derived from such a data set corresponds to steps 1 and 2 in the example analysis process we described above.  In order to ensure the applicability of a pass/fail limit derived in such a manner, the passing rate of all devices deemed "passing" by the mean TRP metric needs to be similar to the passing rate of all such devices in the population.  Thus, a manufacturing tolerance is necessary.

Proposal 1:	RAN4 should agree to select Option 1 from the WF in [2], with the framework to take manufacturing tolerances into account for OTA requirements defined as:
1) It is assumed that nominal UEs are used to collect radiated performance data in the performance phase of the work;
2) A candidate value X to achieve a passing rate of Y% is derived from the radiated performance data;
3) An offset Z is defined to relax X, such that the resulting OTA requirement reflects a passing rate of Y% in the full population of devices withing a certain confidence interval

We note that RAN4 already has experience and past precedent how to determine the values X and Y, and these procedures are largely data-driven and well understood.  We anticipate the performance phase of the work item to address these in detail.

In our understanding, there are two ways to determine the offset Z:  based on analysis of data during the performance phase or based on a consensus value among interested companies.  The former option is purely data-driven, but requires the disclosure of average TRP and manufacturing tolerance per device type.  Although submitted results can be anonymized, not all companies could be comfortable with this approach, since it nonetheless discloses sensitive design data.  The latter option is less precise but enjoys the precedent of existing RAN4 handling of impairment and implementation margins, which is evident in the way RAN4 defines requirements on MPR, MSD, and demodulation.

Proposal 2:	The value Z can be determined by consensus among interested companies in RAN4 during the performance phase of the work.

# 3	Conclusions

This contribution provides our further views on the topic of manufacturing tolerances; the following observations and proposals have been made:

Observation 1:	The passing rate computed from the means of each UE type's TRP distribution overestimates the passing rate computed from the actual population.

Observation 2:	By defining a manufacturing tolerance, which is used to relax the population pass/fail limit, a similar passing rate as expected from the average TRP statistics can be obtained.

Proposal 1:	RAN4 should agree to select Option 1 from the WF in [2], with the framework to take manufacturing tolerances into account for OTA requirements defined as: 1) It is assumed that nominal UEs are used to collect radiated performance data in the performance phase of the work; 2) A candidate value X to achieve a passing rate of Y% is derived from the radiated performance data; 3) An offset Z is defined to relax X, such that the resulting OTA requirement reflects a passing rate of Y% in the full population of devices withing a certain confidence interval

Proposal 2:	The value Z can be determined by consensus among interested companies in RAN4 during the performance phase of the work.

# 4	References

RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

R4-2120689, "WF on FR1 TRP TRS," vivo, 3GPP RAN4 #101, November 2021

R4-2112364, "General considerations of OTA requirements," Apple, 3GPP RAN4 #100, August 2021

R4-2117975, "Views on frequency test points and manufacturing tolerances," Apple, 3GPP RAN4 #101, November 2021