3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                       R4-2200478

Electronic Meeting, January 17th - 25th, 2022

Title: 	TP to TR 38.863 on operating bands and channel bandwidth

Source: 	CATT

Agenda item:	6.13.1.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on operating bands and channel bandwidth for TR 38.863 [1].

- Reference

[1] TR 38.863, 0.1.0

[2] R4-2200162, on remaining open issue for NTN system parameters, CATT

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

7.2.1	Operating bands

The following bands in Table 7.2.1-1 are agreed as exemplary NTN bands in Rel-17. Regarding the band numbering for NTN bands, it is agreed to start from the largest band number in FR1 range for NTN bands which fully within FR1 frequency ranges to better differentiate NTN band and TN bands and ensure contiguous band number allocation for NTN bands, the number can be taken in a decreased order with first come, first service.

Table 7.2.1-1: NTN bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

7.2.2	Channel bandwidth, SCS and spectral utilization

[The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in TS 38.104[15] and TS 38.101-1[16] can be reused for NTN system].

The supported channel bandwidth per operating band should be defined based on NTN operator input. The details could be found in the Table 7.2.2-1.

Table 7.2.2-1:  Channel bandwidths and SCS per NTN operating band in FR1

| NTN satellite band # | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |
| n256 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| n255 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |

---------------------------------------------------End of Text proposal---------------------------------------------------------