3GPP TSG-RAN WG4 Meeting # 101b-e	R4-2200494

# Electronic Meeting, January 17-25, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Implementation aspects of increasing MOP for PC2 inter-band ULCA

Agenda Item:	5.35.2 Feasibility and impact study [Power_Limit_CA_DC]

Document for:	Approval

# Introduction

During R4#101-e meetings, additional input was provided in [2] on applicable cases and further agreements are captured in way forward [1]. In this contribution, we further examine the different PC2 inter-band cases with a focus on the cases where one band is based on a 2Tx PC2 or PC1.5 implementation.

# Discussion

## Implementation aspect of increased power for PC2 inter-band UL CA

In the scope of the PC2 intra-band feature, there are discussions about UEs where at least one band supports HPUE, can exceed the PC2 maximum power. This is essentially related to the same PA architecture issue discussed in [3]

- 2x1/2 (23+23) architecture cannot exceed 26dBm (+2dB with power class range)

- 1+1/2 (23+26) architecture can reach up to 27.75dBm (+2dB with power class range)

- 2x1 (26+26) architecture can reach up to 29dBm (+2dB with power class range). Note that this case, could be considered either as PC2 or PC1.5.

One band may implement one of the 2Tx architectures described in [3]. However, since 3Tx is not supported in R17, cases with 2UL, including 2TX transmissions in one band, are not allowed. If we believe that the maximum reachable power can be deducted from the individual power class declared for each band, the preceding constraint may create corner cases where HPUE is declared in one band and uses TxD or MIMO. Thus, The second PA shall not be used, which leads to a consideration of two cases:

- One band declares PC2 with TxD (2x1/2 23+23 architecture), and then the band can only reach 23dBm since it only uses one PA

- One band declares PC2 without TxD, and at least one 26dBm PA is available, then the band can reach 26dBm

Table 1 provides the power sums for the different cases that are valid for PC2 intra-band cases, with or without increased power. It should be noted that if transmissions are limited to 2Tx maximum, such as that agreed to in release 17, there is no case that exceeds 29dBm (+2dB for top of range).

Table 1: Per band power configurations for PC2 inter-band combinations with associated maximum reachable power

Proposal for increased power for PC2 inter-band UL combinations:

- For dual UL cases, the maximum power limit for PC2 intra-band CA MOP range uses the power sum of the declared per band power class with the following consideration on TxD signaling:

  - If a band declares PC2 and signals TxD, then the power sum shall assume PC3 for this band and only one PA is used

  - If a band declares PC2 without signaling TxD, then the power sum shall assume PC2 for this band and only one PA is used

  - If a band declares PC1.5, only 1Tx is used, and the power sum shall assume PC2 for this band

- For single UL cases, both PC2 and PC1.5 in one band can be considered for PC2 inter-band combinations with 2Tx

- Valid PC2 intra-band combinations shall only the cases where the 2UL or 1UL power sum of upper limit is less than or equal to 29dBm +2dB, after consideration of TxD and PC1.5 signaling.

# Conclusions

In this contribution, we examined all possible UL configurations for PC2 inter-band ULCA cases, and analyzed the cases where one of the bands uses a 2Tx PC2 or PC1.5 configuration. This enabled us to develop a proposal that specifies the feasibility of increased power.

Proposal for increased power for PC2 inter-band UL combinations:

- For dual UL cases, the maximum power limit for PC2 intra-band CA MOP range uses the power sum of the declared per band power class with the following consideration on TxD signaling:

  - If a band declares PC2 and signals TxD, then the power sum shall assume PC3 for this band and only one PA is used

  - If a band declares PC2 without signaling TxD, then the power sum shall assume PC2 for this band and only one PA is used

  - If a band declares PC1.5, only 1Tx is used, and the power sum shall assume PC2 for this band

- For single UL cases, both PC2 and PC1.5 in one band can be considered for PC2 inter-band combinations with 2Tx

- Valid PC2 intra-band combinations shall only the cases where the 2UL or 1UL power sum of upper limit is less than or equal to 29dBm +2dB, after consideration of TxD and PC1.5 signaling.

# References

- R4-2120064 WF on increasing MOP for CA/DC, Qualcomm Incorporated, RAN4#101e

- R4-2119441 Handling of maximum output power limit for PC2 inter-band combinations, Skyworks Solutions Inc., RAN4#101-e

- R4-2200499 Requirement and signaling aspect of features requiring two transmit paths, Skyworks Solutions Inc., RAN4#101b-e