3GPP TSG-RAN WG4 Meeting # 101b-e	R4-2200497

# Electronic Meeting, January 17-25, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Signaling for contiguous ULCA cases

Agenda Item:	6.3.2.2 HPUE for TDD intra-band contiguous UL CA

Document for:	Approval

# Introduction

During R4#100-e meetings, PC2 contiguous UL CA MPR was captured in a CR [3] and is implemented in 38.101-1. Although the MPR values are correct, some signaling aspects related to enabling implementations with 2LO with two PC2 PAs require additional consideration. In this contribution, we provide further clarification and guidance on related signaling aspects.

# Discussion

## Signalling for UL CA without MIMO

The contiguous UL CA MPR requirement already exists in 38.101-1 in:

- Table 6.2A.2.1-1 and Table 6.2A.2.1-2 for PC3 contiguous and non-contiguous allocations respectively

- Table 6.2A.2.1-1a and Table 6.2A.2.1-3 for PC2 contiguous and non-contiguous allocations respectively

While the MPR requirement is the same, some clarification is still needed for the applicability and signalling differences for the 1LO/1PA versus 2LO/2PA (2x1) cases. The specification states that the tables are applicable when the signaling is absent for dualPA-Architecture IE. Although this is true, the 2LO case may have to report uplinkTxDC-TwoCarrierReport-r16 and is only applicable to bandwidth class C.

As proposed in [1], it should be noted that PC2 delta MPR agreed to in R4-2119954 for PC2 contiguous ULCA+MIMO 23+23dBm case can apply for PC2 contiguous ULCA without MIMO (TxD is signaled). In [2], we propose to add this configuration to the PC2 ULCA 1Tx cases. To differentiate from the above cases, this implementation signals TxD

Proposal on MPR applicability for contiguous UL CA without MIMO for PC3 and PC2

- Applicable to Tables 6.2A.2.1-1, 6.2A.2.1-2, 6.2A.2.1-1a and 6.2A.2.1-3

- Remove limitation to absence of dualPA-Architecture IE reporting

- Add that uplinkTxDC-TwoCarrierReport-r16 is applicable to bandwidth class C

# Conclusions

In this contribution, we reassess the signaling aspects related to contiguous ULCA and make the following proposal.

Proposal on MPR applicability for contiguous UL CA without MIMO for PC3 and PC2

- Applicable to Tables 6.2A.2.1-1, 6.2A.2.1-2, 6.2A.2.1-1a and 6.2A.2.1-3

- Remove limitation to absence of dualPA-Architecture IE reporting

- Add that uplinkTxDC-TwoCarrierReport-r16 is applicable to bandwidth class C

# References

- R4-2200499 Requirement and signaling aspect of features requiring two transmit paths, Skyworks Solutions Inc., RAN4#101b-e

- R4-2200495 MPR for PC2 intra-band contiguous ULCA&MIMO 26+23 case, Skyworks Solutions Inc., RAN4#101b-e

- R4-2114956 CR on PC2 intra-band UL contiguous CA RF requirements	Huawei, HiSilicon, Skyworks, RAN4#100-e