3GPP TSG-RAN WG4 Meeting #101-bis-e                                                                   R4-2200505

E-meeting, 17th – 25th January, 2022

Agenda item:				5.26.1.2

Source:				CableLabs, Charter Communications

Title:			NR-U Triple Punctured Channel SEM for 100 MHz Bandwidth

Document for:			Approval

# Introduction

RAN4 #101-e meeting discussed adding the NR-U 100 MHz bandwidth and defining the corresponding spectrum emission mask (SEM) for the new triple punctured case introduced by the 100 MHz bandwidth [1] and [2]. The candidate SEMs are -28 dBr at 10 MHz from the punctured channel edges [3] and -25 dBr at 15 MHz from the edges [4] and [5].

# Proposed Triple Punctured SEM for 100 MHz Bandwidth

CableLabs conducted a comprehensive simulation analysis in [6] that compared three SEMs: (1) -31 dBr at 20 MHz from the edges; (2) -28 dBr at 10 MHz from the edges; and (3) -25 dBr at 10 MHz from the edges. SINR degradation of a NR-U network (victim) or Wi-Fi device using the punctured channels are estimated. The -25 dBr SEM of the aggressor network generates significant SINR and throughput degradation on the victim network. Note that the -25 dBr at 15 MHz SEM proposed in [4] is even worse than the SEM (3) we analyzed. The -28 dBr SEM is a good compromise for the aggressor RF design and coexistence. We would urge RAN4 to consider the -28 dBr SEM as described below:

Proposal 1: -28 dBr at 10 MHz from the edges. The SEM decreases from 0 dBr at the channel edges (-30 and 30 MHz) to -20 dBr at 1 MHz from the edges (-29 and 29 MHz), then decreases to -28 dBr at 10 MHz from the channel edges (-20 and 20 MHz), the SEM has a flat bottom of -28 dBr between -20 and 20 MHz (see Table 1 and Figures 1 and 2).

The proposed SEM is illustrated in Figure 1 and Table 1. Points A to F in Table 1 are defined in Figure 2.

Figure 1. Proposed SEM for the interior triple punctured case.

Table 1. NR-U Triple Punctured SEM for 100 MHz Bandwidth.

|  | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| ∆Frequency (MHz) | -30 | -29 | -20 | 20 | 29 | 30 |
| ∆Power (dBr) | 0 | -20 | -28 | -28 | -20 | 0 |

Figure 2. Definition of points A to F in SEM.

# Conclusion

We propose the SEM for NR-U 100 MHz bandwidth channel with triple punctures.

Proposal 1: -28 dBr at 10 MHz from the edges.

# References

[1] R4-2120033, “WF on adding 100 MHz channel BW in NR-U bands n46 and n96,” Qualcomm, RAN4 #101-e, Nov. 2021.

[2] R4-2119911, “Email discussion summary for [101-e][111] NR_bands_R17_BWs,” Ericsson, Nov. 2021.

[3] R4-2119301, “NR-U Punctured Channel SEM for 100 MHz Bandwidth,” CableLabs and Charter Communications, RAN4 #101-e, Nov. 2021.

[4] R4-2119197, “Further discussion on the introduction of 100MHz for NR-U,” ZTE, RAN4-#101-e, Nov. 2021.

[5] R4-2117879, “100 MHz channel bandwidth for NR-U,” Huawei and HiSilicon, RAN4-#101-e, Nov. 2021.

[6] R4-2112031, “NR-U Punctured Channel SEM for 100 MHz Bandwidth,” CableLabs and Charter Communications, RAN4 #100-e, Aug. 2021.