3GPP TR 38.868 V0.0.3 (2022-01)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Optimizations of pi/2 BPSK uplink power in NR;

User Equipment (UE) radio transmission and reception;

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<NR V2X (vehicle-to-everything), Sidelink, Public safety, Proximity based Services>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	6

4.1	Justification	6

4.2	Objective	7

5	 Evaluation of link level simulation	7

5.1 Qualcomm link level simulation results	7

5.2 Huawei link level simulation results	10

5.3 Nokia link level simulation results	11

6	 Evaluation of Pulse shaping filters	12

6.1 Pulse shaping filter simulation results	12

6.1.1 Huawei simulation results for pulse shaping filter	12

6.1.2 Nokia simulation results for pulse shaping filters	14

6.2  Net gain analysis of combined Tx and Rx impacts	15

6.2.1 Nokia simulation results for net gain analysis of combined Tx and Rx impacts	15

6.2.1.1 Net gain simulation results for TDL-C300ns	15

6.2.1.2 Net gain simulation results for TDL-A 30ns	17

6.2.1.3 Net gain simulation results for TDL-D 30ns	19

7	 SAR mitigation solutions	22

8	 Feasibility analysis of power enhancement	22

8.1 Qualcomm results for power enhancement	22

9	 Conclusion and recommendations	23

Annex A: Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document is a technical report for optimization of pi/2 BPSK NR uplink power in Rel-17. The purpose is to evaluate the feasibility of increasing the UE’s uplink power in TDD bands for pi/2 BPSK modulation assuming use of existing UE power classes as indicated per band or band combination. The objectives are applicable to FR1 TDD bands n34, n39, n40, n41, n77, n78 and n79. The justification and objectives of this work item are outlined in section  4.1 and section  4.2.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

DMRS	Demodulation Reference Signal

DSRC	Dedicated Short-Range Communications

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

FR2	Frequency Range 2

ITS	Intelligent Transportation System

LDPC	Low Density Parity Check

LTE	Long Term Evolution

LOS	Line-Of-Sight

MPR	Maximum Power Reduction

NF	Noise Figure

NLOS	Non-Line-Of-Sight

NR	New Radio

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

PRR	Package Reception Ratio

ProSe	Proximity-based Services

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SNR	Signal-to-Noise Ratio

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

# 4	Background

## 4.1	Justification

Coverage enhancement is a study item led by RAN1 for Rel-17. It has been identified that uplink channels are the bottleneck in many of the evaluated scenarios in terms of the coverage achieved. Given a fixed number of antennas and fixed MCS choices, an improvement in the uplink link budget is only possible through an increase in the UE’s UL power.

The current MPR tables may not fully exploit spectrum shaping of pi/2 BPSK waveforms. Meaningful reduction in MPR for certain waveforms could be achieved if UEs exclusively rely on ‘strong’ shaping. Increase in maximum achievable power may also be feasible relative to the MPR0 power level.

Using precedent in Rel-15 and Rel-16 for high power transmissions, duty cycle restrictions will help maintain average power levels at 23 dBm for compliance with SAR requirements.

In this study item, we propose to exploit strong spectrum shaping to realize UL power gains for pi2BPSK waveforms.

## 4.2	Objective

The objective of this study is to evaluate the feasibility of increasing the UE’s uplink power in TDD bands for pi/2 BPSK modulation assuming use of existing UE power classes as indicated per band or band combination. The objectives are applicable to FR1 TDD bands n34, n39, n40, n41, n77, n78 and n79.

- Identify achievable UE Tx power for pi/2 BPSK with the pulse shaping filter studied in this study item. [RAN4]

- Evaluate SAR-related duty-cycle restrictions and reporting mechanisms [RAN4]

- Identify shaping filter characteristics necessary to enable the new power capability while ensuring good and robust BS receiver performance. [RAN4]

  - Justify specification of a pulse shaping filter for this new identified UE power capability if it differs from filter impulse response specification in TS38.101-1 clause 6.4.2.4.1.E [RAN4]

  - Evaluate possible pulse shaping filter requirement applicable to the identified new UE power capability if achievable [RAN4]

  - Identify if necessary, changes that are needed to EVM equalizer flatness mask requirements to capture necessary filter shaping. Changes to the existing 14 dB p-p baseline to be assessed in relation to any potential gains in UL link performance while still ensuring robust BS receiver performance for all UEs in a cell. [RAN4]

Note: whether or not a new UE power class will be introduced for the identified achievable UE Tx power for pi/2 BPSK will be decided at the drafting stage of the following WI.

# 5	 Evaluation of link level simulation

The company contributions for link level simulations are given below:

#### 5.1 Qualcomm link level simulation results

The following table gives the link level parameters that were considered in the simulations:

| Parameter | Value |
| --- | --- |
| Pulse shaping filter | Filter configuration conforms to 38.101-1 |
| Channel model | TDL-A30ns |
| MCS Code rate | 0 |
| Waveform | DFTS OFDM with pi/2 BPSK filtered by same filter as for Rel-16 DMRS |
| # of DMRS symbols/slot | 2 |
| # of data symbols/slot | 12 |
| # of RBs | 2, 4, 8, 16 |
| TX/RX configuration | 1TX/4RX |
| BW | 100 MHz |
| UE speed | 3 km/h |
| SCS | 30 kHz |
| HARQ configuration | No retransmissions |

Table 5.1.1 – Link level simulation parameters

Below are the link simulation results for 2, 4, 8 and 16 RBs

Figure 5.1.1: Waveform with number of  RB=2

Figure 5.1.2: Waveform with number of RB=4

Figure 5.1.3: Waveform with number of RB=8

Figure 5.1.4: Waveform with number of RB=16

For a BLER=10-1   the following is observed from the simulation results:

| Waveform RB # | SNR @ BLER=10-1 No filter | SNR @ BLER=10-1 with filter | Delta (dB) |
| --- | --- | --- | --- |
| 2 | -6.9 | -6.5 | 0.4 |
| 4 | -8.7 | -8.5 | 0.2 |
| 8 | -9.9 | -9.7 | 0.2 |
| 16 | -10.8 | -10.7 | 0.2 |

Table 5.1.2: Filter degradation as a function of RB#

Based on this data the maximum degradation due to pulse shaping is seen to be 0.4 dB.

#### 5.2 Huawei link level simulation results

The Table 2 below is the recommended parameter list for link level simulation.

| Parameter | Value |
| --- | --- |
| Pulse shaping filter | Filter configuration conforms to 38.101-1 |
| Channel model | TDL-C300ns, TDL-A30, TDL-D30 |
| MCS | 0 |
| Code rate | 1/8, 1/4, 1/3, 1/2, 2/3 |
| Waveform | DFTS OFDM with pi/2 BPSK filtered by same filter as for Rel-16 DMRS |
| # of DMRS symbols/slot | 2 |
| # of data symbols/slot | 12 |
| # of RBs | [2, 4, 8, 16, 64] |
| TX/RX configuration | 1TX/4RX |
| BW | 100 MHz |
| UE speed | 3 km/h |
| SCS | 30 kHz |
| HARQ configuration | No retransmissions |

Table 5.2.1: Recommended Parameter list

With the simulation assumptions in Table 2, Figure below shows the BLER performance for NLoS channel with narrow (100MHz, 8 PRB) and large bandwidth allocation (100MHz, 64 PRB), respectively.

- Small RB allocation (8PRB)						(b) Large RB allocation (64PRB)

Figure 5.2.1: BLER performance of different shaping filters

#### 5.3 Nokia link level simulation results

Simulation parameters are shown in Table 3 and the link simulation results for 2-64 PRB bandwidths in the below figures.

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 100MHz |
| SCS | 30kHz |
| Channel model | TDL-C 300ns, TDL-A 30ns, TDL-D 30ns |
| UE speed | 3km/h |
| Channel estimation | Frequency domain |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| DMRS config | Low PAPR sequence type 2, 2 symbols |
| Waveform | DFT-S-OFDM |
| HARQ config | No retransmissions |
| Num PRBs | 2,4,8,16,64 |
| MCS | 0 |
| Channel | PUSCH, 14 OFDM symbols |
| Frequency hopping | No |
| BLER | 10% |
| Spectral shaping filter | 3-tap, FD implementation 1+D (not used for DMRS symbols) Triangular filters |

Table 5.3.1 Simulation assumptions

Figure 5.3.1 Required SNR for 10% BLER for different channel profiles

Based on company results the maximum degradation due to pulse shaping is seen to be up to 0.87dB for all 3 channel models.

# 6	 Evaluation of Pulse shaping filters

# 6.1 Pulse shaping filter simulation results

#### 6.1.1 Huawei simulation results for pulse shaping filter

Figure 6.1 illustrates the evaluated FDSS filter in term of the spectrum shape for the example of allocation of 20 PRBs.

Figure 6.1.1: Spectrum shape of the FDSS filters

To further elaborate our consideration and understanding of shaping filter, the comparison of different implementation is provided in figures below.

Figure 6.1.2: PAPR of different shaping filters (CBW 100MHz, 64RB)

Figure 6.1.3: PAPR of different shaping filters (CBW 10MHz, 8RB)

These results indicate that the PAPR for a given CCDF is lower for more aggressive pulse shaping filtering.

#### 6.1.2 Nokia simulation results for pulse shaping filters

The following figure shous the tested filters.

Figure 6.2.1 Tested filters

0.335 is the most aggressive 3-tap filter that meets the current spectral flatness requirements, less aggressive 3-tap filters are included too (0.2 and 0.28). Triang A is the most aggressive triangular window that meets the current spectral flatness requirements, and Triang B is a less aggressive triangular window, with similar frequency shape as the 0.2 3-tap filter.

A subset of the results for the evaluation of the pulse shaping filters from the transmitter point of view from [3] is shown in the following figures, where the filter that provides the highest output power is selected for each allocation configuration.

Figure 6.2.2 Shaping filter that provides the maximum output power for (left) 10 MHz channel BW, and (right) 50 MHz channel BW

Conclusions of the simulations (all available results are not included here due to vast amount of them but they can be found in [3],):

For all the tested channel bandwidth, and allocation configurations, there are filters that conform to 38.101-1 Rel-16 requirements that offer larger or same output power than the [1+D] filter when the DMRS are shaped.

For all the tested channel bandwidth, and allocation configurations, there are filters that conform to 38.101-1 Rel-16 requirements that offer larger or same output power than the [1+D] filter when the DMRS are not shaped.

#### 6.2  Net gain analysis of combined Tx and Rx impacts

##### 6.2.1 Nokia simulation results for net gain analysis of combined Tx and Rx impacts

In order to characterize the filters, it is needed to consider both the transmitter performance (i.e., the achievable output power) from our paper [3], and the link level performance obtained in [4]. With that information, it is possible to compute the net gain with respect to a reference filter as:

Net Gain dB=PoutFilter i-PoutRef. filter+SNR10%Ref. filter-SNR10%Filter i

Where PoutFilter i is the output power of the filter being compared against the reference filter, PoutRef. filter is the output power of the reference filter, SNR10%Ref. filter is the required SNR to achieve 10% BLER of the reference filter, and SNR10%Filter i is the required SNR to achieve 10% BLER of the filter being compared against the reference filter.

The following slides show the net gain of the filters with respect to the [1+D] filter with DMRS not shaped, for pi/2 BPSK considering the transmitter [3] and link level [4] performance for the following allocations and channels:

2, 4, 8, 16 and 64 PRB

TDL-C 300 ns, TDL-A 30 ns and TDL-D 30 ns

Filters tested:

[-0.2 1 -0.2], [-0.28 1 -0.28], [-0.335 1 -0.335], Triang A [6 14] and Triang B [6 8]

The tested filters assume that the filter is transparent to the gNB, and the DMRS are shaped. For the [1+D], matched filter is used in reception.

##### 6.2.1.1 Net gain simulation results for TDL-C300ns

For 2 and 4 PRB, [1+D] without DMRS shaped performs better than all the filters in center allocations (0.1-0.2 dB better than the 0.2 filter and 0-0.1 dB better than Triang B).

For 8 and 16 PRB, [1+D] without DMRS shaped is never the best for any allocation. Triang B is the best filter, providing gain for most of the possible allocations within the channel.

For 64 PRB, in central allocations, the performance is the same as Triang B. In edge band allocations, the 0.335 filter provides up to 0.9 dB gain.

For small allocation sizes, it must be noted that less aggressive filters perform better than aggressive filters in central band allocations.

There is not a single best solution for all the evaluated cases. Depending on the allocation configuration, different filters (i.e., more or less aggressive) perform differently.

Results are given in the below figures

Figure 6.2.1.1.1 2 PRB

Figure 6.2.1.1.2 4 PRB

Figure 6.2.1.1.3 8 PRB

Figure 6.2.1.1.4 16 PRB

Figure 6.2.1.1.5 64 PRB

Observation: For allocation sizes ≤ 16 PRB, less aggressive filters perform better than aggressive filters in central band allocations.

##### 6.2.1.2 Net gain simulation results for TDL-A 30ns

For 2 and 4 PRB, [1+D] without DMRS shaped performs better than all the filters in all allocations (0.1-0.2 dB better than the Triang B filter).

For 8 and 16 PRB, [1+D] without DMRS shaped is never the best for any allocation position.

For 64 PRB, in central allocations, the performance is the same as Triang B. In edge band allocations, the 0.335 filter provides up to 0.8-0.9 dB gain.

For small allocation sizes, it must be noted that less aggressive filters perform better than aggressive filters in central band allocations.

There is not a single best solution for all the evaluated cases. Depending on the allocation configuration, different filters (i.e., more or less aggressive) perform differently.

Results are given in the below figures

Figure 6.2.2.1 2 PRB

Figure 6.2.2.2 4 PRB

Figure 6.2.2.3 8 PRB

Figure 6.2.2.4 16 PRB

Figure 6.2.2.5 64 PRB

##### 6.2.1.3 Net gain simulation results for TDL-D 30ns

For 2 and 4 PRB, [1+D] without DMRS shaped performs better than all the filters in all allocations (0.1-0.2 dB better than the Triang B filter)

For 8 and 16 PRB, [1+D] without DMRS shaped is never the best for any allocation. In central band allocations, Triang B provides gain. 0.335 filter provides up to 1 dB gain in edge band allocations.

For 64 PRB, in central band allocations Triang B provides gain. In edge allocations, Triang A, 0.28 and 0.335 filters provide up to 0.8 dB gain.

For small allocation sizes, it must be noted that less aggressive filters perform better than aggressive filters in central band allocations.

There is not a single best solution for all the evaluated cases. Depending on the allocation configuration, different filters (i.e., more or less aggressive) perform differently.

Results are given in the below figures

Figure 6.2.3.1 2 PRB

Figure 6.2.3.2 4 PRB

Figure 6.2.3.3 8 PRB

Figure 6.2.3.4 16 PRB

Figure 6.2.3.5 64 PRB

# 7	 SAR mitigation solutions

# 8	 Feasibility analysis of power enhancement

#### 8.1 Qualcomm results for power enhancement

Investigations were done at f=3450MHz using the parameters given in the following table.

| Parameter | Value |
| --- | --- |
| Pulse shaping filter | [0.28 0.91 0.28] |
| Waveform | DFTS OFDM with pi/2 BPSK with Rel-16 DMRS |
| Start RB | [10 30 50 70 90] |
| LCRB | [2 4 8 16 64] |
| BW | 40 MHz |
| SCS | 30 kHz |

Table 8.1.1: Measurement parameters

Each Pi/2 BPSK waveform was filtered using a 3-tap filter given in table 1. This filter was selected to give waveforms with PAPRs around 2dB which is was thought to be a good compromise between achieving higher output power and limiting excessive filtering of the signal. For each start RB/LCRB combination the PA output was analyzed for IBE, EVM, ACLR, SEM and output power. The table below gives the measured results based on one PA sample. The increase in output power for various pi/2 BPSK waveforms above the PC2 MPR0 power level is given along with the SEM and IBE margins. The pi/2 BPSK output power is constrained by practical implementation considerations.

| Waveform | Output Power above PC2 MPR0 | SEM margin | IBE margin |
| --- | --- | --- | --- |
|  | (dBm) | (dB) | (dB) |
| 2RB10 | 1.4 | 10.7 | 16.4 |
| 2RB30 | 1.2 | 12.3 | 15.5 |
| 2RB50 | 1.3 | 26.6 | 22.7 |
| 2RB70 | 1.1 | 22.3 | 15.6 |
| 2RB90 | 1.0 | 11.9 | 16.3 |
| 4RB10 | 1.2 | 12.3 | 15.8 |
| 4RB40 | 1.1 | 16.7 | 15.9 |
| 4RB50 | 1.0 | 28.4 | 22.2 |
| 4RB70 | 1.0 | 17.4 | 15.7 |
| 4RB90 | 1.0 | 13.9 | 15.6 |
| 8RB10 | 1.1 | 14.0 | 15.7 |
| 8RB30 | 1.0 | 18.0 | 15.4 |
| 8RB50 | 1.0 | 28.3 | 23.8 |
| 8RB70 | 0.9 | 21.1 | 14.8 |
| 16Rb10 | 1.1 | 16.3 | 16.6 |
| 16RB30 | 1.2 | 26.9 | 22.4 |
| 16RB50 | 1.0 | 28.4 | 22.9 |
| 16RB70 | 1.1 | 19.2 | 16.4 |
| 64RB10 | 1.0 | 8.4 | 20.1 |
| 64RB30 | 1.3 | 10.9 | 20.9 |

Table 8.1.2: Measured PA results

The IBE, EVM, ACLR and SEM were all seen to pass with sufficient margin for these waveforms.

Observation 1: Measurements on a PC2 PA revealed that Pi/2 BPSK waveforms can deliver approximately 1 dB of extra power compared to PC2 MPR0 power

# 9	 Conclusion and recommendations

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-05 | RAN4 #99e | R4-2109373 |  |  |  | TR38.xxx v0.0.1: TR skeleton for optimizations of pi/2 BPSK uplink power in NR | 0.0.1 |
| 2021 | RAN4#100e | R4-2109377 |  |  |  | General information for TR skeleton for optimizations of pi.2 BPSK uplink power in NR | 0.0.2 |
| 2022 | RAN4#101-bis-e |  |  |  |  | Added contents of TP R4-2117644 | 0.0.3 |