3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200508

17 –25 January 2022

Source:	Qualcomm Incorporated

Title:	Calculation of MSD for V2X_1A-n47A and V2X_n1A-47A and accompanying TP

Agenda item:	5.25.1

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN#94e new inter-band V2X combinations V2X_1A-n47A and V2X_n1A-47A were introduced [1]. In this paper we present the MSDs required for n47 for these V2X band combinations.

# 2. 	Discussion

In [1] the inter-band combinations V2X_1A-n47A and V2X_n1A-47A were introduced. For these combinations the 3rd harmonic of band 1 can fall into n47 causing degradation in sensitivity. The 3rd harmonic of band 1 can couple into the n47 receiver through the PCB or can be conducted from the band 1 antenna to the n47 antenna. The below table details the MSD calculation for these V2X combinations.

Table 1: TX to RX H3 budget

|  | B1/n1 PA-->CV2x n47 Rx |
| --- | --- |
| Bandwidth (MHz) | 10 |
| SCS (kHz) | 15 |
| 3GPP Sensitivity(dBm) | -92.5 |
| PA out B1/n1 H3 (dBm) | -7 |
| B1 Tx Filter Rejection+ FE filtering | -71 |
| PCB coupling to B1/n1 PRX/DRX LNA | -70 |
| Tx-Rx Antenna Isolation | -10 |
| n47 front-end loss (dB) | 4 |
| PRX/DRX B1/n1 H3 conducted into n47 RX (dBm) | -88 |
| PRX/DRX B1/n1 H3 coupled into n47 RX (dBm) | -73 |
| PRX/DRX total H3 (dBm) | -72.9 |
| PRX/DRX noise (dBm) | -89.0 |
| Dual sensitivity (dBm) | -73.3 |
| Desensitization (dB) | 19.2 |

The MSD values for the other SCS and BW combinations were calculated in a similar manner and based on these results the following MSD recommendations are made for each V2X combination and BW.

Table 2: n47 MSD for V2X_1A-n47A

| BW(MHz) | 10 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- |
| MSD (dB) | 19.2 | 15.9 | 14.2 | 13.0 |

Table 3: n47 MSD for V2X_n1A-n47A

| BW(MHz) | 10 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- |
| MSD (dB) | 20.1 | 16.3 | 14.7 | 13.2 |

Proposal 1: Adopt the MSD’s given in Table2 and Table3 for n47 for the following V2X combinations: V2X_1A-n47A and V2X_n1A-47A

Proposal 2: Support of all V2X band and SCS combinations is optional

Proposal 3: Support of all V2X band combinations that are made release independent to an earlier release is optional in the earlier release

# Conclusion

We present the analysis for MSD for inter-band combinations V2X_1A-47A and V2X_n1A-n47A and make the following proposals.

Proposal 1: Adopt the MSD’s given in Table2 and Table3 for n47 for the following V2X combinations: V2X_1A-n47A and V2X_n1A-47A

Table 2: n47 MSD for V2X_1A-n47A

| BW(MHz) | 10 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- |
| MSD (dB) | 19.2 | 15.9 | 14.2 | 13.0 |

Table 3: n47 MSD for V2X_n1A-n47A

| BW(MHz) | 10 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- |
| MSD (dB) | 20.1 | 16.3 | 14.7 | 13.2 |

Also, proposals on the optionality of supporting inter-band V2X combinations are outlined.

Proposal 2: Support of all V2X band and SCS combinations is optional

Proposal 3: Support of all V2X band combinations that are made release independent to an earlier release is optional in the earlier release

# Annex - TP for TR 37.875 on MSD for n47 for V2X_1A-n47A and V2X_n1A_47A

## --Start of changes1

## MSD test configurations are specified below for V2X_1A-n47A. The MSD values given in tables 6.1.5.4-1

#### 6.1.5.4	MSD

Table 6.1.5.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n47 | 15 | 50 | 10 | 50 | 10 | 19.2 |
| 1 | n47 | 15 | 50 | 10 | 105 | 20 | 15.9 |
| 1 | n47 | 15 | 50 | 10 | 160 | 30 | 14.2 |
| 1 | n47 | 15 | 50 | 10 | 216 | 40 | 13.0 |

## --End of changes1

## --Start of changes2

#### 6.3.4.4	MSD

## MSD test configurations are specified below for V2X_1A-n47A. The MSD values given in tables 6.3.5.4-1

Table 6.3.5.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | n47 | 60 | 30 | 40 | 50 | 10 | 20.1 |
| n1 | n47 | 60 | 30 | 40 | 105 | 20 | 16.3 |
| n1 | n47 | 60 | 30 | 40 | 160 | 30 | 14.7 |
| n1 | n47 | 60 | 30 | 40 | 216 | 40 | 13.2 |

--End of changes2

# References

[1] R4-2105367, CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A, RAN4#98-bis, Apr. 12-20 2021

[2] R4-2105368, CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A, RAN4#98-bis, Apr. 12-20 2021

[3] R4-2107815, Discussion and TP for TR 38.375 on SMD for V2X_n79A-n47A and V2X_n79A_47A, RAN#99-e, May 19-28 2021

[4] R4-2113420, Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A-47A, RAN4#100-e, August 16 -27 , 2021

[5] R4-2112285, Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP, RAN4#100-e, August 16 -27 , 2021