3GPP TSG-RAN4 Meeting #101b-e	R4-2200536

# Electronic meeting, Jan. 17 – 25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on TCI chain update for Rel-17 FeMIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-1-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - legacy TCI chain only works for QCL info in Rel-15/Rel-16. In Rel-17, both UL and DL TCI are designed in unified TCI framework. Therefore, it need to update the TCI chain definition in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - DL TCI chain and UL TCI chain are newly added for Rel-17 unified TCI state. For DL TCI chain, SSB associated with a different PCID is added as a source RS. For UL TCI state chain, SSB, SRS or CSI-RS will be considered as source RS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The TCI chain will not work for Rel-17 unified TCI state framwork. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - 3.6.7, 3.6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==========================Start of first change =============================

### 3.6.7	Applicability of QCL

For the requirements specified in this version of the specification for TCI state switching, a reference signal is considered to be QCLed to another reference signal if it is in the same TCI chain as the other reference signal, provided that the number of Reference Signals in the chain is no more than 4. It is assumed there is single QCL type per TCI chain.

A TCI chain consists of an SSB, and one or more CSI-RS resources, and the TCI state of each Reference Signal includes another Reference Signal in the same TCI chain.

DMRS of PDCCH or PDSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state.

For the requirements specified in this version of the specification for separate DL TCI state or joint TCI state switching, a reference signal is considered to be QCLed to another reference signal if it is in the same DL TCI chain as the other reference signal, provided that the number of Reference Signals in the chain is no more than 4. It is assumed there is single QCL type per DL TCI chain.

A DL TCI chain consists of an SSB, and one or more CSI-RS resources, or SSB associated with a different PCID, and the TCI state of each Reference Signal includes another Reference Signal in the same DL TCI chain.

DMRS of PDCCH or PDSCH is QCLed with the reference signal in its active TCI state and any other reference signal that is QCLed, based on above criteria, with the reference signal in the active TCI state.

==========================End of first change =============================

==========================Start of 2nd change =============================

### 3.6.x Applicability of UL spatial relation

For the requirements specified in this version of the specification for separate UL TCI state switching, a reference signal is considered to provide reference for determining UL spatial relation for another reference signal if it is in the same UL TCI chain as the other reference signal, provided that the number of Reference Signals in the chain is no more than 4.

A UL TCI chain consists of an SSB, and one or more CSI-RS resources, or SRS, and the TCI state of each Reference Signal includes another Reference Signal in the same UL TCI chain.

==========================End of 2nd change =============================