3GPP TSG RAN4 Meeting #101bis-e	R4-2200544

Electronic Meeting, 17th – 25th January, 2022

Title: 	Discussion on Rel-17 RAN4 UE feature list

Source: 	Intel Corporation

Agenda item:	4

Document for:	Discussion

# Introduction

This meeting marks the starting point of RAN4 Rel-17 UE feature list. We are supposed to provide our initial version of feature list to other WGs after RAN4 #101bis-e and in this paper we share some of our views on specific topics.

# Discussion

## NR and MR-DC measurement gap enhancements [NR_MG_enh]

The following UE features are proposed for the Rel-17 NR and MR-DC measurement gap enhancements WI. The proposed features across the three sub-topics of the work item: pre-configured gaps, concurrent measurement gaps and NCSG. The corresponding discussions can be found in our other three parallel papers respectively in [1][2][3].

Table 1. NR_MG_enh feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1-1 | Pre-configured measurement gap with network-controlled activation and deactivation mechanism | 1) support of pre-configured measurement gap with network-controlled mechanism for activation and deactivation |  | yes | no | UE does not support pre-configured measurement gap with Network-controlled mechanism | per UE | No | No |  |  | Optional with capability signalling |
| X-1-2 | Pre-configured measurement gap with UE autonomous activation and deactivation mechanism | 1) support of pre-configured measurement gap with UE autonomous mechanism for activation and deactivation |  | yes | no | UE does not support pre-configured measurement gap with UE autonomous mechanism | per UE | No | No |  |  | Optional with capability signalling |
| X-2 | Concurrent measurement gaps | 1) Support of configuration of more than 1 per-UE/per-FR measurement gap configurations |  | yes | no | UE cannot be configured with concurrent gaps | per UE | No | No |  | The maximum supported number of concurrent gaps is FFS and pending RAN4 discussion. | Optional with capability signalling |
| X-3 | Network controlled small gap (NCSG) | Support of NCSG |  | yes | no | UE cannot be configured with NCSG | per-UE | No | No |  |  | Optional with capability signalling |

## Positioning enhancements for NR [NR_pos_enh]

The following RAN4 UE feature groups are proposed for NR positioning enhancements in Rel-17.

Table 2 NR_pos_enh feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1 | FFS: per-FR MG for PRS measurement | Capability of supporting per-FR MG for PRS measurement | Rel-15 per-FR gap (independentGapConfig) | yes | no |  | Per UE | No | No | N/A |  | Optional with capability signalling |

## Extending current NR operation to 71GHz [NR_ext_to_71GHz]

The following UE features are proposed for the Rel-17 Extending current NR operation to 71GHz WI. More details can be found in our companion paper [4].

Table 3. NR_ext_to_71GHz feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | FR2-2 channel bandwidths for each SCS in each band for DL and UL for a single CC | Support of FR2-2 channel bandwidths 1) 120 kHz SCS: {100, 400} MHz CBW 2) 480 kHz SCS: {400, 800, 1600} MHz CBW 3) 960 kHz SCS: {400, 800, 1600, 2000} MHz CBW | FFS | Yes | No | UE cannot support some UE channel bandwidths | Per band | N/A | Applicable to FR2-2 only | N/A | UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed. | NA |
| x-2 | 64QAM for PUSCH for FR2-2 | 1) Support of 64QAM modulation for FR2-2 PUSCH | FFS | Yes | No | UE cannot support PUSCH 64QAM transmission | Per band | N/A | Applicable to FR2-2 only | N/A |  | Optional with capability signalling |
| x-3 | [Improved ON/ON transient period] | 1) Support of improved ON/ON transient period of X < 5us (X is FFS) | FFS | Yes | No | UE does not support improved ON/ON transient period and support 5us transient period | Per UE | N/A | Applicable to FR2-2 only | N/A | Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value | Optional with capability signalling |

## Further RRM enhancements for NR and DC [NR_RRM_enh2]

Following the approach that we had in Rel-16, we propose the below feature groups for FeRRM in Rel-17.

Table 4. NR_RRM_enh2 feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | SRS antenna port switching RRM requirements | 1) Support of SRS antenna port switching RRM requirements | R15 RAN1 feature group 2-55 SRS Tx switch | Yes | No | The UE does not meet the specified RRM requirements for SRS antenna port switching in Rel-17 | Per UE | No | No | N/A |  | Mandatory with capability signalling if the prerequisite group is supported by the UE |
| x-2 | Handover with PSCell RRM requirements | 1) Support of Handover with PSCell RRM requirements |  | Yes | No | The UE does not meet the specified RRM requirements for handover with PSCell in Rel-17 | Per UE | No | No | N/A |  | Optional with capability signalling |
| x-3 | PUCCH SCell activation RRM requirements | 1) Support of PUCCH SCell activation RRM requirements |  | Yes | No | The UE does not meet the specified RRM requirements for PUCCH SCell activation in Rel-17 | Per UE | No | No | N/A |  | Optional with capability signalling |

## NR HST enhancements in FR1 [NR_HST_FR1_enh]

In the last meeting, RAN4 approved an LS to RAN2 to inform about the agreements on UE capabilities related to HST enhancements in FR1 in Rel-17. The LS was sent in R4-2120286 [5]. According to the RAN4 agreements, we list the corresponding features in the below table as part of the RAN4 Rel-17 UE feature list. More details about feature group “Support of enhanced Demodulation requirements for CA in HST SFN FR1” can be found in our companion paper [6].

Table 5. NR_HST_FR1_enh feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Support of enhanced RRM requirements for CA in HST FR1 | 1) Support of enhanced RRM requirements for CA in HST FR1 | Rel-16 RAN4 feature 10-1, 10-4 or 10-5 | Yes | No | UE is not able to meet the enhanced RRM requirements for CA in HST FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |
| x-2 | Support of enhanced inter-frequency measurements in CONNECTED mode HST FR1 | 1) Support of enhanced inter-frequency measurements in HST FR1 in connected mode | Rel-16 RAN4 feature 10-1, 10-4 or 10-5 | Yes | No | UE is not able to meet the enhanced inter-frequency measurement requirements in CONNECTED mode HST FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |
| x-2a | Support of enhanced inter-frequency measurements in IDLE mode HST FR1 | 1) Support of enhanced inter-frequency measurements in HST FR1 in IDLE mode | Rel-16 RAN4 feature 10-1, 10-4 or 10-5 | No | No | UE is not able to meet the enhanced inter-frequency measurement requirements in IDLE mode HST FR1 | Per-UE | No | FR1 only | N/A |  | Optional without capability signalling |
| x-3 | Support of enhanced Demodulation requirements for CA in HST SFN FR1 | 1) Support of demodulation processing for HST SFN CA scenario in FR1 | Rel-16 RAN4 feature 10-2 | Yes | No | UE is not able to apply demodulation processing for HST SFN CA scenario in FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |

## NR HST enhancements in FR2 [NR_HST_FR2]

The following UE features are proposed for the Rel-17 NR FR2 HST WI. More details can be found in our companion paper [7].

Table 6. NR_HST_FR2 feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Support of FR2 HST operation | 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST 3) Support of demodulation processing for FR2 HST | N/A | Yes | No | UE is not able to meet the enhanced requirements in HST FR2 | Per Band | No | Applicable to FR2 only | N/A | FR2 UE power class PC6 signalling is used to indicate support of feature group | Optional with capability signalling |

## Further enhancement on NR demodulation performance [NR_demod_enh2-Perf]

In our companion paper [8], we provide the detailed view on feature list for Rel-17 Further enhancement on NR demodulation performance WI. In Table 7, we summarize our proposal for this WI. We would like to note that features related to MMSE-IRC requirements are not captured in this table under assumption that requirements will be defined in the release independent manner, but it is the subject of further RAN4 discussion. Also, at current stage, we list two CRS-IM features for different scenarios (DSS and non-DSS), but these features can be merged later, based on outcome of RAN4 discussion on network assistance signalling and UE blind detection.

Table 7. NR_demod_enh2-Perf feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | CRS-IM receiver for DSS scenario | 1) Support of CRS-IM receiver in scenarios with serving and interference cells are in DSS mode and with overlapping spectrum |  | Yes | No | UE can’t apply mitigation of inter-cell LTE CRS interference | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |
| x-2 | CRS-IM receiver for non-DSS scenario | 1) Support of CRS-IM receiver in scenarios with serving cell is NR cell and interference cell(s) is(are) LTE cell(s) and with overlapping spectrum |  | Yes | No | UE can’t apply mitigation of inter-cell LTE CRS interference | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |

# Conclusions

This meeting marks the starting point of RAN4 Rel-17 UE feature list. We are supposed to provide our initial version of feature list to other WG-s after this meeting so in this paper we share some of our views on specific topics.

# References

- R4-2200537, Discussion on pre-configured measurement gap, Intel Corporation

- R4-2200538, Discussion on multiple and independent concurrent measurement gaps in NR, Intel Corporation

- R4-2200539, Discussion on NCSG in NR, Intel Corporation

- R4-2201923, CR work split and UE feature list for NR ext. to 71GHz, Intel Corporation, RAN4 #101-bis-e, January 2022.

- R4-2120286, LS on signalling for RRM enhancements for Rel-17 NR FR1 HST, CMCC

- R4-2200521, Discussion on HST FR1 CA PDSCH performance requirements, Intel Corporation, RAN4 #101-bis-e, January 2022.

- R4-2201877, Views on FR2 HST PDSCH performance requirements, Intel Corporation, RAN4 #101-bis-e, January 2022.

- R4-2201603, Discussion on UE feature list and capability signalling for NR Demodulation Enhancements WI, Intel Corporation, RAN4 #101-bis-e, January 2022.