3GPP TSG-RAN WG4 Meeting #101bis-e 	R4-2200587

Electronic Meeting, 17st - 25th Jan, 2022

Source:	ZTE Corporation

Title:	Views on multiple concurrent and independent MGs

Agenda Item:	6.11.2.2

Document for:	Approval

- Introduction

In the last meeting, the following agreements on the concurrent and independent MGs were achieved [1]:

General: Whether and when to start the discussion of joint requirements among different objectives in this WI - RAN4 focuses on the individual requirements in this meeting - Whether to ask RAN2 to allow simultaneous configurations of pre-configured gap, concurrent gap and NCSG in RRC signalling design for forward compatibility - Ask RAN2 to take joint operations (among Pre-MG, NCSG and concurrent gaps) into account in RRC - signalling design for forward compatibility. - Detail wording to be discussed in the LS draft. - Spec impact - Take the analysis of spec impact on TS38.133 in R4-2117602 as a starting point and FFS for - TS36.133. - Details and any further adjustments are subject to further technical discussions and will be left to the - individual email thread discussion Applicability and configurations: - Whether PRS measurement for positioning can only be exclusively associated with only one of the instance of multiple gaps at least for R17 - PRS measurement for positioning including all positioning frequency layers is associated with only one - of the concurrent gaps - It is up to network whether to associate a concurrent gap only to PRS measurement - Whether to support 2G/3G measurement with concurrent gap - RAN4 to focus on NR and EUTRAN measurement requirements with concurrent gaps before - considering 2G/3G. - It is up to RAN2 to decide whether to support gap association to 2G/3G from signalling - perspective - Note: The understanding of “2G/3G is not supported with concurrent gap” is that UE expects - network to configure only one MG if any 2G/3G measurements are configured, regardless whether NR or EUTRAN measurements are configured. UE capability related issues: - Support of indices 11 and 12 - Support the 2 additional combinations (Index 11 and Index 12) Index; # of simultaneous MG. Per-FR1; Per-FR2; Per-UE. 11; 2; 0; 0. 12; 0; 2; 0. - Clarification on gap pattern #24 and #25 - It is RAN4’s understanding that GP#24 and #25 are only used as per-UE gap. No change of this - conclusion is expected in this WI. - Clarification on configuration limitations or restrictions regarding the GPs supported by the UE - Besides the potential outcome of UE capability, applicability of existing MG patterns, and overhead - cap discussions, no additional configuration limitations or restrictions regarding which of the GPs supported by the UE that can be configured as concurrent gap. Overlapping: - Company preference on introducing FO, FPO, PFO, PPO scenarios - Postpone this decision to next meeting Measurement requirements: - Whether to additionally consider the limitation that each reference signal can only be measured in one MG pattern - Measurements for different frequency layers but with the same reference signal can be associated to - different concurrent MGs - CSSF calculation - CSSF should be calculated separately for each gap and only the frequency layers sharing this gap - should be counted in - Note: how to deal with overlapping concurrent MGs is up to Sub-topic 2-3 Impact to other L1 measurements: - How to capture the impact on L1 measurements due to concurrent gap - Take Rel-15 principle as a starting point, e.g., - L1 measurements are only expected to be performed outside gap. - In FR1, L3 and L1 measurements can be performed at the same time. - In FR2, L3 and L1 measurements are not expected to be perform at the same time. - FFS how to specify the impact of concurrent gap on L1 measurement period in a generic manner. - Others: - Whether to introduce a transition period for gap configuration/deconfiguration - Do not introduce a transition period for gap configuration/de-configuration

Multiple issues around concurrent MGs have been widely discussed and significant progress was made during 101 meeting. In this contribution, we provide some discussions on the following remaining open issues.

- UE capability related issues

- Whether to allow simultaneous configuring per-UE gap and per-FR gap for per-FR gap capable UEs

- Max number of concurrent gap across all FRs for per-FR gap capable UEs

- Overlapping issues

- Proximity condition for overlapping

- Rule for colliding gap occasions, if one of FO, FPO, PFO, PPO cases is introduced

- Overhead

- Whether to define an overhead cap for concurrent gaps

- Measurement requirements

- Further clarification on limitation that each reference signal can only be measured in one MG

- pattern

- Measurement delay

- Discussion

2.1 UE capability related issues

2.1.1 Whether to allow simultaneous configuring per-UE gap and per-FR gap for per-FR gap capable UEs

For the issue of Whether to allow simultaneous configuring per-UE gap and per-FR gap for per-FR gap capable UEs, it is still undetermined until 101 meeting :

- Open issue - FFS the use case of simultaneous configuring per-UE gap and per-FR gap. Consider the - identified use cases to make decision in RAN4#101b-e meeting.

PRS measurement is widely regarded as a use case of simultaneous configuring per-UE gap and per-FR gap until now. If such simultaneous configuring supported for the UE being capable of per-FR gap and concurrent gaps, then we can not see clear reason to give any limitation for the simultaneous configuring. In other words, NW can configure simultaneous per-UE gap and per-FR gap depending on the measurement demand, not only limit to the PRS measurement.

Proposal 1: Since the demand of PRS measurement, UE should support simultaneous configuring per-UE gap and per-FR gap for the UE being capable of per-FR gap and concurrent gaps.

Proposal 2: Once the simultaneous configuring supported, no need to introduce additional limitation, NW can decide whether executing such simultaneous configuring depend on the RRM measurement demand.

2.1.2 Max number of concurrent gap across all FRs for per-FR gap capable UEs

We should note that it was agreed to assume max 2 MGs in an FR as a starting point during 99 meeting, which is an important background to discuss this issue. The following options were discussed during last meeting:

- Open issue - Option 1: 3 - Option 2: 4 - Option 3: Up to UE capability

From our view, each FR shall be treated equally for the per-FR gap capable UE. Based on this understanding, the max number of concurrent gaps across all FRs should be 4. For the number of 3, which means one FR is reduced compared with the other FR, we can find any excuse for such unfair reduction.

Proposal 3: Based on the starting point of max 2 MGs in an FR, we support 4 MGs for the two FRs.

We can summarize the possible acceptable gap combinations according our above analysis. Our modifications were highlighted in yellow.

| Index | # of simultaneous MG |  |  | RAN4 conclusion |
| --- | --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | FFS Supported |
| 4 | 0 | 1 | 1 | FFS Supported |
| 5 | 1 | 1 | 1 | FFS |
| 6 | 2 | 2 | 0 | FFS Supported |
| 7 | 0 | 0 | 1 | Supported |
| 8 | 1 | 1 | 0 | Supported |
| 9 | 1 | 0 | 0 | Supported |
| 10 | 0 | 1 | 0 | Supported |
| 11 | 2 | 0 | 0 | Supported |
| 12 | 0 | 2 | 0 | Supported |

2.2 Overlapping issues

2.2.1 Proximity condition for overlapping

In order to avoid the heavy interruption in concurrent gaps configuration, the gap collision can be define then some of the unnecessary gap occasion can be dropped. It has been approved that two types of collision defined for the overlapping case, i.e.

- Condition #1: The gaps are physically fully or partially overlapping in time domain

- Condition #2: The gaps are not physically overlapping in time domain but the minimal distance between the two gap instances is equal or less to X

- Until now only the value of X for FR2 is still suspending.

- following conditions apply - Condition #1: The gaps are physically fully or partially overlapping in time domain - Condition #2: The gaps are not physically overlapping in time domain but the minimal distance between the two gap instances is equal or less to X - X = 1 or 4 ms for FR1 - X = [1, 2, or 4] ms for FR2 - FFS if split between FR1/FR2 is needed

We can not understand the technical reason for the existence of X=2 ms for FR2. If without specific technical consideration, we suggest using the unified candidate values for both FR1 and FR2. So that the judgement from UE can be simple. Especially for inter-band CA with the band combination between FR1 and FR2, it is hard to determine when to apply the candidates for FR1 and when to apply the candidates for FR2 if two different candidate ranges defined. So we prefer X = 1 or 4 ms for FR2.

- Proposal 4: If without specific technical consideration for X=2, we suggest using unified candidate values for both FR1 and FR2.

2.2.2 Rule for colliding gap occasions, if one of FO, FPO, PFO, PPO cases is introduced

The possible options for this issue is still diverse, the following options discussed during 101 meeting:

- Open issue - Option 1: Priority rule - UE will only do the measurement w.r.t. the gap with higher priority on all colliding occasions - The priority can be configurable or fixed - FFS whether to resume data scheduling during dropped gap occasions - Option 2: Gap-sharing rule - Given X% gap sharing, the measurement w.r.t. one gap will share roughly X% of the time, while the other gap shares the remaining - The sharing factor can be configurable or fixed - [Data schedule is not expected during both gaps] - Option 3: A hybrid priority and gap-sharing rule - Sharing is only considered between equal priority gaps - Data scheduling is expected during the dropped gap occasions - Option 4: Gap cancellation rule - A pattern telling UE how to drop gaps in each colliding occasion. - Data scheduling is expected during the dropped gap occasions - Option 5: Compromised proposal from moderator - Introduce gap sharing rule. - Request RAN2 to reserve some RRC signaling for different sharing factors. - The signalling design may consider the possibility of resuming data scheduling on dropped gaps - Rel-17 requirements will only consider sharing ratios 0% and 100%.

First, Option 3 is not operable. The above section 2.1.2 has clarified that concurrent gap supports a maximum of two per-UE gaps or a maximum of two per-FR gaps per FR. Since as much as two gaps existing for one FR, We cannot talk about the hybrid of priority+sharing, only either priority or sharing applying. Only for the case of more than two gaps configuring, the hybrid of priority+sharing are possible. But such case is beyond the agreed max number of concurrent gaps. Therefore, Option3 can be excluded first.

For Option 4, which is similar as Option 1. We understand it is hard for the NW to determine the exact canceling pattern in advance, compared with configuring canceling pattern, configuring canceling rule or priority rule is more feasible. So we believe keep Option 1 existing, correspondingly Option 4 can be removed.

For Option 5, which only consider sharing ratios 0% and 100%, in our opinion, which equals to no sharing. In that case, only configuring 1 gap is enough. Configuring 2 concurrent gaps and meanwhile set the sharing ratios as 0% and 100%, which is too redundant. So Option 5 can also be removed.

Finally only Option 1 and Option 2 are keep as reasonable candidates. The specific operation mechanism under Option 1 and Option 2 are analyzed as follows.

Option 1 can really handle the overlapping issue well. Once a gap1 instance collides with a gap2 instance, depending on the priority order, UE can decide which instance should be canceled. However, since not any possible overlapping case within FO/FPO/PFO/PPO was precluded, if applying Option 1 for all of these overlapping cases, then the possible use cases of configuring concurrent gaps would be quite limited. Probably the only use case is configuring high priority on the gap with longer MGRP. For other cases, option 1 is no sense since the gap with lower priority would be always skipped, only configuring one gap with smaller MGPR is enough. So, Option 1 will limit the use cases of concurrent gaps.

For Option 2, how to realize the sharing between multiple gaps should be studied concretely. Firstly we want to clarify that the sharing here is different from the gap sharing between intra-f and inter-f measurements in legacy Rel-16. The gap sharing in legacy Rel-16 means a gap is shared between {intra-f MO with gap, inter-f MO with gap, inter-RAT MO, intra-f MO without gap but SMTC totally overlapping with gap, inter-f MO without gap but SMTC totally overlapping with gap}. However the sharing here means between all overlapping instances, UE determine the ratio of instances belonging to gap1 should be kept according to the sharing factor, and then for the ratio of (1- sharing factor), the instances belonging to gap2 can be kept. As shown in Figure 1 below. For each overlapping instance, whether canceling gap2 or gap1, which depends on UE implementation, just similar as the gap sharing in legacy Rel-16, each gap instance should be allocated to which MO, it is not specified in the standard.

Figure 1 An example of sharing rule between gap1 and gap2

To sum up, Option 2 is more feasible and preferred by us.

Proposal 5: Option 3, 4 and 5 can be removed firstly, Option 1 and 2 can be kept.

Proposal 6: Between Option 1 and Option 2, we believe Option 2 is more flexible and preferred by us.

2.4 Overhead

For the issue of whether to define an overhead cap for concurrent gaps, the following options were discussed during last meeting:

- Option 1: Yes - Option 2: No - Option 3: Introduce a UE capability for those UE who does not need cap

From our perspective, the overhead can fully controlled by NW. Further more, with the help of collision handling, which can alleviate the throughput deterioration caused by too heavy overhead of gaps. Therefore, throughput loss caused by concurrent gaps is more controllable, so we think define an overhead cap is unnecessary. It is funny that the NW configures its own concurrent MGs and then configures an overhead cap to restrict the concurrent MGs. Why not take overhead cap into account when configuring concurrent MG? So, we do not think it is necessary to define such overhead cap and send it to NW implementation.

Proposal 7: It is no need to define an overhead cap for concurrent gaps. NW can fully control the concurrent gaps overhead when configuring.

2.5 Measurement requirements

2.5.1Further clarification on limitation that each reference signal can only be measured in one MG pattern

To specify the association between the multiple frequency layer and concurrent gaps, one limitation was agreed in 101 meeting as below:

- Measurements for different frequency layers but with the same reference signal can be associated to different concurrent MGs

We believe there is some confusion when applying this limitation into the association between the CSI-RS frequency layers and concurrent gaps. The focus is how to interpret the highlighted ‘same reference signal’.

Compared with the signaling structure of SSB MO, the signaling structure of CSI-RS MO is more complicated. The structure of CSI-RS MO is listed as follows, to clearly showing, only the configuration skeleton was kept. As shown that within one CSI-RS MO, one CSI-RS-ResourceConfigMobility can be included, and within such CSI-RS-ResourceConfigMobility, as much as 96(maxNrofCSI-RS-CellsRRM) CSI-RS-CellMobility can be included. Each CSI-RS-CellMobility can contain as much as 96(maxNrofCSI-RS-ResourcesRRM) CSI-RS-Resource-Mobility. The so called ‘same reference signal’ means the same CSI-RS-CellMobility or the same CSI-RS-Resource-Mobility? We prefer the latter, since for the former, it’s too strict. But we believe this ambiguous should be clarified.

MeasObjectNR ::= SEQUENCE { refFreqCSI-RS ARFCN-ValueNR OPTIONAL, -- Cond CSI-RS referenceSignalConfig ReferenceSignalConfig, cellsToRemoveList PCI-List OPTIONAL, -- Need N cellsToAddModList CellsToAddModList OPTIONAL, -- Need N ..., } ReferenceSignalConfig::= SEQUENCE { ssb-ConfigMobility SSB-ConfigMobility OPTIONAL, -- Need M csi-rs-ResourceConfigMobility SetupRelease { CSI-RS-ResourceConfigMobility } OPTIONAL -- Need M } CSI-RS-ResourceConfigMobility ::= SEQUENCE { subcarrierSpacing SubcarrierSpacing, csi-RS-CellList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility, ..., } CSI-RS-CellMobility ::= SEQUENCE { cellId PhysCellId, csi-rs-MeasurementBW SEQUENCE { nrofPRBs ENUMERATED { size24, size48, size96, size192, size264}, startPRB INTEGER(0..2169) }, density ENUMERATED {d1,d3} OPTIONAL, -- Need R csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility } CSI-RS-Resource-Mobility ::= SEQUENCE { csi-RS-Index CSI-RS-Index, slotConfig CHOICE { ms4 INTEGER (0..31), ms5 INTEGER (0..39), ms10 INTEGER (0..79), ms20 INTEGER (0..159), ms40 INTEGER (0..319) }, associatedSSB SEQUENCE { ssb-Index SSB-Index, isQuasiColocated BOOLEAN } OPTIONAL, -- Need R frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)) }, firstOFDMSymbolInTimeDomain INTEGER (0..13), sequenceGenerationConfig INTEGER (0..1023), ... }

Proposal 8: Some further clarification should be added into the limitation of “Measurements for different frequency layers but with the same reference signal can be associated to different concurrent MGs” for the case of CSI-RS frequency layer. The meaning of “same reference signal” is ambiguous.

2.5.2 Measurement delay

For the requirement of measurement delay, it has been split into two discussion: The measurement delay within gap and outside gap. For both of them still FFS during 101 meeting.

For the discussion of measurement delay outside gap case, in our view, one issue should be firstly determined that whether an gap should be associated to a MO/frequency layer without gap. As summarized in below, for different cases, the measurement period requirement are different.

Table 1 Summary of legacy Rel-16 Measurement period

| Intra-f or Inter-f | With or without gap | Overlapping between gap and SMTC | Kp | CSSF | Measurement period (Take DRX cycle≤ 320ms case as example) |
| --- | --- | --- | --- | --- | --- |
| Intra-f | Without gap | totally overlapping | 1 | CSSFwithin_gap,i | FR1: max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra FR2: max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
|  |  | partial overlapping | Kp = 1/(1- (SMTC period /MGRP)) | CSSFoutside_gap,i |  |
|  |  | non-overlapping | 1 | CSSFoutside_gap,i |  |
| Intra-f | With gap | All cases | N/A | CSSFwithin_gap,i | FR1: max(200ms, ceil(1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra FR2: max(400ms, ceil(1.5 x Mmeas_period with_gaps) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
|  |  |  |  | CSSFwithin_gap,i |  |
|  |  |  |  | CSSFwithin_gap,i |  |
| Inter-f | Without gap | totally overlapping | 1 | CSSFwithin_gap,i | FR1: max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter FR2: max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
|  |  | partial overlapping | Kp = 1/(1- (SMTC period /MGRP)) | CSSFoutside_gap,i |  |
|  |  | non-overlapping | 1 | CSSFoutside_gap,i |  |
| Inter-f | With gap | All cases | N/A | CSSFwithin_gap,i | FR1: Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter FR2: Max(400ms, (1.5 Mmeas_period_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |

We believe even for the MO/frequency layer without gap, NW should also associate a gap within concurrent gaps for such MO/frequency layer, so that UE can determine whether the measurement period for this MO/frequency should be calculated based on which principle. e.g. in Figure 2, if not providing the associated gap for the MO SMTC configuration, whether the case is partial overlapping or totally overlapping? As summarized in table 1, for different cases, different Kp or CSSF should be applied. So, we suggest NW should configure the associated gap even for a MO/frequency layer without gap.

Figure 2 An example of a MO/frequency layer configuration under the concurrent gap configuration

Proposal 9: In order to identify the Kp and CSSF according to the exact overlapping case, we suggest NW should configure the associated gap between concurrent gaps even for a MO/frequency layer without gap.

For the discussion of measurement delay within gap case, we can not find the demand of improvement. Re-using the mechanism of CSSFwithin_gap,i in legacy Rel-16 for each gap is enough. For all the MOs/frequency layers with gap which associated to a same gap, they all participate in the sharing of this gap.

Proposal 10: For the measurement delay within gap case, not need any new solution. Just re-using the mechanism of CSSFwithin_gap,i in legacy Rel-16 for each gap is enough. All MOs/frequency layers with gap associated with a same gap would participate in the sharing of this gap.

- Conclusion

In this contribution, we have the following observations and proposals for multiple concurrent and independent MGs:

- Proposal 1: Since the demand of PRS measurement, UE should support simultaneous configuring per-UE gap and per-FR gap for the UE being capable of per-FR gap and concurrent gaps.

- Proposal 2: Once the simultaneous configuring supported, no need to introduce additional limitation, NW can decide whether executing such simultaneous configuring depend on the RRM measurement demand. Proposal 3: Based on the starting point of max 2 MGs in an FR, we support 4 MGs for the two FRs.

- Proposal 4: If without specific technical consideration for X=2, we suggest using unified candidate values for both FR1 and FR2.

- Proposal 5: Option 3, 4 and 5 can be removed firstly, Option 1 and 2 can be kept.

- Proposal 6: Between Option 1 and Option 2, we believe Option 2 is more flexible and preferred by us.

- Proposal 7: It is no need to define an overhead cap for concurrent gaps. NW can fully control the concurrent gaps overhead when configuring.

- Proposal 8: Some further clarification should be added into the limitation of “Measurements for different frequency layers but with the same reference signal can be associated to different concurrent MGs” for the case of CSI-RS frequency layer. The meaning of “same reference signal” is ambiguous.

Proposal 9: In order to identify the Kp and CSSF according to the exact overlapping case, we suggest NW should configure the associated gap between concurrent gaps even for a MO/frequency layer without gap.

Proposal 10: For the measurement delay within gap case, not need any new solution. Just re-using the mechanism of CSSFwithin_gap,i in legacy Rel-16 for each gap is enough. All MOs/frequency layers with gap associated with a same gap would participate in the sharing of this gap.

- Reference

R4-2120303 “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc.