3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2200589

Electronic Meeting, 17st - 25th Jan, 2022

Source:	ZTE Corporation

Title:	Discussion on UL gap for Tx power management

Agenda Item:	6.4.3.1

Document for:	Approval

- Introduction

In 101 e-meeting, one WF [1] was approved, the agreements are summarized as follows.

- Tx power management: RF aspect Agreement: - The minimum Delta EIRP gain is at least max((Ppeak_EIRP-[21]dBm-margin) + 10*log10(Z/20), 3dB), where the margin is 2dB, where - Ppeak_EIRP is peak EIPR with zero MPR - Margin is the implementation margin including false alarm and etc - Z is duty cycle in number of percentage for reference measurement channel, e.g. when UL duty cycle is 10%, Z=10. Agreement: - UE will report P-MPRgapon when UL gap is activated - At most UE should report 0~3dB P-MPR in the PHR - UE supports UL gap shall also support R16 MPE reporting at least when UL gap is activated. - It is FFS about P-MPRgapoff reporting when UL gap is not activated and the related delta P-MPR, i.e. (P-MPRgapoff -P-MPRgapon). - Tx power management:RRM aspect Agreements: - In R17, the following 4 gap configurations are introduced - The same requirements should be specified for all gap configurations - UGL indicates the number of consecutive static UL slots configured as UL gap per UGRP UGL [ms]; UGRP [ms]; UGL/UGRP. ULGP #0; 1.0; 20; 5%. ULGP #1; 1.0; 40; 2.5%. ULGP #2; 0.5; 160; ~0.31%. ULGP #3; 0.125; 5; 2.5%. Agreements: - UL gaps are configured by the network using RRC configuration. - UL gaps are deconfigured by the network using RRC configuration. - Related to activation and deactivation of UL gaps: - The UL gaps can be activated when configured (using RRC signalling). - FFS: The UL gaps can additionally and optionally be activated and deactivated using MAC command after UL gap is configured by RRC Signaling - The UL gaps are deactivated when deconfigured (using RRC signalling). Agreements: - With the agreement that UE can explicitly indicate to NW on “need for UL gap” and “no need for UL gap”, RAN4 will leave the detailed signaling design to RAN2 Agreements: - Once the UL gap is configured, and activated, the UE will perform BPS sensing only in the static UL slot, i.e. no DL slot or special slot will be used as UL gap. For example, in case of ULGP#1, DDDSU configuration with 120KHz SCS, the following figure depicts the available slots for UL gap. - UE is not expected to be scheduled with UL transmission during the gap. Details FFS, e.g., RACH Agreements: - No interruption across FR. FR2 UL gap does not cause FR1 interruption. Agreements: - UL gap slot is static UL slot. UL gap slot applies same TA as other UL slots. Agreements: - No conflict b/w TDD config and UL gap. UL gap is always configured in static UL slot Agreements: - FFS the impact on measurement gap configuration and requirement due to UL gap.

According to the conclusions achieved during 101 e-meeting, for the Tx power management use case, the detailed performance metric and the corresponding threshold have been identified. But for the test methodology, without any progress. For the application of UL gap for Tx power management, the conformance test should also be clarified. In this document, we give some further discussion on the remaining open issues.

- Other additional requirements to be tested

- Test methodology

- Discussion

2.1 Other additional requirements to be tested

It was agreed that for Tx power management use case, only type 1 gap is considered(all UE RF requirements will apply). For type 1 gap, NW can assign those resources within UL gap to other UEs for UL transmission, so the existing RF requirements should not be broken, such as transmit off power. For BPS with help of UL gap operation, the target UE still need to emit the radar signal to detect the body, in order to avoid interference to other UE scheduled UL transmission during the UL gap, the transmit off power requirement should be ensured.

Proposal 1: UE should also meet the existing transmit off power requirement during the BPS UL gap to avoid causing interference.

2.2 Test methodology

The UL gap for Tx power management is derived from MPE issue, which means that UE can use UL gap to reduce power reduction (P-MPR) through more accurate BPS. In fact, there are various method to archive the same purpose, like radar, thermal sensor, etc., but not all of these methods require UL gap to perform calibration. What we should verify is the gain brought by the introduction of UL gap itself, and the influence of other calibration method without UL gap need to be precluded. We believe this is a basic principle for the design of test setup.

It has been agreed that not introducing phantom in the test in last meeting, based on the UE category given by [2], four types of UE are listed as follows:

| Type of UE | P-MPR value when human targets are not close to Tx antenna |  | NOTE |
| --- | --- | --- | --- |
|  | Without UL gap | With UL gap |  |
| A | High | High | - UE cannot implement the human detection even if UL gaps are configured |
| B | Mid / Low | Mid / Low | - UE that shows good performance without UL gap and does not require any UL gaps |
| C (Targeted UE) | High | Low / Very Low | - UE that don't shows good performance without UL gap, so it requires any UL gaps for improvement |
| D | Mid / Low | Low / Very Low | - UE that shows good performance without UL gap, but it requires any UL gaps for further improvement. |

For Type A and Type B UEs, no performance gain can be brought by UL gap. For Type D UE, only weak gain can be brought by UL gap. Only Type C UE is our targeted UE, UL gap can bring obvious gain with the help of BPS during UL gap.

The following options are proposed during last meeting:

- Option 1: Based on P-MPR reporting without phantom to distinguish UE type A, B and C. - Option 2: Based on P-MPR reporting with phantom to distinguish UE type A, B, C and/or D - Option 3: Based on peak EIRP reporting without phantom - Option 4: Based on hybrid of peak EIRP reporting and P-MPR reporting with phantom - Option 5: Based on P-MPR reporting without phantom, but with RADAR jamming - Option 6: Leave the test method design to RAN5

We believe Option 1 and Option 2 can meet the primary purpose analyzed above, however considering the phantom is not allowed to introduce, Option 1 is preferred, i.e. the test setup can be shown as following table:

| Test step | Blocking/phantom | UL gap | P-MPR of UE |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Type A | Type B | Type C |
| Step 1 | No | Yes | High | Low | Low |
| Step 2 | No | No | High | Low | High |

Proposal 2: For the test methodology, Option 1 is preferred.

- Conclusion

In this contribution, we have the following proposals for UL gap Tx power management use case:

Proposal 1: UE should also meet the existing transmit off power requirement during the BPS UL gap to avoid causing interference.

Proposal 2: For the test methodology, Option 1 is preferred.

- Reference

R4-2119962, “WF on FR2 enhancement part 2: UL gaps”, Apple.

R4-2112635, “RF requirements and NW configuration for UL gap”, NTT DOCOMO.