3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200611

# Electronic Meeting, 17th to 25th Jan, 2022

Agenda Item:	6.7.1

Source: 	MediaTek Inc.

Title: 	Discussion on capability signalling for HPUE NR DC with UE supporting TxD

Document for:	Discussion

- 1. Introduction

This is a re-submission to continue discuss power class reporting for HPUE NR DC. The power class issue for HPUE has been discussed under HPUE SAR WI for several meetings in RAN4. A new WID was agreed in RAN-P#93 [1] for more discussion about CA/DC power class cases include PC3+PC3, PC3+PC2, PC2+PC3, PC2+PC2 for the corresponding bands within the combination. In this contribution, we further discuss UE capability signalling aspect which is more related to existing requirements.

- 2. Discussion

In the past, the NR power class signalling for EN-DC had been discussed in RAN4 for several meetings and it was agreed to apply new NR power class signalling in [2]. Similar situation could happen for PC2 NR-CA/DC cases that power class are comprised with following table (modified from [2]):

| Case | NR-CA/DC power class | PCell/MCG power class | SCell/SCG Power class |
| --- | --- | --- | --- |
| A | PC3 | PC3 | PC3 |
| B | PC2 | PC2 | PC3 |
| C | PC2 | PC3 | PC2 |
| D | PC3 | PC2 | PC3 |
| E | PC3 | PC3 | PC2 |
| F | PC2 | PC2 | PC2 |
| G | PC3 | PC2 | PC2 |
| H | PCX (>PC2) | PC2 | PC2 |
| I | PC2 | PC3 | PC3 |

Table 1. PC2 NR CA/DC power class configuration table

In Rel-17 RAN4 introduce a new feature, NR PC2 CA. It is foreseen that NR PC2 DC will soon appear since most of NR DC specs are referring to same band combo of NR CA combos. Per band power class under CA/DC operation could become an issue since there’s no signalling indicating them in current RAN2 specs. A possible solution is network refer to single band power class capability reporting when operate under CA/DC. However this solution is not proper for UE supporting TxD.

For example, case C in the table 1, an issue could happen in SCell/SCG that if UE supports PC2 for SCell/SCG under non-CA mode and TxD(2Tx) is implemented to enable PC2 for the band, under CA/DC operation SCell/SCG can only support 1x23dBm  since 3Tx in FR1 configuration is not yet supported. Then the power class for SCell/SCG band would be different between non-CA operation and CA/DC mode. New signalling for PC2 NR CA/DC to tell per band power class per combination under CA/DC mode would be another solution to solve this issue.

| CA/DC power class (supported in RAN2 spec) | PCell/MCG power class in EN-DC (not supported in RAN2 spec) | SCell/SCG Power class in EN-DC (not supported in RAN2 spec) | Single band Power class (supported in RAN2 spec) |
| --- | --- | --- | --- |
| PC2 | PC2? PC3? | PC2? PC3? | PC2 with 2Tx |

Table2. PC2 NR CA/DC power class configuration issue

Observation 1: For PC2 NR CA/DC, there’s no signalling to indicate per band power class per combination that it is unclear how the network can handle uplink transmission power control.

Observation 2: For UE supporting non-CA PC2 operation with TxD(2TX) implementation, power class may be different for the band in CA/DC mode.

With the above observations, here’s our proposal,

Proposal 1: A new power class signalling per band per combination is needed for PC2 NR CA/DC starts from Rel-17.

Proposal 2: Send an LS to RAN2 to inform the new signalling is needed.

Conclusion

Observation 1: For PC2 NR CA/DC, there’s no signalling to indicate per band power class per combination that it is unclear how the network can handle power control.

Observation 2: For UE supporting non-CA PC2 operation with TxD(2TX) implementation, power class may be different for the band in CA/DC mode.

Proposal 1: A new power class signalling per band per combination is needed for PC2 NR CA/DC starts from Rel-17.

Proposal 2: Send an LS to RAN2 to inform the new signalling is needed.

Reference:

- RP-212622_New WID Increasing UE power high limit for CA and DC, RAN-P#93

- RP-192103 On EN-DC power class_v1, RAN-P#85