3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2200652

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.22.2.2

Source: 	vivo

Title: 	Further discussion on efficient activation/de-activation mechanism for one SCG

Document for:	Discussion

- Introduction

In the RAN4 #101-e meeting, the general aspects of activation/deactivation mechanism for SCG requirements were discussed. The agreements and open issues are captured in the WF [1].

In this paper we provide our views on the open issues.

- Discussion

The following was captured in the WF [1] on activation/deactivation mechanism for SCG.

- RAN4 are to discuss the potential RRM impacts due to efficient activation/de-activation mechanism for one SCG - Measurement requirements for deactivated SCG (Issue 2-1) - DRX configuration for deactivated SCG measurement requirements (Issue 2-1-1) - updated CSSF (if L3 measurements on deactivated PSCell is agreed to be relaxed in RAN2) (Issue 2-1-2) - “measCycleSCell” like scheme (if L3 measurements on deactivated PSCell is agreed to be relaxed in RAN2) (Issue 2-1-3) - MCG measurement requirements (Issue 2-1-4) - whether existing measurements reporting requirements can apply for measurement on deactivated PSCell (Issue 2-1-5) - SCG Activation/deactivation delay (Issue 2-2) - UE processing time (Tprocessing) in PSCell activation delay (Issue 2-2-1) - time/frequency tracking time (T∆) in PSCell activation delay (Issue 2-2-2) - Requirements for PSCell activation delay (Issue 2-2-3) - Interruption requirements (Issue 2-3) - Baseline for interruption due to PSCell activation/deactivation (Issue 2-3-1) - Whether to define interruption due to PSCell activation/deactivation in asynchronous deployment (Issue 2-3-2) - Interruption due to L3 measurement on deactivated SCG (Issue 2-3-3) - RLM/BFD/BFR/Beam management on deactivated PSCell (Issue 2-4) - Whether RLM/BFD requirements on deactivated PSCell can be relaxed (Issue 2-4-2) - Interruption requirement due to RLM and BFD on deactivated PSCell (Issue 2-4-3) - Interruption due to L1-RSRP requirements on deactivated PSCell (Issue 2-4-4) - Others (Issue 2-5) - whether UE shall meet the existing Te and Tq when PSCell is deactivated (Issue 2-5-1) - if the answer of issue 2-5-1 is yes, SSB monitoring rate (Issue 2-5-2)

2.1 Measurement requirements for deactivated SCG

- DRX configuration for deactivated SCG measurement requirements (Issue 2-1-1) - Option 1: SCG DRX cycle should be applied when specifying the L3 requirements of deactivated SCG. - Option 2: for deactivated SCG, the DRX cycle that is used to determine measurement period scaling can be replaced by a hypothetical DRX cycle (if RAN2 agree to introduce). - Option 3: wait for RAN2 - “measCycleSCell” like scheme (if L3 measurements on deactivated PSCell is agreed to be relaxed in RAN2) (Issue 2-1-3) - Option 1: wait for RAN2 progress on relaxed measurement cycle on deactivated PSCell - Option 1a: Send LS to RAN2 to ask whether any new parameter for relaxed measurement cycle on deactivated PSCell is planned to be introduced and whether any potential issues from RAN2 perspective are foreseen if measCycleSCell is used for deactivated PSCell L3 measurement period determination which is currently based on SMTC period. - Option 2: use the principles from deactivated SCell (longer cycles) - Option 3: For deactivated PSCell L3 measurement period requirements, SMTC period is replaced with measCycleSCell

Since the DRX cycle and measCycleSCell configuration are the two main factor that affect the measurement period, Issue 2-1-1 and Issue 2-1-3 are discussed together here. As mentioned in Issue 2-1-3 Option 2, we think it is reasonable to use the principles from deactivated SCell. Take Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1) as an example here.

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp)x max(measCycleSCell, DRX cycle) x CSSFintra.

For deactivated SCell measurement, measCycleSCell is important to determine the measurement period regardless of whether or not DRX is configured. We think similar parameter is also important for deactivated PSCell measurement. Without this parameter, the measurement period may need to rely on DRX cycle which may not be configured. If no DRX cycle is configured, then UE measurements could be too frequent for deactivated PSCell measurement. Having fixed DRX cycle, e.g., hypothetical DRX cycle, it will lose the feasibility of controlling measurement delay and balancing power saving. Therefore, for deactivated PSCell, we think it is necessary to have a similar parameter of measCycleSCell, which is used for deactivated SCell measurement only according to existing spec.

Since meascycleSCell cannot be used for deactived PSCell measurement directly, it would be necessary to send LS to ask RAN2 to define a similar parameter to measCycleSCell to be used for deactivated PSCell measurement.

Proposal 1: Measurement period requirements for deactivated PSCell is based on a configured measurement cycle, which is similar to measCycleSCell for deactivated SCell measurement.

Proposal 2: Send LS to ask RAN2 to define measurement cycle for deactivated PSCell measurement.

- updated CSSF (if L3 measurements on deactivated PSCell is agreed to be relaxed in RAN2) (Issue 2-1-2) - Option 1: updated CSSF on deactivated PSCell updated CSSF can relax measurement on deactivated PSCell and increase measurement on other active SCells - Option 2: Do not consider modifying deactivated PSCell measurement in terms of CSSF - Option 3: wait for RAN2 conclusion on whether to relax L3 measurement on deactivated PSCell.

In our view, it is beneficial to update CSSF to relax measurement on deactivated PSCell. This can indeed increase the measurement opportunities of other active SCells. On the other hand, we need to consider the case the deactivated PSCell may not be activated as PSCell for SCG activation. In this case, it is not necessary to prioritize the deactivated PSCell measurement. As for how to update CSSF to relax measurement on deactivated PSCell, the value of CSSF for PSCC should be increased. The measurement on deactivated PSCC and SCC would have same priority.

Proposal 3: Update CSSF to relax measurement on deactivated PSCell. The value of CSSF for PSCC should be increased and the measurement on deactivated PSCC and SCC would have same priority.

- MCG measurement requirements (Issue 2-1-4) - Option 1(Qualcomm, Apple, MTK, Huawei, vivo, Ericsson): RAN4 doesn’t need to specifically define new requirement for MCG measurement. However, MCG measurement requirement can be adjusted if any impact due to deactivated SCG is identified. - Option 2(Nokia): There is no impact on MCG measurement requirements due to deactivated PSCell if MCG and SCG are in different FR.

he CSSF of MCG SCells depends on the measurement of activated SCells and deactivated SCells/PSCell. In our opinion, If CSSF follows the update method of Issue 2-1-2, the measurement frequency on SCC and PSCC should be the same. we still think that there is no need to specifically define new requirements for MCG measurement, the MCG measurement requirements can be adapted and changed accordingly if it is necessary.

Proposal 4: There is no need to specifically define new requirements for MCG measurement and its corresponding measurement requirements can be adapted and changed accordingly.

- whether existing measurements reporting requirements can apply for measurement on deactivated PSCell (Issue 2-1-5) - Revised Option 1(Qualcomm, Huawei, vivo, Ericsson): The existing measurements reporting requirements can be used as a baseline for measurement on deactivated PSCell. - FFS whether the same requirements in terms of accuracy and side condition shall be applied to measurements on deactivated PSCell. - Option 2: existing measurements reporting requirements can apply for measurement on deactivated PSCell

The existing measurement reporting requirements can be applied to deactivated PSCell. As for whether the same requirements in terms of accuracy and side condition shall be applied to measurements on deactivated PSCell, we think that there is no difference in measurement report requirements between PSCell and SCell in the activated state. Therefore, in the deactivated state, we do not observe any reasons to change the requirements in terms of accuracy and side condition for deactivated PSCell.

Proposal 5: The existing measurement report requirements can be applied to deactivated PSCells.

2.2 SCG Activation/deactivation delay

- UE processing time (Tprocessing) in PSCell activation delay (Issue 2-2-1) - Option 1: - Tprocessing = 20ms NR PSCell is in FR1 in EN-DC. - Tprocessing = 40 ms if NR PSCell is in FR2 in EN-DC or NR-DC - Option 2: Tprocessing = 0ms - Option 3a: Tprocessing defined in PSCell addition can be reused if PSCell is added and directly enter the activated status; otherwise, Tprocessing = 0 can be expected when PSCell/SCell is activated from a deactivated status - Option 3b: Tprocessing = 0 shall be assumed at least for the case where PSCell being activated is same cell as previously being deactivated, irrespective of whether RA is carried out as part of the activation. FFS regarding SCG mobility (PSCell change) during deactivated SCG state. - Other options are not precluded.

We prefer to Option 3a and Option 3b. It is still necessary to formulate different value of Tprocessing based on the state of the PSCell before activation:

1. When PSCell being activated is same cell as previously being deactivated, the software does not need to be programmed again, so Tprocessing could be zero.

2. When PSCell is added and directly enter the activated status, Tprocessing defined in PSCell addition can be reused in this case and Tprocessing = 40ms.

3. Regarding SCG mobility (PSCell change) during deactivated SCG state, in this scenario, Tprocessing would be the same as for PSCell addition.

Proposal 6: UE processing time (Tprocessing) in PSCell activation delay can be defined,

- When PSCell being activated is same cell as previously being deactivated, Tprocessing could be zero.

- When PSCell is added and directly enter the activated status, Tprocessing = 40ms.

- Regarding SCG mobility (PSCell change) during deactivated SCG state, in this scenario, Tprocessing would be the same as for PSCell addition.

- time/frequency tracking time (T∆) in PSCell activation delay (Issue 2-2-2) - Option 1: Time/frequency tracking time (T∆) in PSCell activation delay is needed - Option 2: Time/frequency tracking time defined in PSCell addition can be reused if PSCell is added and directly enter the activated status; otherwise, time/frequency tracking time defined in SCell activation can be removed when PSCell/Scell is activated from a deactivated status - Option 3: time/frequency tracking time (T∆) in PSCell activation delay is needed T∆ = 0 is to be assumed when UE is configured with RLM/BFD measurements in PSCell in addition to RRM measurements. - Other options are not precluded.

In our opinion, time/frequency tracking time (T∆) in PSCell activation delay is needed to obtain fine time tracking and acquire full timing information in general. When RLM/BFD measurements are configured and RLM/BFD measurements were performed in PSCell deactivated state, UE can obtain fine timing information from the RLM/BFD measurements, then it will maintain fine tracking of time and frequency in PSCell. In the case, we think T∆ can be 0.

Proposal 7: In general, time/frequency tracking time (T∆) in PSCell activation delay is needed to obtain fine timing information. When RLM/BFD measurements are configured, fine time and frequency tracking information can be available for PSCell. In the case, T∆ can be 0.

- Requirements for PSCell activation delay (Issue 2-2-3) - Option 1: - RACH based PSCell activation delay can be defined as Tconfig_PSCell = TRRC_delay + [Tprocessing] + Tsearch + [T∆]+ TPSCell_ DU + 2 ms Where TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. - RACH-less PSCell activation delay can be defined as Tconfig_PSCell = TRRC_delay + [Tprocessing] + Tsearch + [T∆]+ TIU + 2 ms where TIU is the interruption uncertainty in acquiring the first PUSCH transmission occasion when UE is configured with RACH-less SCG. Note: the value of Tprocessing and T∆ depends on Issue 2-2-1 and Issue 2-2-2. - Option 2: Tconfig_PSCell = TRRC_delay + [Tsearch] + TRF_start + T∆ + [TPSCell_ DU] + 2 ms - Option 3: Tactivate_SCG = TRRC_delay + T∆ + TPSCell_ DU + 2 ms

When PSCell being activated is same cell as previously being deactivated, Tsearch can be 0. But if PSCell is unknown, neither the time for AGC settling and PSS/SSS detection nor he SW processing time needed by UE can be deleted. In this scenario, Tprocessing and Tsearch need to be retained. In order to consider the uniformity of the formula in different scenarios, in general, Option 1 is acceptable as the baseline of PSCell activation delay.

Proposal 8: In order to consider the uniformity of the formula in different scenarios, in general, Option 1 is used as baseline for PSCell activation delay.

2.3 Interruption requirements

- Baseline for interruption due to PSCell activation/deactivation (Issue 2-3-1) - If PSCell is added and directly enter the activated status Existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length. - If PSCell is activated from a deactivated status - Option 1: existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length. - Option 2: interruption requirement for Scell activation can be reused (Table 8.2.4.2.2-1)

When PSCell is activated from a deactivated status, the interruption procedure is similar to interruption due to SCell activation/deactivation, and interruption length for SCell activation/deactivation for inter-band DC/CA is 0.5ms. From this point, interruption requirement for Scell activation can be reused.

When PSCell is added and directly enter the activated status, the interruption procedure is similar to interruption due to PSCell addition/release. Therefore, existing requirements for interruption due to PSCell addition/release can be used as baseline. And it needs longer interruption length, i.e., 1ms.

Proposal 9:

- If PSCell is added and directly enter the activated status

Existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length.

- If PSCell is activated from a deactivated status

interruption requirement for Scell activation can be reused (Table 8.2.4.2.2-1)

- Whether to define interruption due to PSCell activation/deactivation in asynchronous deployment (Issue 2-3-2) - Option 1: additional interruption shall be allowed due to PSCell activation/deactivation in asynchronous deployment. - Revised option 2: - For SCG activation/deactivation in ENDC, When SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133. - For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

There is such a description about asynchronous deployment in TS38.133 8.2.4.2.2: “the requirements for Async apply for asynchronous NR-DC if the active serving cell is not in the same CG as any of the SCell or PSCells being added or released”. In our opinion, for PSCell activation/deactivation, the active serving cell is not in the same CG. The additional interruption shall be allowed due to PSCell activation/deactivation as a kind of asynchronous deployment. As for the more specific two cases in EN-DC or NR-DC, the interruption requirement given by Option 2 is reasonable: For SCG activation/deactivation in ENDC, the interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133. And for SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

Proposal 10: For PSCell activation/deactivation, the active serving cell is not in the same CG. The additional interruption shall be allowed due to PSCell activation/deactivation in asynchronous deployment. As for the more specific two cases in EN-DC or NR-DC:

- For SCG activation/deactivation in EN-DC,

When SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133.

- For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

- Interruption due to L3 measurement on deactivated SCG (Issue 2-3-3) - Option 1: Use the existing interruptions during measurements on deactivated SCC. - Option 2: - For deactivated PSCell measurements, the current interruption requirement during RRM measurements on SCell dormancy applies ([X]%). - For deactivated SCell measurements, the current interruption requirement during measurements on deactivated inter-band SCC applies. - Other options are not precluded.

For deactivated SCell measurements, the current interruption requirement during measurements on deactivated inter-band SCC can be applied.

When PSCell is configured for measCycleSCell, interruption requirement on deactivated inter-band SCC can be uesd directly. For deactivated PSCell measurements, from our point of view in Issue 2-1-3, we think it is necessary to have a similar parameter of measCycleSCell.

Proposal 11: The current interruption requirement on deactivated inter-band SCC can be reused directly for L3 measurement for deactivated SCG.

2.4 RLM/BFD/BFR/Beam management on deactivated PSCell

- Whether RLM/BFD requirements on deactivated PSCell can be relaxed (Issue 2-4-2) - Option 1: Relaxed RLM/BFD measurement requirements for deactivated PSCell are defined. FFS: The conclusion for RLM/BFD measurement relaxation in power saving enhancement WI can be considered. - Option 2: RLM/BFD requirements for deactivated PSCell follow the deactivated PSCell measurement cycle (the TDRX parameter used in exiting requirements) - Option 3: wait for RAN2 conclusion

In the last meeting, Relaxed RLM/BFD measurement requirements for deactivated PSCell are defined. The table below is the agreement on RLM/BFD measurement relaxation for power saving enhancement [2]. We think it is reasonable to use this as the starting point for Relaxed RLM/BFD requirements on deactivated PSCell.

- The relaxation factor for FR1: - TRS is the periodicity of SSB for the case of SSB based, and the periodicity of CSI-RS for the case of CSI-RS based. - K0, FR1 =1 for 80 ms < MAX(TDRX, TRS) ≤ 160 ms. - K1, FR1=[2, 3 or 4] for 40 ms < MAX(TDRX, TRS) ≤ 80 ms - K2, FR1=[2, 3, or 4] for MAX(TDRX, TRS) ≤ 40 ms - FFS select between [2,3,4] - The relaxation factor for FR2 SSB: - K0, FR2, SSB = 1 for [80] ms < MAX(TDRX, TSSB) ≤ 160 ms - K1, FR2, SSB= [1.5 or 2] for MAX(TDRX, TSSB) ≤ [80] ms for SSB based relaxation. - The relaxation factor for FR2 CSI-RS: - K0, FR2, CSI-RS =1 for 80 ms < MAX(TDRX, TCSI-RS) ≤ 160 ms - K1, FR2, CSI-RS = 2 for MAX(TDRX, TCSI-RS) ≤ 80 ms for CSI-RS based relaxation.

We know that the existing conclusion for RLM/BFD measurement relaxation in power saving enhancement WI is for activated state measurement. In the deactivated state, the measurement frequency would be lower than that in the activated state. Therefore, on this basis, it is considered to further relax the deactivated PSCell.

On this basis, we present the modified requirements for RLM/BFD requirements on deactivated PSCell below.

- The relaxation factor for FR1/FR2:

  - TRS is the periodicity of SSB for the case of SSB based, and the periodicity of CSI-RS for the case of CSI-RS based.

  - K0=2, for 160 ms < max (TDRX, TRS) ≤ 320 ms.

  - K0=4, for 80 ms < max (TDRX, TRS) ≤ 160 ms.

  - K1=8, for max (TDRX, TRS) ≤ 80 ms.

Proposal 12: The conclusion for RLM/BFD measurement relaxation in power saving enhancement WI coule be considered. The specific requirements on deactivated PSCell are:

- The relaxation factor for FR1/FR2:

  - TRS is the periodicity of SSB for the case of SSB based, and the periodicity of CSI-RS for the case of CSI-RS based.

  - K0=2, for 160 ms < max (TDRX, TRS) ≤ 320 ms.

  - K0=4, for 80 ms < max (TDRX, TRS) ≤ 160 ms.

  - K1=8, for max (TDRX, TRS) ≤ 80 ms.

- Interruption requirement due to RLM and BFD on deactivated PSCell (Issue 2-4-3) - Option 1: The same principle as the interruption due to SCell dormancy is applied ([X]%), details are FFS. - Option 2: RAN4 to consider at least the following scenarios when defining interruption requirements for deactivated SCG: - - RRM based on SSBs, - - RRM based on SSBs, RLM and BFD based on SSBs, - - RRM based on SSBs, RLM and BFD based on CSI-RS. - Other options are not precluded.

The interruption requirement due to SCell dormancy can be used as baseline. As for when resources overlap between RRM and RLM/BFD, although the measurement time will be affected, the impact of such changes will be reflected in the Psharing factor, there is no need to artificially classify scenarios and define different requirements to increase the complexity of UE implementation. At the same time, how to configure RRM measurements and/or RLM/BFD measurements can be appropriately managed by NW. From this point of view, it is unnecessary in our opinion to make different requirements for different scenarios in Option 2.

Proposal 13: To use the interruption requirement due to SCell dormancy([X]%) as the baseline for interruption requirement due to RLM and BFD on deactivated PSCell, details are FFS.

- Interruption due to L1-RSRP requirements on deactivated PSCell (Issue 2-4-4) - Option 1: the interruption requirement for dormant Scell measurement (8.2.4.2.13) can be reused. - Option 2: L1-RSRP measurement doesn’t be supported on PSCell.

Regarding whether L1-RSRP measurement for deactivated PSCell is supported, it seems that RAN2 does not yet have an accurate answer. We think that the specific requirements can wait for the final conclusion of RAN2.

Proposal 14: whether L1-RSRP measurement requirements for deactivated PSCell is supported, it can wait for the final conclusion of RAN2.

2.5 Others

- whether UE shall meet the existing Te and Tq when PSCell is deactivated (Issue 2-5-1) - Option 1: yes, the existing UE initial transmit timing error (Te) and gradual timing adjustment requirements are met at least until the TAT is running. - Option 2: further study

In our understanding, this issue talks about UL transmissions toward PSCell after PSCell activation. We think it is necessary to wait for further conclusion from RAN2 on whether there are any UL transmissions when PSCell is deactivated.

Proposal 15: whether UE shall meet the existing Te and Tq when PSCell is deactivated, we think it is necessary to wait for further conclusion from RAN2 on whether there are any UL transmissions when PSCell is deactivated.

- if the answer of issue 2-5-1 is yes, SSB monitoring rate (Issue 2-5-2) - Option 1: RAN4 further discusses whether to meet Te requirements the SSB should be monitored once every 160 ms or with the same rate with which the UE performs RRM requirements on PSCell. - Option 2: further study

This issue needs to be discussed further based on the conclusions of Issue 2-5-1.

Proposal 16: whether to meet Te requirements the SSB should be monitored once every 160 ms or with the same rate with which the UE performs RRM requirements on PSCell, it needs to be discussed further based on the conclusions of Issue 2-5-1.

- Conclusions

Proposal 1: Measurement period requirements for deactivated PSCell is based on a configured measurement cycle, which is similar to measCycleSCell for deactivated SCell measurement.

Proposal 2: Send LS to ask RAN2 to define measurement cycle for deactivated PSCell measurement.

Proposal 3: Update CSSF to relax measurement on deactivated PSCell. The value of CSSF for PSCC should be increased and the measurement on deactivated PSCC and SCC would have same priority.Proposal 4: There is no need to specifically define new requirements for MCG measurement and its corresponding measurement requirements can be adapted and changed accordingly.Proposal 5: The existing measurement report requirements can be applied to deactivated PSCells.Proposal 6: UE processing time (Tprocessing) in PSCell activation delay can be defined,

- When PSCell being activated is same cell as previously being deactivated, Tprocessing could be zero.

- When PSCell is added and directly enter the activated status, Tprocessing = 40ms.

- Regarding SCG mobility (PSCell change) during deactivated SCG state, in this scenario, Tprocessing would be the same as for PSCell addition.

Proposal 7: In general, time/frequency tracking time (T∆) in PSCell activation delay is needed to obtain fine timing information. When RLM/BFD measurements are configured, fine time and frequency tracking information can be available for PSCell. In the case, T∆ can be 0.

Proposal 8: In order to consider the uniformity of the formula in different scenarios, in general, Option 1 is used as baseline for PSCell activation delay.Proposal 9:

- If PSCell is added and directly enter the activated status

Existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length.

- If PSCell is activated from a deactivated status

interruption requirement for Scell activation can be reused (Table 8.2.4.2.2-1)Proposal 10: For PSCell activation/deactivation, the active serving cell is not in the same CG. The additional interruption shall be allowed due to PSCell activation/deactivation in asynchronous deployment. As for the more specific two cases in EN-DC or NR-DC:

- For SCG activation/deactivation in EN-DC,

When SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133.

- For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133.

Proposal 11: The current interruption requirement on deactivated inter-band SCC can be reused directly for L3 measurement for deactivated SCG.

Proposal 12: The conclusion for RLM/BFD measurement relaxation in power saving enhancement WI coule be considered. The specific requirements on deactivated PSCell are:

- The relaxation factor for FR1/FR2:

  - TRS is the periodicity of SSB for the case of SSB based, and the periodicity of CSI-RS for the case of CSI-RS based.

  - K0=2, for 160 ms < max (TDRX, TRS) ≤ 320 ms.

  - K0=4, for 80 ms < max (TDRX, TRS) ≤ 160 ms.

  - K1=8, for max (TDRX, TRS) ≤ 80 ms.

Proposal 13: To use the interruption requirement due to SCell dormancy([X]%) as the baseline for interruption requirement due to RLM and BFD on deactivated PSCell, details are FFS.

Proposal 14: whether L1-RSRP measurement requirements for deactivated PSCell is supported, it can wait for the final conclusion of RAN2.

Proposal 15: whether UE shall meet the existing Te and Tq when PSCell is deactivated, we think it is necessary to wait for further conclusion from RAN2 on whether there are any UL transmissions when PSCell is deactivated.

Proposal 16: whether to meet Te requirements the SSB should be monitored once every 160 ms or with the same rate with which the UE performs RRM requirements on PSCell, it needs to be discussed further based on the conclusions of Issue 2-5-1.

References

[1] R4-2120334 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei

[2] R4-2120313 WF on RLM/BFD relaxation for UE Power Saving enhancements, MediaTek inc