3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2200653

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.22.2.3

Source: 	vivo

Title: 	Further discussion on conditional PSCell change and addition

Document for:	Discussion

- Introduction

In the RAN4 #101-e meeting, it mainly discussed the general aspects of conditional PSCell change and addition requirements. The results of the agreement and outstanding issues are captured in the WF[1].

In this paper we are providing our views on the open issues.

- Discussion

The following was captured in the WF[1] on conditional PSCell change and addition.

- RAN4 are to discuss the potential RRM impacts due to conditional PSCell change and addition - TUE_preparation in Conditional PSCell change delay (Issue 3-1-2) - Tprocessing in conditional PSCell addition delay (Issue 3-2-1) - TUE_preparation in conditional PSCell addition delay (Issue 3-2-3)

- TUE_preparation in Conditional PSCell change delay (Issue 3-1-2) - Option 1(Ericsson): preparation time TUE_preparation is changed from 10 to 8ms. - Option 2 (Qualcomm, Apple, MTK, Huawei, vivo): preparation time TUE_preparation shall be 10ms.

The conditional PSCell change delay is defined as in clause 8.11B.2

Tconfig_PSCell_Conditional = TRRC_processing + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where

TUE_preparation is the UE preparation time for conditional PSCell change, and starts after UE realizes the condition of PSCell change is met and identity of new PSCell is determined.

More specifically, for RRC processing, conditional handover and conditional PSCell change have certain similarities, so we can use conditional handover RRC processing as a reference.

During CHO procedure, RRC processing time is splitted into two parts: TRRC_1 is defined the same as existing processing time for RRC reconfiguration message containing CHO configuration. And TRRC_2 is the Second segment of RRC processing time, starts after UE realizes the condition is met and identity of target cell is determined. During TRRC_2, UE is allowed to finish associated operations earlier and leave the source cell.

Similarly, as for conditional PSCell change, after finishing the measurement of the target PSCell and UE realizes the condition is met, it may include some RRC processing related to release the existing PSCell and apply the target PSCell configuration. It is common understanding that the operations in the Tpreparation as the second part contributes most to the total RRC processing delay. Therefore, it is reasonable to keep Tpreparation as 10ms.

Proposal 1: TUE_preparation in conditional PSCell change delay shall be 10ms.

- Tprocessing in conditional PSCell addition delay (Issue 3-2-1) - Option 1(Apple, ZTE, MTK, Huawei, vivo, Nokia): - For ENDC, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2; - For NRDC Tprocessing = 40 ms. - Option 2(Qualcomm, Ericsson): Tprocessing = 40ms is used regardless of FR. - TUE_preparation in conditional PSCell addition delay (Issue 3-2-3) - Option 1(Ericsson): preparation time TUE_preparation is changed from 10ms to 8ms. - Option 2 (Qualcomm, Apple, MTK, Huawei, vivo): preparation time TUE_preparation shall be 10ms.

In RAN4#100e meeting, it is agreed that conditional PSCell addition delay can use the existing requirement of conditional PSCell change as a baseline. I	n conditional PSCell change,

Tprocessing = 20 ms, when source and target cells are in the same FR

Tprocessing = 40 ms when source and target cells are in different FRs

At the same time, it was also agreed that CPAC related requirements are developed for

- ENDC: FR1+FR1, FR1+FR2

- NRDC: PCell on FR1+ PSCell FR2

Therefore, combining the above two agreements can infer that: For EN-DC, the target PSCell and source cell support in the same FR and in different FRs, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2; For NRDC, it only supports in the same FR, Tprocessing should be 40 ms.

Proposal 2: In conditional PSCell addition delay, for EN-DC, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2; for NR-DC Tprocessing = 40 ms.

TUE_preparation in conditional PSCell addition delay shall also be 10ms, the reason is similar to TUE_preparation in conditional PSCell change explained above.

Proposal 3: TUE_preparation in conditional PSCell addition delay shall be 10ms.

- Conclusions

Proposal 1: TUE_preparation in conditional PSCell change delay shall be 10ms.

Proposal 2: In conditional PSCell addition delay, for EN-DC, Tprocessing = 20 ms if NR PSCell is in FR1, Tprocessing = 40 ms if NR PSCell is in FR2; for NR-DC Tprocessing = 40 ms.

Proposal 3: TUE_preparation in conditional PSCell addition delay shall be 10ms.

References

[1] R4-2120334 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei