3GPP TSG-RAN4 WG4 Meeting #101-bis-e	R4-2200663

# Electronic meeting, January 17 – January 25, 2022

Agenda item:			6.21.2.4

Source:	vivo

Title:	Further discussion on impact to existing UE positioning and RRM requirements

Document for:	Discussion

- Introduction

The revised WID for NR positioning enhancement [1] was approved in the RAN#91-e meeting.  One of the objectives is to discuss the impact on the existing RAN4 requirements for positioning.

- Discuss and specify new as well as the impact on the existing RAN4 requirements for positioning and other RRM measurements and corresponding procedures [RAN4]

In the RAN4#101-e meeting, SRS antenna port switching impact on UE Rx-Tx time difference was discussed. The WF on this part are captured in [2] as follows.

SRS antenna port switching on UE Rx-Tx time difference - FFS: whether Rel-15 SRS (SRS-Resource) can be used for UE Rx-Tx time difference - FFS: impact of SRS antenna switching on UE behavior and UE Rx-Tx time difference accuracy

In this contribution, we further provide our views on impact of SRS antenna port switching to UE Rx-Tx measurement.

- Discussion

In the last meeting, the question whether Rel-15 SRS (SRS-Resource) can be used for UE Rx-Tx time difference was raised. As we mentioned in the RAN4#101-e meeting, the conclusion of RAN1 from [3] in the RAN1#99-e meeting was reviewed. The agreement on this part was duplicated as below:

Agreement: Support reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements for positioning in NR. - Note: There is no impact to specifications managed by RAN1 - Note: There is no impact to specifications managed by RAN4 for UE requirements - Note: No new UE behaviour is expected

It can be seen Rel-15 SRS resource set is only used for NR UL RTOA, AOA and gNB RSRP measurement for positioning, excluding UE measurement. And the highlighter part also made it clear that there is no impact to specifications managed by RAN4 for UE requirements when Rel-15 SRS-Resource is reused. Therefore, we propose that Rel-15 SRS (SRS-Resource) cannot be used for UE Rx-Tx time difference measurement. Based on this conclusion, SRS antenna port switching has no impact on UE behaviour and UE Rx-Tx time difference accuracy.

Proposal 1: Rel-15 SRS (SRS-Resource) cannot be used for UE Rx-Tx time difference measurement.

Proposal 2: SRS antenna port switching has no impact on UE behaviour and UE Rx-Tx time difference accuracy.

- Summary

Proposal 1: Rel-15 SRS (SRS-Resource) cannot be used for UE Rx-Tx time difference measurement.

Proposal 2: SRS antenna port switching has no impact on UE behaviour and UE Rx-Tx time difference accuracy.

- References

- RP-210903 Revised WID on NR Positioning Enhancements, Intel Corporation, CATT

- R4-2120419 WF on NR Positioning Enhancements (Part 1)

- R4-2110151 Final Report of 3GPP TSG RAN WG1 #99 v1.0.0