3GPP TSG-RAN4 WG4 Meeting #101bis-e	R4-2200664

# Electronic meeting, January 17-25, 2022

Source: 	vivo

Title: 	Simulation results for PRS measurement with reduced sample

Agenda Item:	6.21.2.2

Document for:	Information

Introduction

In this paper, we further provide simulation results for PRS measurement with reduced sample based on the simulation assumptions in [1].

Simulation results

UE Rx-Tx time difference

Table 1 and Table 2 are the measurement accuracy of UE Rx-Tx time difference under AWGN channel, TDL-A/C and TDL-D channel respectively. Es/Iot is -3dB or -6dB, and the value of samples number is 1, 2, or 4.

Table 1. Simulation results for AWGN channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = -3dB |  |  |  |  | Accuracy [90%] (Tc), SINR = -6dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sample =1 | Sample =2 | Sample =4 | Difference between sample=1 and sample=4 | Difference between sample=2 and sample=4 | Sample =1 | Sample =2 | Sample =4 | Difference between sample=1 and sample=4 | Difference between sample=2 and sample=4 |
| FR1 | 24 | 15 | 4 | 194.7 | 154.6 | 100.0 | 94.7 | 54.6 | 197.0 | 159.3 | 95.7 | 101.3 | 63.6 |
|  |  |  | 1 | 193.8 | 163.1 | 118.1 | 75.7 | 45.0 | 196.2 | 162.5 | 128.9 | 67.2 | 33.6 |
|  | 52 | 15 | 4 | 90.6 | 67.8 | 38.6 | 52.0 | 29.2 | 92.5 | 70.9 | 44.8 | 47.6 | 26.0 |
|  |  |  | 1 | 92.0 | 70.2 | 46.9 | 45.0 | 23.2 | 90.0 | 75.4 | 44.2 | 45.8 | 31.2 |
|  | 104 | 15 | 4 | 45.0 | 33.2 | 18.0 | 27.0 | 15.2 | 45.1 | 32.3 | 19.3 | 25.8 | 13.0 |
|  |  |  | 1 | 45.5 | 36.7 | 20.9 | 24.6 | 15.7 | 45.5 | 33.3 | 21.5 | 24.0 | 11.8 |
|  | 24 | 30 | 4 | 98.7 | 76.5 | 47.0 | 51.8 | 29.5 | 97.4 | 73.3 | 49.3 | 48.1 | 24.0 |
|  |  |  | 1 | 97.3 | 79.8 | 56.3 | 40.9 | 23.5 | 95.6 | 73.6 | 55.2 | 40.4 | 18.3 |
|  | 48 | 30 | 4 | 49.1 | 38.1 | 25.3 | 23.8 | 12.7 | 47.7 | 37.7 | 23.4 | 24.3 | 14.3 |
|  |  |  | 1 | 49.1 | 39.8 | 26.9 | 22.2 | 12.9 | 47.3 | 37.2 | 26.2 | 21.1 | 11.0 |
|  | 132 | 30 | 4 | 18.8 | 12.9 | 7.0 | 11.9 | 6.0 | 19.4 | 13.5 | 7.4 | 12.0 | 6.1 |
|  |  |  | 1 | 19.3 | 13.1 | 8.6 | 10.6 | 4.4 | 19.3 | 14.3 | 7.8 | 11.5 | 6.5 |
|  | 24 | 60 | 4 | 49.0 | 38.8 | 24.5 | 24.5 | 14.3 | 49.1 | 39.2 | 23.7 | 25.3 | 15.5 |
|  |  |  | 1 | 48.8 | 40.2 | 28.6 | 20.2 | 11.6 | 49.4 | 39.4 | 32.9 | 16.4 | 6.5 |
|  | 64 | 60 | 4 | 19.7 | 13.8 | 8.3 | 11.4 | 5.5 | 19.6 | 13.7 | 7.7 | 11.9 | 6.0 |
|  |  |  | 1 | 19.9 | 15.2 | 10.6 | 9.3 | 4.6 | 19.7 | 14.0 | 9.3 | 10.5 | 4.8 |
|  | 132 | 60 | 4 | 9.7 | 6.7 | 4.0 | 5.8 | 2.7 | 9.5 | 5.9 | 3.9 | 5.7 | 2.1 |
|  |  |  | 1 | 9.8 | 6.5 | 4.0 | 5.8 | 2.5 | 9.7 | 7.0 | 4.5 | 5.1 | 2.4 |
| FR2 | 24 | 60 | 4 | 48.8 | 38.0 | 27.6 | 21.2 | 10.4 | 48.5 | 38.0 | 27.2 | 21.3 | 10.8 |
|  |  |  | 1 | 49.3 | 41.5 | 31.7 | 17.6 | 9.9 | 49.3 | 41.9 | 31.6 | 17.7 | 10.3 |
|  | 64 | 60 | 4 | 19.5 | 14.7 | 8.7 | 10.8 | 6.0 | 19.6 | 14.6 | 10.0 | 9.6 | 4.5 |
|  |  |  | 1 | 19.6 | 14.1 | 10.3 | 9.3 | 3.9 | 19.9 | 15.3 | 9.1 | 10.8 | 6.2 |
|  | 132 | 60 | 4 | 9.7 | 6.9 | 3.8 | 6.0 | 3.1 | 9.6 | 7.0 | 3.9 | 5.8 | 3.2 |
|  |  |  | 1 | 9.5 | 6.5 | 4.3 | 5.2 | 2.2 | 9.7 | 7.0 | 4.4 | 5.3 | 2.5 |
|  | 32 | 120 | 4 | 19.8 | 14.8 | 9.4 | 10.4 | 5.4 | 20.1 | 14.9 | 9.2 | 10.9 | 5.7 |
|  |  |  | 1 | 19.9 | 15.3 | 11.2 | 8.7 | 4.1 | 19.1 | 14.3 | 11.0 | 8.2 | 3.3 |
|  | 64 | 120 | 4 | 9.9 | 7.0 | 4.0 | 5.9 | 3.0 | 9.9 | 7.7 | 4.3 | 5.6 | 3.4 |
|  |  |  | 1 | 9.8 | 7.2 | 4.4 | 5.4 | 2.9 | 10.0 | 7.1 | 5.0 | 4.9 | 2.0 |
|  | 128 | 120 | 4 | 4.9 | 3.6 | 1.9 | 3.1 | 1.8 | 4.9 | 3.5 | 1.9 | 3.0 | 1.6 |
|  |  |  | 1 | 4.9 | 3.3 | 2.0 | 2.9 | 1.3 | 4.8 | 3.2 | 2.1 | 2.7 | 1.1 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2. Simulation results for Fading channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = -3dB |  |  |  |  | Accuracy [90%] (Tc), SINR = -6dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sample =1 | Sample =2 | Sample =4 | Difference between sample=1 and sample=4 | Difference between sample=2 and sample=4 | Sample =1 | Sample =2 | Sample =4 | Difference between sample=1 and sample=4 | Difference between sample=2 and sample=4 |
| FR1 TDL-A | 24 | 15 | 4 | 214.1 | 196.9 | 183.8 | 30.3 | 13.1 | 223.1 | 185.6 | 171.0 | 52.1 | 14.6 |
|  |  |  | 1 | 260.7 | 198.9 | 182.1 | 78.6 | 16.8 | 630.3 | 201.6 | 188.6 | 441.7 | 13.0 |
|  | 52 | 15 | 4 | 105.0 | 93.8 | 91.2 | 13.7 | 2.6 | 98.6 | 91.8 | 82.8 | 15.8 | 8.9 |
|  |  |  | 1 | 104.3 | 92.9 | 87.4 | 16.9 | 5.5 | 104.9 | 89.8 | 78.5 | 26.4 | 11.3 |
|  | 104 | 15 | 4 | 93.4 | 90.5 | 66.5 | 26.9 | 24.0 | 100.3 | 84.4 | 62.5 | 37.8 | 21.8 |
|  |  |  | 1 | 91.2 | 74.9 | 61.3 | 29.8 | 13.6 | 95.4 | 83.8 | 65.2 | 30.2 | 18.6 |
|  | 24 | 30 | 4 | 109.9 | 98.2 | 91.0 | 18.8 | 7.2 | 110.9 | 100.3 | 92.4 | 18.5 | 7.9 |
|  |  |  | 1 | 124.9 | 97.6 | 89.4 | 35.5 | 8.2 | 137.5 | 96.4 | 89.6 | 47.9 | 6.8 |
|  | 48 | 30 | 4 | 91.6 | 79.2 | 59.9 | 31.7 | 19.3 | 86.6 | 85.1 | 70.6 | 16.1 | 14.6 |
|  |  |  | 1 | 89.7 | 77.2 | 75.7 | 14.0 | 1.4 | 88.9 | 82.7 | 67.2 | 21.6 | 15.4 |
|  | 132 | 30 | 4 | 79.6 | 42.0 | 41.9 | 37.6 | 0.1 | 45.3 | 42.4 | 39.7 | 5.6 | 2.7 |
|  |  |  | 1 | 84.1 | 42.2 | 41.3 | 42.8 | 0.8 | 50.6 | 42.3 | 42.2 | 8.4 | 0.1 |
|  | 24 | 60 | 4 | 92.4 | 80.2 | 72.2 | 20.3 | 8.1 | 93.8 | 85.2 | 71.3 | 22.5 | 13.9 |
|  |  |  | 1 | 97.6 | 79.9 | 69.8 | 27.7 | 10.1 | 90.0 | 82.5 | 69.2 | 20.7 | 13.3 |
|  | 64 | 60 | 4 | 84.5 | 42.5 | 42.4 | 42.1 | 0.1 | 51.2 | 41.9 | 41.4 | 9.8 | 0.5 |
|  |  |  | 1 | 79.1 | 41.5 | 41.4 | 37.7 | 0.1 | 55.8 | 42.3 | 41.9 | 13.9 | 0.5 |
|  | 132 | 60 | 4 | 37.6 | 34.9 | 31.4 | 6.2 | 3.4 | 37.3 | 36.3 | 36.2 | 1.1 | 0.1 |
|  |  |  | 1 | 35.7 | 32.7 | 31.0 | 4.7 | 1.7 | 37.8 | 35.7 | 35.1 | 2.7 | 0.6 |
| FR2 TDL-C | 24 | 60 | 4 | 91.5 | 90.0 | 80.6 | 11.0 | 9.4 | 92.3 | 88.1 | 83.5 | 8.8 | 4.6 |
|  |  |  | 1 | 88.7 | 84.8 | 84.2 | 4.5 | 0.6 | 91.5 | 88.4 | 87.2 | 4.3 | 1.2 |
|  | 64 | 60 | 4 | 74.3 | 73.6 | 71.1 | 3.3 | 2.5 | 74.6 | 73.6 | 71.0 | 3.5 | 2.5 |
|  |  |  | 1 | 75.2 | 77.2 | 73.9 | 1.3 | 3.3 | 75.1 | 73.6 | 72.8 | 2.2 | 0.7 |
|  | 132 | 60 | 4 | 84.9 | 85.0 | 85.3 | -0.4 | -0.3 | 85.1 | 85.1 | 85.1 | 0.1 | 0.0 |
|  |  |  | 1 | 84.2 | 84.2 | 84.2 | 0.0 | 0.1 | 84.7 | 84.7 | 84.7 | 0.0 | 0.0 |
|  | 32 | 120 | 4 | 74.5 | 74.3 | 73.6 | 0.9 | 0.7 | 73.0 | 70.2 | 70.0 | 3.0 | 0.1 |
|  |  |  | 1 | 75.6 | 73.7 | 73.3 | 2.2 | 0.3 | 76.0 | 74.7 | 75.5 | 0.4 | -0.8 |
|  | 64 | 120 | 4 | 83.5 | 84.2 | 84.5 | -1.1 | -0.3 | 84.7 | 84.8 | 84.9 | -0.2 | -0.1 |
|  |  |  | 1 | 83.8 | 84.1 | 83.6 | 0.2 | 0.5 | 84.5 | 83.9 | 83.9 | 0.7 | 0.0 |
|  | 128 | 120 | 4 | 78.8 | 79.2 | 79.5 | -0.8 | -0.3 | 78.8 | 79.4 | 79.5 | -0.8 | -0.2 |
|  |  |  | 1 | 79.2 | 79.3 | 79.2 | 0.0 | 0.1 | 78.8 | 78.0 | 78.5 | 0.3 | -0.5 |
| FR1 TDL-D | 24 | 15 | 4 | 202.1 | 183.9 | 180.7 | 21.4 | 3.3 | 210.0 | 189.6 | 176.5 | 33.5 | 13.1 |
|  |  |  | 1 | 221.3 | 192.3 | 183.2 | 38.1 | 9.1 | 2615.2 | 201.1 | 192.9 | 2422.3 | 8.2 |
|  | 52 | 15 | 4 | 96.8 | 88.5 | 88.1 | 8.7 | 0.4 | 97.8 | 91.7 | 85.7 | 12.1 | 6.0 |
|  |  |  | 1 | 100.9 | 93.4 | 88.8 | 12.0 | 4.6 | 107.3 | 91.0 | 85.4 | 21.9 | 5.6 |
|  | 104 | 15 | 4 | 46.8 | 44.2 | 43.2 | 3.6 | 1.0 | 47.2 | 42.1 | 40.8 | 6.4 | 1.4 |
|  |  |  | 1 | 47.8 | 45.0 | 43.1 | 4.7 | 1.9 | 48.7 | 44.8 | 42.5 | 6.2 | 2.3 |
|  | 24 | 30 | 4 | 109.8 | 96.5 | 93.3 | 16.5 | 3.2 | 109.2 | 96.2 | 93.1 | 16.1 | 3.1 |
|  |  |  | 1 | 125.0 | 101.5 | 95.7 | 29.3 | 5.8 | 1112.6 | 98.6 | 97.4 | 1015.2 | 1.2 |
|  | 48 | 30 | 4 | 50.3 | 45.9 | 42.9 | 7.4 | 3.0 | 50.4 | 46.7 | 44.9 | 5.5 | 1.8 |
|  |  |  | 1 | 54.7 | 48.2 | 47.6 | 7.1 | 0.6 | 56.4 | 48.1 | 45.7 | 10.7 | 2.4 |
|  | 132 | 30 | 4 | 19.1 | 18.4 | 16.9 | 2.2 | 1.5 | 19.5 | 18.5 | 16.6 | 2.9 | 2.0 |
|  |  |  | 1 | 19.5 | 18.4 | 15.8 | 3.7 | 2.6 | 20.3 | 18.8 | 16.8 | 3.5 | 2.0 |
|  | 24 | 60 | 4 | 50.6 | 46.9 | 44.8 | 5.8 | 2.1 | 55.1 | 47.8 | 46.6 | 8.6 | 1.2 |
|  |  |  | 1 | 61.5 | 50.2 | 47.4 | 14.1 | 2.8 | 973.2 | 48.1 | 44.6 | 928.6 | 3.5 |
|  | 64 | 60 | 4 | 20.1 | 19.5 | 18.9 | 1.2 | 0.6 | 20.0 | 19.4 | 18.8 | 1.2 | 0.7 |
|  |  |  | 1 | 20.2 | 18.9 | 18.6 | 1.6 | 0.3 | 21.8 | 19.5 | 18.0 | 3.8 | 1.6 |
|  | 132 | 60 | 4 | 9.5 | 9.1 | 9.0 | 0.4 | 0.0 | 9.7 | 9.4 | 9.1 | 0.6 | 0.2 |
|  |  |  | 1 | 9.7 | 9.4 | 9.0 | 0.8 | 0.4 | 10.2 | 9.5 | 8.4 | 1.8 | 1.1 |
| FR2 TDL-D | 24 | 60 | 4 | 50.5 | 48.3 | 46.5 | 4.0 | 1.8 | 53.0 | 49.8 | 45.6 | 7.3 | 4.2 |
|  |  |  | 1 | 55.7 | 51.0 | 50.9 | 4.8 | 0.1 | 74.2 | 50.5 | 45.5 | 28.8 | 5.0 |
|  | 64 | 60 | 4 | 19.7 | 19.0 | 17.9 | 1.8 | 1.1 | 19.9 | 19.2 | 18.3 | 1.6 | 0.9 |
|  |  |  | 1 | 20.2 | 18.9 | 18.4 | 1.8 | 0.4 | 21.2 | 19.4 | 18.4 | 2.8 | 1.0 |
|  | 132 | 60 | 4 | 9.7 | 9.4 | 9.1 | 0.7 | 0.3 | 9.5 | 9.0 | 8.8 | 0.7 | 0.2 |
|  |  |  | 1 | 9.7 | 9.1 | 8.1 | 1.6 | 1.0 | 10.0 | 9.5 | 9.5 | 0.5 | 0.0 |
|  | 32 | 120 | 4 | 19.8 | 18.8 | 17.7 | 2.1 | 1.1 | 19.9 | 19.2 | 18.3 | 1.6 | 0.9 |
|  |  |  | 1 | 21.3 | 20.0 | 18.0 | 3.3 | 2.0 | 25.7 | 19.1 | 17.8 | 7.9 | 1.3 |
|  | 64 | 120 | 4 | 9.9 | 9.8 | 9.1 | 0.7 | 0.6 | 10.0 | 9.3 | 9.2 | 0.8 | 0.2 |
|  |  |  | 1 | 10.2 | 9.7 | 9.4 | 0.8 | 0.3 | 10.6 | 9.6 | 9.5 | 1.1 | 0.1 |
|  | 128 | 120 | 4 | 4.9 | 4.8 | 4.7 | 0.2 | 0.0 | 5.0 | 4.8 | 4.6 | 0.4 | 0.1 |
|  |  |  | 1 | 4.9 | 4.7 | 4.4 | 0.5 | 0.2 | 5.1 | 4.8 | 4.8 | 0.3 | 0.1 |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |  |  |  |  |  |  |  |  |

In order to clearly observe the impact of the number of samples on the UE Rx-Tx time difference measurement accuracy, two columns are added to show the difference between sample=1 and sample=4/sample=2 and sample=4. It can be seen from the data collated in Table 1: for AWGN channel, the difference of UE Rx-Tx time difference is within 30Tc for most cases. From the data collated in Table 2, it can be seen that for TDL-A and TDL-C channel, the difference between sample=1 and sample=4/sample=2 and sample=4 can also be guaranteed within 30Tc except small RBs (e.g., 24RBs). For TDL-D channel, whether in FR1 or FR2, the UE Rx-Tx time difference measurement accuracy is similar to that of AWGN channel.

Observation 1: For AWGN channel, the difference of UE Rx-Tx time difference between 1 sample or 2 samples and 4 samples can be guaranteed within 30Tc for most cases when the side condition is -3dB or -6dB.

Observation 2: For TDL-A and TDL-C channel, the difference of UE Rx-Tx time difference between 1 sample or 2 samples and 4 samples can be guaranteed within 30Tc except some small PRS RBs (e.g., 24RBs) when the side condition is -3dB or -6dB.

Observation 3: For TDL-D channel, whether in FR1 or FR2, the UE Rx-Tx time difference measurement accuracy is similar to that of AWGN channel.

PRS RSTD

Simulation results for PRS RSTD measurement accuracy are shown in Table 3 and 4 for AGWN, TDL-A/C and TDL-D channels. Es/Iot is -3dB or -6dB, and the value of samples number is 1, 2, or 4.

Table 3. Simulation results for AWGN channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = [-3, -6] dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sample =1 | Sample =2 | Sample =4 | Difference between sample=1 and sample=4 | Difference between sample=2 and sample=4 |
| FR1 | 24 | 15 | 4 | 176.1 | 188.3 | 197.1 | -20.9 | -8.7 |
|  |  |  | 1 | 184.4 | 182.1 | 182.3 | 2.1 | -0.3 |
|  | 52 | 15 | 4 | 91.2 | 89.0 | 91.0 | 0.2 | -1.9 |
|  |  |  | 1 | 90.2 | 98.8 | 97.2 | -7.0 | 1.6 |
|  | 104 | 15 | 4 | 44.4 | 46.2 | 47.1 | -2.7 | -0.9 |
|  |  |  | 1 | 44.4 | 46.5 | 47.9 | -3.4 | -1.4 |
|  | 24 | 30 | 4 | 92.8 | 96.2 | 103.7 | -10.9 | -7.5 |
|  |  |  | 1 | 87.0 | 93.6 | 91.6 | -4.6 | 2.0 |
|  | 48 | 30 | 4 | 44.9 | 50.1 | 51.5 | -6.6 | -1.3 |
|  |  |  | 1 | 47.0 | 50.0 | 49.3 | -2.3 | 0.7 |
|  | 132 | 30 | 4 | 22.4 | 22.0 | 21.5 | 0.8 | 0.5 |
|  |  |  | 1 | 22.3 | 22.8 | 20.8 | 1.4 | 1.9 |
|  | 24 | 60 | 4 | 43.1 | 46.2 | 48.8 | -5.7 | -2.6 |
|  |  |  | 1 | 45.7 | 46.8 | 48.3 | -2.6 | -1.5 |
|  | 64 | 60 | 4 | 22.6 | 23.2 | 22.7 | -0.2 | 0.4 |
|  |  |  | 1 | 21.8 | 22.8 | 22.6 | -0.8 | 0.2 |
|  | 132 | 60 | 4 | 11.6 | 11.9 | 11.4 | 0.2 | 0.5 |
|  |  |  | 1 | 10.9 | 11.1 | 10.8 | 0.0 | 0.3 |
| FR2 | 24 | 60 | 4 | 48.8 | 38.0 | 27.6 | 21.2 | 10.4 |
|  |  |  | 1 | 49.3 | 41.5 | 31.7 | 17.6 | 9.9 |
|  | 64 | 60 | 4 | 19.5 | 14.7 | 8.7 | 10.8 | 6.0 |
|  |  |  | 1 | 19.6 | 14.1 | 10.3 | 9.3 | 3.9 |
|  | 132 | 60 | 4 | 9.7 | 6.9 | 3.8 | 6.0 | 3.1 |
|  |  |  | 1 | 9.5 | 6.5 | 4.3 | 5.2 | 2.2 |
|  | 32 | 120 | 4 | 19.8 | 14.8 | 9.4 | 10.4 | 5.4 |
|  |  |  | 1 | 19.9 | 15.3 | 11.2 | 8.7 | 4.1 |
|  | 64 | 120 | 4 | 9.9 | 7.0 | 4.0 | 5.9 | 3.0 |
|  |  |  | 1 | 9.8 | 7.2 | 4.4 | 5.4 | 2.9 |
|  | 128 | 120 | 4 | 4.9 | 3.6 | 1.9 | 3.1 | 1.8 |
|  |  |  | 1 | 4.9 | 3.3 | 2.0 | 2.9 | 1.3 |

Table 4. Simulation results for Fading channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = [-3, -6] dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sample =1 | Sample =2 | Sample =4 | Difference between sample=1 and sample=4 | Difference between sample=2 and sample=4 |
| FR1 TDL-A | 24 | 15 | 4 | 265.0 | 246.4 | 230.4 | 34.6 | 16.0 |
|  |  |  | 1 | 637.4 | 398.7 | 879.8 | -242.4 | -481.0 |
|  | 52 | 15 | 4 | 147.9 | 142.9 | 141.7 | 6.2 | 1.2 |
|  |  |  | 1 | 172.1 | 169.6 | 167.1 | 5.0 | 2.5 |
|  | 104 | 15 | 4 | 103.5 | 104.9 | 115.1 | -11.6 | -10.2 |
|  |  |  | 1 | 107.3 | 114.7 | 116.1 | -8.9 | -1.4 |
|  | 24 | 30 | 4 | 149.3 | 141.0 | 140.3 | 9.0 | 0.7 |
|  |  |  | 1 | 236.2 | 200.0 | 336.6 | -100.5 | -136.7 |
|  | 48 | 30 | 4 | 106.5 | 106.4 | 105.1 | 1.3 | 1.3 |
|  |  |  | 1 | 112.0 | 108.8 | 104.1 | 7.9 | 4.7 |
|  | 132 | 30 | 4 | 49.3 | 58.2 | 72.4 | -23.1 | -14.2 |
|  |  |  | 1 | 56.0 | 61.7 | 78.0 | -22.0 | -16.3 |
|  | 24 | 60 | 4 | 113.0 | 103.4 | 102.2 | 10.8 | 1.2 |
|  |  |  | 1 | 143.1 | 123.0 | 290.1 | -147.0 | -167.1 |
|  | 64 | 60 | 4 | 57.1 | 60.0 | 64.6 | -7.5 | -4.6 |
|  |  |  | 1 | 62.2 | 61.0 | 68.4 | -6.2 | -7.4 |
|  | 132 | 60 | 4 | 25.0 | 24.9 | 25.4 | -0.4 | -0.5 |
|  |  |  | 1 | 24.8 | 23.5 | 23.3 | 1.5 | 0.3 |
| FR2 TDL-C | 24 | 60 | 4 | 108.3 | 93.9 | 80.7 | 27.7 | 13.2 |
|  |  |  | 1 | 126.2 | 124.7 | 108.7 | 17.5 | 16.0 |
|  | 64 | 60 | 4 | 60.0 | 52.2 | 44.3 | 15.7 | 7.9 |
|  |  |  | 1 | 58.8 | 53.0 | 45.1 | 13.7 | 7.9 |
|  | 132 | 60 | 4 | 77.4 | 60.3 | 56.8 | 20.6 | 3.4 |
|  |  |  | 1 | 74.1 | 60.7 | 55.4 | 18.7 | 5.2 |
|  | 32 | 120 | 4 | 62.3 | 52.4 | 48.4 | 13.9 | 4.0 |
|  |  |  | 1 | 68.8 | 58.1 | 53.7 | 15.1 | 4.4 |
|  | 64 | 120 | 4 | 76.9 | 60.6 | 57.5 | 19.4 | 3.1 |
|  |  |  | 1 | 77.1 | 59.9 | 55.8 | 21.3 | 4.2 |
|  | 128 | 120 | 4 | 72.1 | 59.4 | 54.0 | 18.0 | 5.3 |
|  |  |  | 1 | 71.6 | 61.0 | 53.0 | 18.6 | 8.0 |
| FR1 TDL-D | 24 | 15 | 4 | 206.7 | 206.4 | 221.8 | -15.1 | -15.4 |
|  |  |  | 1 | 2681.6 | 439.0 | 1750.0 | 931.6 | -1310.9 |
|  | 52 | 15 | 4 | 103.6 | 102.9 | 101.9 | 1.7 | 1.0 |
|  |  |  | 1 | 131.1 | 111.1 | 123.6 | 7.5 | -12.5 |
|  | 104 | 15 | 4 | 50.4 | 45.4 | 48.7 | 1.7 | -3.3 |
|  |  |  | 1 | 56.3 | 50.8 | 52.8 | 3.4 | -2.1 |
|  | 24 | 30 | 4 | 113.8 | 108.8 | 116.7 | -2.9 | -7.9 |
|  |  |  | 1 | 1204.1 | 626.5 | 1582.2 | -378.1 | -955.7 |
|  | 48 | 30 | 4 | 51.1 | 48.4 | 50.4 | 0.7 | -2.0 |
|  |  |  | 1 | 69.0 | 56.6 | 68.0 | 1.0 | -11.3 |
|  | 132 | 30 | 4 | 25.1 | 24.9 | 24.4 | 0.7 | 0.5 |
|  |  |  | 1 | 25.1 | 24.9 | 25.3 | -0.2 | -0.4 |
|  | 24 | 60 | 4 | 59.0 | 49.5 | 51.5 | 7.5 | -2.0 |
|  |  |  | 1 | 968.5 | 104.2 | 798.0 | 170.6 | -693.8 |
|  | 64 | 60 | 4 | 25.0 | 24.7 | 23.3 | 1.6 | 1.3 |
|  |  |  | 1 | 30.2 | 23.2 | 24.8 | 5.4 | -1.7 |
|  | 132 | 60 | 4 | 11.9 | 11.0 | 11.7 | 0.1 | -0.7 |
|  |  |  | 1 | 12.9 | 11.9 | 12.3 | 0.6 | -0.4 |
| FR2 TDL-D | 24 | 60 | 4 | 56.5 | 49.5 | 42.7 | 13.9 | 6.8 |
|  |  |  | 1 | 389.8 | 560.5 | 538.5 | -148.7 | 22.0 |
|  | 64 | 60 | 4 | 25.2 | 20.6 | 19.7 | 5.5 | 0.9 |
|  |  |  | 1 | 24.3 | 21.1 | 20.9 | 3.4 | 0.1 |
|  | 132 | 60 | 4 | 13.1 | 11.8 | 10.5 | 2.6 | 1.4 |
|  |  |  | 1 | 12.5 | 11.9 | 9.2 | 3.3 | 2.6 |
|  | 32 | 120 | 4 | 27.0 | 22.5 | 22.0 | 5.0 | 0.5 |
|  |  |  | 1 | 33.1 | 100.4 | 172.4 | -139.3 | -72.0 |
|  | 64 | 120 | 4 | 13.0 | 12.4 | 11.3 | 1.6 | 1.0 |
|  |  |  | 1 | 13.4 | 12.1 | 13.8 | -0.4 | -1.7 |
|  | 128 | 120 | 4 | 6.1 | 5.9 | 5.8 | 0.2 | 0.0 |
|  |  |  | 1 | 6.4 | 5.8 | 5.8 | 0.6 | 0.0 |

Similar to the trend of UE Rx-Tx time difference results, for AWGN channel, except for the cases of 24RBs, the difference of PRS RSTD measurement accuracy between 1 sample or 2 samples and 4 samples is within 15Tc. From the data collated in Table 4, it can be seen that for TDL-A channel and TDL-C channel, except for the cases of 24RBs, the difference of PRS RSTD measurement accuracy between 1 sample or 2 samples and 4 samples is within 30Tc. For TDL-D channel, whether in FR1 or FR2, RSTD measurement accuracy is slightly worse than the result for AWGN channel under most cases.

Observation 4: For AWGN channel, the PRS RSTD between 1 sample or 2 samples and 4 samples can be guaranteed within 15Tc except the case of 24RBs when the side condition is (-3, -6) dB.

Observation 5: For TDL-A channel and TDL-C channel, the PRS RSTD between 1 sample or 2 samples and 4 samples can be guaranteed within 30Tc except the case of 24RBs when the side condition is (-3, -6) dB.

Observation 6 For TDL-D channel, whether in FR1 or FR2, RSTD measurement accuracy is slightly worse than that of AWGN channel under the most cases.

PRS RSRP

Simulation results for PRS RSRP measurement accuracy are shown in Table 5 and 6 for AGWN, TDL-A/C and TDL-D channels. Es/Iot is -3dB or -6dB, and the value of samples number is 1, 2, or 4.

Table 5. Simulation results for AWGN channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = -3 dB |  |  | Accuracy [90%] (Tc), SINR = -6 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sample =1 | Sample =2 | Sample =4 | Sample =1 | Sample =2 | Sample =4 |
| FR1 | 24 | 15 | 4 | 0.4 | 0.3 | 0.2 | 0.6 | 0.4 | 0.4 |
|  |  |  | 1 | 1.1 | 0.8 | 0.7 | 1.3 | 1.0 | 0.8 |
|  | 52 | 15 | 4 | 0.5 | 0.4 | 0.4 | 0.4 | 0.3 | 0.3 |
|  |  |  | 1 | 0.8 | 0.6 | 0.5 | 0.9 | 0.7 | 0.6 |
|  | 104 | 15 | 4 | 0.3 | 0.3 | 0.2 | 0.3 | 0.2 | 0.2 |
|  |  |  | 1 | 0.6 | 0.4 | 0.3 | 0.6 | 0.5 | 0.4 |
|  | 24 | 30 | 4 | 0.6 | 0.5 | 0.5 | 0.6 | 0.4 | 0.3 |
|  |  |  | 1 | 1.1 | 0.8 | 0.6 | 1.2 | 1.0 | 0.7 |
|  | 48 | 30 | 4 | 0.4 | 0.3 | 0.3 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.8 | 0.6 | 0.4 | 1.0 | 0.8 | 0.6 |
|  | 132 | 30 | 4 | 0.2 | 0.2 | 0.2 | 0.3 | 0.2 | 0.1 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.6 | 0.5 | 0.3 |
|  | 24 | 60 | 4 | 0.5 | 0.4 | 0.3 | 0.5 | 0.4 | 0.3 |
|  |  |  | 1 | 1.1 | 0.8 | 0.7 | 1.3 | 1.0 | 0.8 |
|  | 64 | 60 | 4 | 0.3 | 0.2 | 0.2 | 0.4 | 0.3 | 0.3 |
|  |  |  | 1 | 0.7 | 0.5 | 0.4 | 0.8 | 0.6 | 0.4 |
|  | 132 | 60 | 4 | 0.2 | 0.1 | 0.1 | 0.3 | 0.2 | 0.2 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.7 | 0.6 | 0.5 |
| FR2 | 24 | 60 | 4 | 0.5 | 0.5 | 0.3 | 0.5 | 0.5 | 0.3 |
|  |  |  | 1 | 1.0 | 1.0 | 0.7 | 1.3 | 1.3 | 0.8 |
|  | 64 | 60 | 4 | 0.3 | 0.3 | 0.2 | 0.4 | 0.4 | 0.3 |
|  |  |  | 1 | 0.7 | 0.7 | 0.4 | 0.8 | 0.8 | 0.4 |
|  | 132 | 60 | 4 | 0.2 | 0.2 | 0.1 | 0.3 | 0.3 | 0.2 |
|  |  |  | 1 | 0.5 | 0.5 | 0.3 | 0.7 | 0.7 | 0.4 |
|  | 32 | 120 | 4 | 0.5 | 0.5 | 0.3 | 0.5 | 0.5 | 0.3 |
|  |  |  | 1 | 0.9 | 0.9 | 0.6 | 1.4 | 1.4 | 0.9 |
|  | 64 | 120 | 4 | 0.4 | 0.4 | 0.3 | 0.4 | 0.4 | 0.3 |
|  |  |  | 1 | 0.7 | 0.7 | 0.5 | 1.0 | 1.0 | 0.7 |
|  | 128 | 120 | 4 | 0.2 | 0.2 | 0.1 | 0.4 | 0.4 | 0.3 |
|  |  |  | 1 | 0.5 | 0.5 | 0.3 | 0.8 | 0.8 | 0.6 |

Table 6. Simulation results for Fading channel

| FR | BW (RB) | SCS (kHz) | PRS_Lenth PerSlot | Accuracy [90%] (Tc), SINR = -3 dB |  |  | Accuracy [90%] (Tc), SINR = -6 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sample =1 | Sample =2 | Sample =4 | Sample =1 | Sample =2 | Sample =4 |
| FR1 TDL-A | 24 | 15 | 4 | 0.6 | 0.4 | 0.3 | 0.6 | 0.4 | 0.3 |
|  |  |  | 1 | 1.4 | 1.4 | 1.1 | 2.0 | 1.5 | 1.6 |
|  | 52 | 15 | 4 | 0.4 | 0.3 | 0.2 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.8 | 0.7 | 0.5 | 1.0 | 0.7 | 0.5 |
|  | 104 | 15 | 4 | 0.3 | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.6 | 0.5 | 0.3 |
|  | 24 | 30 | 4 | 0.6 | 0.4 | 0.3 | 0.6 | 0.5 | 0.3 |
|  |  |  | 1 | 1.2 | 1.0 | 0.7 | 1.7 | 1.4 | 1.4 |
|  | 48 | 30 | 4 | 0.3 | 0.3 | 0.2 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.8 | 0.6 | 0.4 | 0.9 | 0.6 | 0.5 |
|  | 132 | 30 | 4 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 | 0.1 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.6 | 0.4 | 0.3 |
|  | 24 | 60 | 4 | 0.6 | 0.4 | 0.3 | 0.6 | 0.5 | 0.3 |
|  |  |  | 1 | 1.1 | 0.8 | 0.6 | 1.4 | 0.9 | 0.8 |
|  | 64 | 60 | 4 | 0.3 | 0.2 | 0.2 | 0.3 | 0.3 | 0.2 |
|  |  |  | 1 | 0.7 | 0.6 | 0.4 | 0.8 | 0.6 | 0.5 |
|  | 132 | 60 | 4 | 0.2 | 0.1 | 0.1 | 0.2 | 0.2 | 0.1 |
|  |  |  | 1 | 0.5 | 0.3 | 0.2 | 0.5 | 0.4 | 0.3 |
| FR2 TDL-C | 24 | 60 | 4 | 0.5 | 0.4 | 0.3 | 0.6 | 0.4 | 0.3 |
|  |  |  | 1 | 1.2 | 0.8 | 0.6 | 1.4 | 1.1 | 0.8 |
|  | 64 | 60 | 4 | 0.3 | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 |
|  |  |  | 1 | 0.6 | 0.4 | 0.3 | 0.8 | 0.6 | 0.4 |
|  | 132 | 60 | 4 | 0.2 | 0.1 | 0.1 | 0.2 | 0.2 | 0.1 |
|  |  |  | 1 | 0.4 | 0.3 | 0.2 | 0.5 | 0.4 | 0.3 |
|  | 32 | 120 | 4 | 0.4 | 0.3 | 0.2 | 0.5 | 0.4 | 0.3 |
|  |  |  | 1 | 0.9 | 0.7 | 0.5 | 1.3 | 0.9 | 0.7 |
|  | 64 | 120 | 4 | 0.3 | 0.2 | 0.2 | 0.3 | 0.3 | 0.2 |
|  |  |  | 1 | 0.6 | 0.4 | 0.3 | 0.9 | 0.6 | 0.4 |
|  | 128 | 120 | 4 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 | 0.1 |
|  |  |  | 1 | 0.4 | 0.3 | 0.2 | 0.6 | 0.4 | 0.4 |
| FR1 TDL-D | 24 | 15 | 4 | 0.6 | 0.5 | 0.4 | 0.7 | 0.5 | 0.4 |
|  |  |  | 1 | 1.4 | 1.2 | 0.9 | 1.9 | 1.5 | 1.5 |
|  | 52 | 15 | 4 | 0.4 | 0.3 | 0.2 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.9 | 0.7 | 0.6 | 1.2 | 0.9 | 0.8 |
|  | 104 | 15 | 4 | 0.3 | 0.2 | 0.2 | 0.3 | 0.3 | 0.2 |
|  |  |  | 1 | 0.6 | 0.5 | 0.3 | 0.8 | 0.6 | 0.6 |
|  | 24 | 30 | 4 | 0.6 | 0.4 | 0.4 | 0.7 | 0.5 | 0.4 |
|  |  |  | 1 | 1.3 | 1.0 | 0.7 | 2.0 | 1.7 | 1.2 |
|  | 48 | 30 | 4 | 0.3 | 0.3 | 0.2 | 0.5 | 0.4 | 0.3 |
|  |  |  | 1 | 0.9 | 0.9 | 0.7 | 1.3 | 1.0 | 1.1 |
|  | 132 | 30 | 4 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 | 0.1 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.6 | 0.5 | 0.4 |
|  | 24 | 60 | 4 | 0.6 | 0.5 | 0.3 | 0.7 | 0.6 | 0.4 |
|  |  |  | 1 | 1.6 | 1.4 | 1.1 | 2.2 | 1.9 | 1.9 |
|  | 64 | 60 | 4 | 0.3 | 0.3 | 0.2 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.8 | 0.6 | 0.4 | 1.0 | 0.7 | 0.6 |
|  | 132 | 60 | 4 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.7 | 0.6 | 0.4 |
| FR2 TDL-D | 24 | 60 | 4 | 0.6 | 0.4 | 0.3 | 0.8 | 0.7 | 0.5 |
|  |  |  | 1 | 1.4 | 1.1 | 0.9 | 1.9 | 1.8 | 1.6 |
|  | 64 | 60 | 4 | 0.3 | 0.2 | 0.2 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.7 | 0.6 | 0.4 | 0.9 | 0.8 | 0.8 |
|  | 132 | 60 | 4 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.7 | 0.5 | 0.4 |
|  | 32 | 120 | 4 | 0.4 | 0.3 | 0.3 | 0.6 | 0.4 | 0.3 |
|  |  |  | 1 | 1.2 | 1.0 | 0.9 | 2.1 | 1.8 | 1.8 |
|  | 64 | 120 | 4 | 0.3 | 0.3 | 0.2 | 0.4 | 0.3 | 0.3 |
|  |  |  | 1 | 0.7 | 0.5 | 0.4 | 0.9 | 0.7 | 0.7 |
|  | 128 | 120 | 4 | 0.2 | 0.2 | 0.1 | 0.4 | 0.3 | 0.2 |
|  |  |  | 1 | 0.5 | 0.4 | 0.3 | 0.7 | 0.5 | 0.5 |

It can be seen from the data collated in Table 5 and Table 6: the PRS-RSRP measurement accuracy is within 2dB no matter what the side conditions, channel environment and the number of samples.

Observation 7: For AWGN channel, the PRS-RSRP measurement accuracy is not sensitive to side condition and sample numbers in all cases.

Observation 8: For Fading channel, the PRS-RSRP measurement accuracy of 1 sample is slightly worse than that of 2 samples and 4 samples in most cases.

Conclusion

Observation 1: For AWGN channel, the difference of UE Rx-Tx time difference between 1 sample or 2 samples and 4 samples can be guaranteed within 30Tc for most cases when the side condition is -3dB or -6dB.

Observation 2: For TDL-A and TDL-C channel, the difference of UE Rx-Tx time difference between 1 sample or 2 samples and 4 samples can be guaranteed within 30Tc except some small PRS RBs (e.g., 24RBs) when the side condition is -3dB or -6dB.

Observation 3: For TDL-D channel, whether in FR1 or FR2, the UE Rx-Tx time difference measurement accuracy is similar to that of AWGN channel.

Observation 4: For AWGN channel, the PRS RSTD between 1 sample or 2 samples and 4 samples can be guaranteed within 15Tc except the case of 24RBs when the side condition is (-3, -6) dB.

Observation 5: For TDL-A channel and TDL-C channel, the PRS RSTD between 1 sample or 2 samples and 4 samples can be guaranteed within 30Tc except the case of 24RBs when the side condition is (-3, -6) dB.

Observation 6 For TDL-D channel, whether in FR1 or FR2, RSTD measurement accuracy is slightly worse than that of AWGN channel under the most cases.

Observation 7: For AWGN channel, the PRS-RSRP measurement accuracy is not sensitive to side condition and sample numbers in all cases.

Observation 8: For Fading channel, the PRS-RSRP measurement accuracy of 1 sample is slightly worse than that of 2 samples and 4 samples in most cases.

- References

- R4-2120330 Link level simulation assumptions for NR PRS measurements with reduced latency, Huawei, HiSilicon