3GPP TSG-RAN4 Meeting #101-e-bis	R4-2200670

# Online, 17 – 25 January, 2022

Agenda item:	6.8.2.2

Source:	Apple

Title:	On R17 FR1 HST inter-frequency measurement

Document for:	Discussion

# Introduction

In RAN4#100e RAN4 requirements for R17 FR1 HST was widely discussed. Corresponding agreement and open items are captured in the approved WF [1]. In this contribution, we continue discussing the open issue regarding inter-frequency measurement in HST.

# Discussion

Issue 2-2-1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

- Agreements in 2nd round:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | FFS |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

The only open issue left is the yellow highlighted FFS for DRX cycle > 320ms. For comparison, following is the number of samples used in PSS/SSS detection time for intra-frequency measurement with gap for HST:

- Intra-frequency measurement with gap: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms

Compared with intra-frequency measurement, additional time should be allowed for potential AGC adjustment, similar as legacy methodology. One possible solution is as what we proposed in the last meeting:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Compared with intra-frequency measurement with gap, one additional DRX cycle is allowed for all the SMTC periodicity configuration. Specifically, 4 DRX cycles when SMTC <= 40ms and 6 DRX cycles when SMTC > 40ms.

Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST for DRX cycle > 320ms, T PSS/SSS_sync_inter is defined as:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

# Conclusion

In this contribution, we further discuss the open issues according to [1]. After discussion, the following conclusions are provided:

Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST for DRX cycle > 320ms, T PSS/SSS_sync_inter is defined as:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

# References

- R4-2120285, WF on RRM for FR1 HST, CMCC