3GPP TSG-RAN4 Meeting #101-e-bis	R4-2200671

# Online, 17 – 25 January, 2022

Agenda item:	6.8.2.3

Source:	Apple

Title:	On R17 FR1 HST L1-SINR

Document for:	Discussion

# Introduction

In RAN4#100e RAN4 requirements for R17 FR1 HST was widely discussed. Corresponding agreement and open items are captured in the approved WF [1]. In this contribution, we continue discussing the open issue regarding L1-SINR measurement in HST.

# Discussion

Issue 3-1-2: how to specify the requirements when L1-SINR is applied in high speed scenario

- Option 1 (HW, QC, Ericsson, MTK, ZTE, Intel, CMCC): Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB

- Option 2 (Nokia, vivo): Current L1-SINR measurement requirement can be reused in HST, no upper bound of side condition

Option 1 is supported. First, 5dB cap comes from high Doppler shift which has been widely discussed since LTE HST speed train WI. Second, even though it is up to NW whether to configure L1-SINR, we don’t think there is much room for NW Tx beam determination based on L1-SINR on top of L1-RSRP in high-speed train scenario, due to the specific UE trajectory and linear deployment of RRH.

Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB.

# Conclusion

In this contribution, we further discuss the open issues according to [1]. After discussion, the following conclusions are provided:

Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB.

# References

- R4-2120285, WF on RRM for FR1 HST, CMCC