3GPP TSG-RAN WG4 Meeting#101-bis-e      	R4-2200712

E-meeting, 17 – 25 Jan, 2022

:

Title:	TP to TR 38.717-03-01: CA_n66-n77-n260

Source:	Nokia, TMO USA

Agenda item:	5.9.1

Document for:	Approval

# 1	Introduction

This contribution introduces CA_n66-n77-n260into TR 38.717-03-01.

**************************** Start of TP *************************************

## 6.X	CA_n66-n77-n260

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n66-n77-n260 | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n77A-n260G | - | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |  |
|  |  | n260 | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n77A-n260H | - | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |  |
|  |  | n260 | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, it means that all single carrier bandwidths for the constituent bands are supported as defined in Table 5.3.5-1 of TS 38.101-1 and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 6.X.3 	Co-existence requirements

### 6.X.4	∆TIB,c and ∆RIB,c values

dTib and dRib are based on CA_n66-n77

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n66-n77-n260 | n66 | 0.6 |
|  | n77 | 0.8 |
|  | n260 | 0 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n260 | n41 | 0.2 |
|  | n66 | 0.5 |
|  | n260 | 0 |

### 6.X.5	REFSENS requirements

MSD studies can be omitted because harmonic interference between FR1 bands have been already studied for CA_n66-n77, and harmonic interference between FR1 bands and FR2 band are negligible.

**************************** Start of TP *************************************