3GPP TSG-RAN WG4 Meeting#101-bis-e      	R4-2200718

E-meeting, 17 – 25 Jan, 2022

Title:	TP to TR 37.717-11-21: DC_2-13_n66-n77

Source:	Nokia, Verizon

Agenda item:	5.19.1

Document for:	Approval

# 1	Introduction

This contribution introduces DC_2-13_n66-n77 combinations into TR 37.717-11-21

********************************* Start of the TP ******************************************

## 7.x	DC_2-13_n66-n77

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2, 13 | n66, n77 |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-13A_n66A-n77A | DC_2A_n66A | 2 | See CA_2A-13A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.x.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_2-13_n66

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |

Table 7.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.x.5	MSD

There are no additional MSD requirements for this band combination.

********************************* End of the TP ******************************************