3GPP TSG-RAN WG4 Meeting#101-bis-e      	R4-2200722

E-meeting, 17 – 25 Jan, 2022

Title:	TP to TR 38.717-05-01: CA_n2-n5-n48-n66-n77

Source:	Nokia, Verizon

Agenda item:	5.19.1

Document for:	Approval

# 1	Introduction

This contribution introduces CA_n2-n5-n48-n66-n77 combinations into 38.717-05-01

********************************* Start of the TP ******************************************

## 5.1	CA_n2-n5-n48-n66-n77

### 5.1.1	Operating bands for CA

Table 5.1.1-1: CA band combination of NR 5DL/1UL

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n7712 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| NOTE 12: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz |  |  |  |

### 5.1.2	Channel bandwidths per operating band for CA

Table 5.1.2-1: Supported bandwidths per CA band combination of NR 5DL/1UL

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configurations | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| CA_n2A-n5A-n48A-n66A-n77A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | 5 | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n5A-n48B-n66A-n77A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A CA_n48B | n2 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n5 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48B Band Combination Set 2 in Table 5.5A.1-1 of 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 5.1.3	∆TIB and ∆RIB values

For CA_n2-n5-n48-n66-n77, the TIB,c and RIB,c values are derived from DC_2-5_n48 and DC_2-66_n77.

Table 5.1.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n5-n48-n66-n77 | n2 | 0.6 |
|  | n5 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.3-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n2-n5-n48-n66-n77 | n2 | 0.2 |
|  | n5 | 0 |
|  | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

********************************* End of the TP ******************************************