3GPP TSG-RAN WG4 Meeting #101b-e              	R4-2200747

Electrical Meeting, 17th Jan- 25th Jan, 2022

Agenda item:	6.9.5.3.1

Source:	Samsung

Title:	Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters on PUSCH requirement for Rel-17 FR2 HST were further discussed. The related agreement was captured in the WF [1] as

- Scope of PUSCH requirement with Uni-directional RRH scenario A/B and and Bi-directional RRH scenario B - Define only one set of requirements for PUSCH - Define HST FR2 model based on Bi-directional scenario-B model. - No test applicability rules are needed - RS configuration for PUSCH and UL timing adjustment - Define requirement with 1 DMRS + PT_RS (L=1, K=2) configuration - Define FRC for 1 DMRS + PT_RS (L=1, K=2) - Define requirement based on the simulation results with 2 DMRS+ PT_RS (L=1, K=2) configuration, but the final requirements are applicable for both 2 DMRS + PT_RS (L=1,K=2) and 3 DMRS+PT_RS (L=1,K=2) - Define FRC for 1+1 DMRS + PT_RS (L=1, K=2) - Define FRC for 1 +1+1 DMRS + PT_RS (L=1, K=2) - FFS, definition of test applicability rules based on BS manufacturer declaration - FOC implementation and MCS - FOC method is up to BS implementation - Do not capture assumptions on FOC scheme in specification - Define PUSCH requirements with MCS(s) between MCS16-20 that is/are feasible and testable. - Companies are encouraged to further check and align the results because a significant span among the initial submitted results is observed ([R4-2117591]). - FFS, the ways to agree on the requirements if the significant span in the results persists - Companies are encouraged to identify suitable MCS(s) for requirements between MCS16-20 in the next meeting - Phase noise modelling - Define 50MHz and 200MHz CBWs with test applicability rule that only one of them is tested based on BS manufacturer - No explicit phase noise modelling in the alignment result

In this contribution, the view on remaining issue are provided for PUSCH requirement. Meanwhile, the initial simulation results are provided to investigate the performance impact with different FOC methods

# 2	Discussion

Test Applicability rule for RS configuration

For FR2 HST, 1 DMRS, 2DMRS and 3 DMRS configurations are considered for PUSCH requirement. Since PTRS is configured, the number of DMRS is not the bottleneck of frequency tracking for FR2. The basic Doppler tracking performance can be guaranteed with 1 DMRS configuration. Based on the simulation results, similar performance can be achieved for 2 DMRS and 3 DMRS configurations. With more DMRS symbols, the overhead of RS is higher than 1 DMRS+PTRS. The following is overhead analysis based on different RS configurations.

Table 2-1: Overhead of RS with different RS configurations

| RS configuration | BW (MHz) | Position of DMRS | Overhead of DMRS (length of data is 10, number of CDM group =2 Mapping type B | Overhead of PTRS (length of data is 10, number of CDM group=2, mapping type B) | Total |
| --- | --- | --- | --- | --- | --- |
| 1 DMRS +PTRS (L=1,K=2) | 200 | {0} | 792/15840 | 594/15840 | 1386/15840 |
| 2DMRS + PTRS (L=1, K=2) | 200 | {0,8} | 1584/15840 | 528/15840 | 2112/15840 |
| 3DMRS+ PTRS (L=1, K=2) | 200 | {0,4,8} | 2376/15840 | 462/15840 | 2838/15840 |
| 1 DMRS +PTRS (L=1,K=2) | 50 | {0} | 192/3840 | 144/3840 | 336/3840 |
| 2DMRS + PTRS (L=1, K=2) | 50 | {0,8} | 384/3840 | 128/3840 | 512/3840 |
| 3DMRS+ PTRS (L=1, K=2) | 50 | {0,4,8} | 576/3840 | 112/3840 | 688/3840 |

Observation 1:  The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Observation 2:  Similar performance can be achieved for 2 DMRS configuration and 3 DMRS configuration.

In Rel-15 BS demod requirement for FR2, both 1 DMRS and 2 DMRS configuration are configured for PUSCH requirement. The related test applicability and BS manufacture for difference RS configuration are defined as follows

- Unless otherwise stated, for BS type 2-O, PUSCH requirement tests shall apply only for the additional DM-RS position declared to be supported (see D.101 in table 4.6-1). If both options (i.e., pos0 and pos1) are declared to be supported, the tests shall be done for pos1.

| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| --- | --- | --- | --- | --- | --- |

Observation 3:  Existing Rel-15 test applicability rule and BS manufacture with different RS configuration cannot guarantee Rel-17 FR2 HST BS test with more than 2 DMRS configuration.

BS manufacture may declare to support with all the three DMRS configurations or subset of DMRS configurations. To reduce the test effort, and guarantee the basic baseband performance, it is not necessary to test all the supported DMRS configurations.

Proposal 1:  FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If the all the DMRS configuration or subset of DMRS configuration declared to be supported, the test shall be done for the minimum number of DMRS supported, or only one of supported DMRS configuration based on BS manufacture declaration.

| DMRS configuration declared to be supported | PUSCH requirement tests |
| --- | --- |
| {pos0, pos1, pos2} | Option 1: either test with pos0, or pos1,or pos2 only based on BS declaration Option 2: only test for pos0 |
| {pos0, pos1} | Option 1: either test with pos0 or pos1 only based on BS declaration Option 2: only test for pos0 |
| {pos0, pos2} | Option 1: either test with pos0 or pos2 only based on BS declaration Option 2: only test for pos0 |
| {pos1,pos2} | Option 1: either test with pos1 or pos2 only based on BS declaration Option 2:only test for post1 |
| {pos0}, or {pos1}, or {pos2} | Only test DMRS declared to be supported |

| D.1XX | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2 high speed train scenario, i.e., {pos0},{pos1},{pos2},{pos0,pos1}, {pos0 pos1}, {pos0,pos2}, {pos1,pos2} and all | n/a | n/a | x |
| --- | --- | --- | --- | --- | --- |

FOE implementation and MCS

Regarding the FOE implementation, RAN4 has a lot of discussion for two kinds of FOC method. As agreed, the FOC method is up to BS implementation.

Regarding MCS, in FR1 HST, MCS 16 is specified for PUSCH requirement. Based on our results, both Post-FFT FOC and Pre-FFT FOC are feasible, meanwhile, the performance different is around 2dB. Since RAN4 is to define minimum requirement, to verify the basic baseband processing, MCS16 can be considered for requirement.

For non-HST scenario, up to 64QAM are considered for PUSCH requirement. Based on our results, MCS17 is also feasible, this is no OTA testability issue identified for MCS17. Meanwhile, minor performance difference for both Post-FFT and Pre-FFT FOC is existed, which can guarantee proper BS implementation to meet requirement  Therefore, we are fine to define both MCS 16 and MCS 17 for PUSCH requirement, if the simulation results among companies are aligned well.

Regarding MCS 20, considering the large gap for two FOC implementation is existed, it is not proper to define requirement for MCS20 based on Post-FFT FOC assumption. Therefore, we are open to define corresponding manufacturer declaration on supporting MCS 20 with pre-FFT FOC and apply requirements on MCS 20 only based on this declaration.

Proposal 2:  Both MCS 16 and MCS17 are feasible for FR2 HST PUSCH requirements

FRC

Regarding the FRC table, considering there is no agreement about MCS for requirement. It can be updated if MCS is determined.

# 3	Initial Simulation Results

In this subsection, the initial simulation results for scenario B under bi-directional,. Also, the performance with different MCS level are investigated to check the impact on high modulation order with different FOC method

Table 3-1 Simulation Assumption for PUSCH requirement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna layout |  | 1T2R, ULA Low Correlation |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Option11+0 Option2: 1+1 Option3: 1+1+1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
|  | Bandwidth | 50MHz and 200MHz |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | KPT-RS=2 |
|  | Time density (LPT-RS) | LPT-RS = 1 |
| MCS |  | MCS16 to MCS20 (64 QAM table) |
| Propagation channel |  | Bi-directional with sceanro B |

The following table is provided the initial results with MCS 16 to MCS20 are considered for different DMRS configuration

Table 3-1:  Initial results for different MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m), 120KHz, 200MHz (to be updated)

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP (Pre-FFT FOC) | SNR@70 of TP (Post-FFT FOC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+0 | 7.2 | 9.7 |
| Case 2 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+1 (0,8) | 7.1 | 9.5 |
| Case 3 | 1T2R | 120KHz 200MHz | B | 10 | 16 | 1+1+1(0.4.8) | 7.1 | 9.5 |
| Case4 | 1T2R | 120KHz 200MHz | B | 10 | 17 | 1+0 | 7.9 | 11.2 |
| Case 5 | 1T2R | 120KHz | B | 10 | 17 | 1+1 (0.8) | 7.6 | 10.7 |
| Case 6 | 1T2R | 200MHz | B | 10 | 17 | 1+1 (0,4,8) | 7.9 | 10.9 |
| Case 7 | 1T2R | 120KHz | B | 10 | 18 | 1+0 |  |  |
| Case 8 | 1T2R | 200MHz | B | 10 | 18 | 1+1 (0.8) |  |  |
| Case 9 | 1T2R | 120KHz | B | 10 | 18 | 1+1 (0,4,8) |  |  |
| Case 10 | 1T2R | 120KHz 200MHz | B | 10 | 19 | 1+0 |  |  |
| Case 11 | 1T2R | 120KHz, 200MHz | B | 10 | 19 | 1+1 (0.8) |  |  |
| Case 12 | 1T2R | 120KHz, 200MHz | B | 10 | 19 | 1+1 (0,4,8) |  |  |
| Case 13 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+0 | 10.4 | 17.7 |
| Case 14 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1(0,8) | 10.3 | 17.2 |
| Case 15 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1+1(0,4,8) | 10.4 | 17.5 |

Table 3-2:  Initial results for different MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m), 120KHz, 50MHz (to be updated)

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP (Pre-FFT FOC) | SNR@70 of TP (Post-FFT FOC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 50MHz | B | 10 | 16 | 1+0 |  |  |
| Case 2 | 1T2R | 120KHz,50MHz | B | 10 | 16 | 1+1 (0,8) |  |  |
| Case 3 | 1T2R | 120KHz, 50MHz | B | 10 | 16 | 1+1+1(0.4.8) |  |  |
| Case4 | 1T2R | 120KHz 50MHz | B | 10 | 17 | 1+0 |  |  |
| Case5 | 1T2R | 120KHz, 50MHz | B | 10 | 17 | 1+1 (0.8) |  |  |
| Case6 | 1T2R | 120KHz 50MHz | B | 10 | 17 | 1+1 (0,4,8) |  |  |
| Case7 | 1T2R | 120KHz 50MHz, | B | 10 | 18 | 1+0 |  |  |
| Case8 | 1T2R | 120KHz 50MHz | B | 10 | 18 | 1+1 (0.8) |  |  |
| Case9 | 1T2R | 120KHz, 50MHz | B | 10 | 18 | 1+1 (0,4,8) |  |  |
| Case 10 | 1T2R | 120KHz 50MHz | B | 10 | 19 | 1+0 |  |  |
| Case 11 | 1T2R | 120KHz 50MHz | B | 10 | 19 | 1+1 (0.8) |  |  |
| Case 12 | 1T2R | 120KHz 50MHz | B | 10 | 19 | 1+1 (0,4,8) |  |  |
| Case 13 | 1T2R | 120KHz 50MHz | B | 10 | 20 | 1+0 |  |  |
| Case 14 | 1T2R | 120KHz 50MHz | B | 10 | 20 | 1+1(0,8) |  |  |
| Case 15 | 1T2R | 120KHz, 50MHz | B | 10 | 20 | 1+1+1(0,4,8) |  |  |

Observation 4: Around 2 or 3 dB difference between post-FFT and pre-FFT FOC methods for MCS 16 and MCS17

Observation 5: Around 7dB difference between post-FFT and pre-FFT FOC methods for MCS 20

Observation 6: Requirement with MCS16 to MCS 20 are feasible and can be testable, for both Post-FFT and Pre-FFT FOC operation.

# 4	Conclusion

In this contribution, the view on the PUSCH requirement for Rel-17 FR2 HST is provided. Meanwhile, the initial simulation results with different MCS level are provided to check the feasibility of high modulation order in HST with different FOC methods.

Observation 1:  The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Observation 2:  Similar performance can be achieved for 2 DMRS configuration and 3 DMRS configuration.

Observation 3:  Existing Rel-15 test applicability rule and BS manufacture with different RS configuration cannot guarantee Rel-17 FR2 HST BS test with more than 2 DMRS configuration.

Proposal 1:  FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If the all the DMRS configuration or subset of DMRS configuration declared to be supported, the test shall be done for the minimum number of DMRS supported, or only one of supported DMRS configuration based on BS manufacture declaration.

Proposal 2:  Both MCS 16 and MCS17 are feasible for FR2 HST PUSCH requirements

Observation 4: Around 2 or 3 dB difference between post-FFT and pre-FFT FOC methods for MCS 16 and MCS17

Observation 5: Around 7dB difference between post-FFT and pre-FFT FOC methods for MCS 20

Observation 6: Requirement with MCS16 to MCS 20 are feasible and can be testable, for both Post-FFT and Pre-FFT FOC operation.

# References

R4-2120703, WF on BS demodulation requirement for FR2 HST,  Nokia, Samsung